3GPP TSG-RAN WG3 Meeting #126	R3-247453

# Orlando, US, 18th – 22nd November, 2024

Agenda Item	: 11.2

Source	: Tejas Networks

Title		: (TP to TR 38.743) AI/ML assisted Network Slicing

Document for	: Discussion and Decision

# 1   Introduction

In this contribution, we propose an amendment to Clause 4.1 of Technical Report (TR) 38.743.

Current Solutions
As specified in TR 38.743 (V19.0.0), two solutions are currently supported for AI/ML-based network slicing:

Solution 1: AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the 	                 gNB.

Solution 2: AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

We propose an additional solution where AI/ML Model Training can be conducted in both the OAM (FL Server) and gNB (FL Client), with AI/ML Model Inference taking place in the gNB.

Proposed Solution
 	  	Solution 3: AI/ML Model training is located both at OAM (FL server) and gNB (FL Client) and AI/ML   						 model inference is located at gNB

Justifications for our proposed solution
       1. The proposed solution eliminates the need to transfer raw data (e.g., UE throughput data) between    		 gNBs and the OAM, which is a key limitation of the first solution.

2. This solution also enables access to insights from other gNBs through the aggregation process within 		 the OAM, overcoming the limitation of the second solution.

3. This solution encourages diversity in the models, like enabling integration of new gNBs with 		   	     		   advanced models into the existing system.

# 2   Discussion

2.1. Need for Federated learning approach

Consider a scenario where a premium vehicle (UE) is moving from Location A to Location B and consuming multimedia content all along the route and operator want to create a separate slice for this premium user and want to charge him separately for the data consumed by this user along the route.

In this case suppose UEs are moving in various random trajectories along this route, resulting in multiple handovers across a network of gNBs. Each gNB and the OAM is involved in various phases of Network slicing.

For proper operational management of this network slice, the operator will require each gNB which is part of this slice and the OAM which is managing this slice needs information regarding the UE traffic in its vicinity for calculating the slices at various future time instants (T1, T2, T3, …Tn). Also, since the UEs are moving in various random trajectories across the terrain it results in multiple handovers across for each gNBs. In this scenario, sitting in the instance of time Tn, it becomes extremely difficult for each gNB and its associated OAM in the Network slice operation phase to do resource capacity planning and modification in needed of the Network slices for Tn+1

At the time instance Tn, we can predict the aggregated UE traffic for k gNBs for the Tn+1 time instance (for uni-step prediction), such as B1n+1, B2n+1, B3n+1,.. BKn+1 and multiple time instances in advance, Tn+1,Tn+2,Tn+3,…Tn+z (for multistep prediction) using the current aggregated UE traffic measured at each gNBs and also the aggregated UE traffic measured from the various past Tn-1 time periods by training the models located at both gNB(Client) and OAM. The name of this approach is Federated Learning (FL).

Another use case of FL is that it can be used for the aerial fleet management of connected drones, which can be used for delivery, surveillance etc. As drones operate across wide areas, they will frequently move between different gNBs. The different types of data (location, altitude, speed etc) from drones are collected by the gNBs (FL client) and are used for training the local models in the gNBs. After the training, each gNB will send their model parameters to the OAM (FL Server). The OAM aggregates the model parameters from the different gNBs and send updated model containing aggregated model parameters back to the gNBs enabling enhanced coordinated flight patterns and improved overall performance for the drones by the collective intelligence derived from the entire fleet, while maintaining data privacy and optimizing flight efficiency.

A detailed mechanism for how to operate this FL was discussed in R3-244039

Observation 1: For FL, privacy is preserved since there is no need for exchange of UE throughput between source/target nodes.

Observation 2: FL approach for the AI/ML aided network slicing requires the model to be trained locally at the gNB (FL Client) and then consolidated into a global model at the OAM (FL Server).

Proposal 1: RAN3 WG to consider the use case of AI/ML model training to be done at gNB and aggregated at OAM.

Proposal 2: If Proposal 1 is agreeable, RAN3 WG should consider the TP for 38.743 as provided by us in Annex.

# 3   Conclusion

Following is our observations and proposals:

Observation 1: For FL, privacy is preserved since there is no need for exchange of UE throughput between source/target nodes.

Observation 2: FL approach for the AI/ML aided network slicing requires the model to be trained locally at the gNB (FL Client) and then consolidated into a global model at the OAM (FL Server).

Proposal 1: RAN3 WG to consider the use case of AI/ML model training to be done at gNB and aggregated at OAM.

Proposal 2: If Proposal 1 is agreeable, RAN3 WG should consider the TP for 38.743 as provided by us in Annex.

# Annex TP to 38.743

# 4	Use cases and Solutions

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Support of network slicing in NG-RAN is defined in TS 38.300 [2].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyse metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

### 4.1.2	Solutions and standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

-    AI/ML Model training is located both at OAM (FL Server) and gNB (FL Client) and AI/ML model inference is located at gNB

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

-    AI/ML Model training is located both at OAM (FL Server) and gNB-CU (FL Client) and AI/ML model inference is located at gNB-CU

#### 4.1.2.2	Input data of AI/ML based Network Slicing

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.3	Output data of AI/ML based Network Slicing

AI/ML-based network slicing model in a gNB can generate the following information as output:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)-	Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.4	Feedback of AI/ML based Network Slicing

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

-	Measured Radio resource status per slice

-	Measured Slice available capacity

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE.

NOTE:	Exact level of finer granularity to be defined in normative phase.

#### 4.1.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

-	Enhanced existing procedure to collect predicted information between gNBs:

■	Predicted radio resource status per slice

■	Predicted slice available capacity