3GPP TSG-RAN WG3 #126	R3-247530

Orlando, USA,  18 – 22 November, 2024

Agenda item:	9.2

Source:	Jio

Title:	Discussion on Periodic E-CID Measurement Procedure

Document for:	Discussion and Decision

# 1	Introduction

Enhanced Cell ID (E-CID) is one of the methods specified by 3GPP to determine the location of the UE using measurements from the 5G radio network.

The procedures for E-CID method are specified in 3GPP TS 38.455.

In periodic E-CID procedure, the Location Measurement Function (LMF) initiates the measurement with the gNodeB by specifying a periodicity for reporting the measurement for a particular UE. The gNodeB in turn enables periodic measurement for RSRP/RSRQ on the air interface with specified periodicity.

The gNodeB is supposed to report the measurement until either the LMF “Terminates” the procedure or the UE context is released from the gNodeB.

The negative scenario where the “Termination Command” from LMF does not reach the gNodeB due to any reason is not handled in the current specification. In such a negative scenario, as the “Termination Command” from LMF is not received, the gNodeB in-turn does not terminate the UE periodic measurement on RRC and also keeps sending “E-CID Measurement Report” to the LMF. This causes loss of radio resource due to un-necessary RRC Signaling (Measurement Reports) and also un-necessary signaling on the NRPPa (E-CID Measurement Report).

This proposal is to address this gap in the specification for periodic E-CID measurement. (Same has been discussed in “R3-247532 (CR to 38.455) Periodic E-CID Measurement Procedure”.

The proposal is to introduce a “Count” parameter so that same parameter can be forwarded to the UE for RRC Measurement Report also.

When the gNodeB has completed sending the E-CID Measurement Report equal to the configured “Count” value then the Periodic E-CID Measurement Procedure is automatically deemed as completed / terminated at both gNodeB and LMF. This can be referred to as “Implicit Termination”.

# 2	Discussion

The current specification does not address the negative scenario where the “TERMINATION COMMAND” from the LMF fails to reach the gNodeB. In this case, because the gNodeB does not receive the “TERMINATION COMMAND,” it continues to request periodic measurements from the UE over RRC and to send “E-CID MEASUREMENT REPORTS” to the LMF. This results in unnecessary RRC signaling and NRPPa signaling, leading to inefficient radio resource utilization. This proposal aims to address this gap in the specification for periodic E-CID measurement procedures.

The proposal is to introduce a “Count” parameter so that same parameter can be forwarded to the UE for RRC Measurement Report also.

Proposal 1:The LMF shall set the "MAX MEASUREMENT COUNT" IE to specify to the gNodeB the maximum number of "TA," "RSRP," and "RSRQ" measurements that the gNodeB is required to report to the LMF, according to the periodicity defined by the "MEASUREMENT PERIODICITY" IE.

Proposal 2: The periodic E-CID measurement procedure shall be terminated in one of the following ways:

a) The LMF sends an "E-CID MEASUREMENT TERMINATION COMMAND" to the gNodeB before the gNodeB has sent an "E-CID MEASUREMENT REPORT" equal to the configured "MAX MEASUREMENT COUNT."

b) The gNodeB sends an "E-CID MEASUREMENT FAILURE INDICATION" to the LMF upon "UE CONTEXT RELEASE" before completing the reporting up to the "MAX MEASUREMENT COUNT."

c) "IMPLICIT TERMINATION": The procedure shall be considered complete when the time period equivalent to "MAX MEASUREMENT COUNT" multiplied by "MEASUREMENT PERIODICITY" elapses on both the gNodeB and the LMF, following the sending of an "E-CID MEASUREMENT INITIATION RESPONSE" by the gNodeB and its reception by the LMF.

# 3	Conclusion

Proposal 1:The LMF shall set the "MAX MEASUREMENT COUNT" IE to specify to the gNodeB the maximum number of "TA," "RSRP," and "RSRQ" measurements that the gNodeB is required to report to the LMF, according to the periodicity defined by the "MEASUREMENT PERIODICITY" IE.

Proposal 2: The periodic E-CID measurement procedure shall be terminated in one of the following ways:

a) The LMF sends an "E-CID MEASUREMENT TERMINATION COMMAND" to the gNodeB before the gNodeB has sent an "E-CID MEASUREMENT REPORT" equal to the configured "MAX MEASUREMENT COUNT."

b) The gNodeB sends an "E-CID MEASUREMENT FAILURE INDICATION" to the LMF upon "UE CONTEXT RELEASE" before completing the reporting up to the "MAX MEASUREMENT COUNT."

c) "IMPLICIT TERMINATION": The procedure shall be considered complete when the time period equivalent to "MAX MEASUREMENT COUNT" multiplied by "MEASUREMENT PERIODICITY" elapses on both the gNodeB and the LMF, following the sending of an "E-CID MEASUREMENT INITIATION RESPONSE" by the gNodeB and its reception by the LMF.