3GPP TSG-RAN WG3 Meeting #R3-126		           R3-247604

Orlando, USA, Nov 18-22, 2024

Agenda Item:	11.2

Source: 	Rakuten

Title:  	Discussion for AI/ML based Network Slicing

Document for:	Discussion and decision

# Introduction

Related to UE performance feedback of the handed over UEs from the target node to the source node, we list below the relevant references:

From 38.743:

# Discussion

In the previous meetings, it has been discussed and agreed that after successful HO completion from source to target gNB, the target gNB will send UE performance metrics to the source gNB. This will assist the source gNB to evaluate the merits of its mobility decisions and act as feedback to validate the accuracy of its AI/ML training. To support this, the source gNB is required to maintain the UE context of the UE which has been handed over to the target gNB until it has received the feedback information.

## Relative transmission of UE Context Release

For handover, the UE Context Release procedure is initiated by the target NG-RAN node to indicate to the source NG-RAN node that radio and control plane resources for the associated UE context are allowed to be released.

To support the reporting of UE performance metric from the target NG-RAN node to source NG-RAN node for Rel 18 AI/ML, there are two options with respect to the UE Context Release message sent from the target NG-RAN node to the source NG-RAN node at the completion of a successful handover.

Option 1: If the source NG-RAN node has included a Data Collection ID IE in the HANDOVER REQUEST message, then the source NG-RAN node will ignore the UE Context Release it receives from the target NG-RAN node, until after it receives the Data Collection Update. After receiving the Data Collection Update procedure, the source NG-RAN node will act on the earlier received UE Context Release. This is a deviation from the legacy behavior on the recipient of the UE Context Release procedure by a receiving entity.

Observation 1: To support Rel 18 AI/ML, use of the legacy mechanism, where the target NG RAN node sends a UE Context Release to the source NG RAN node on completion of a successful HO, might lead to a change in the behavior at the source NG RAN node on receiving the UE Context Release from the target NG RAN node.

Option 2: After successful HO completion, the target NG-RAN node holds back on sending the UE Context Release to the source node. It defers this procedure until after it sends the Data Collection Update message to the source NG-RAN node. In this case, the behavior at the source NG-RAN node on receiving UE Context Release remains the same as in the legacy mechanism. The behavior at the target NG-RAN node changes from the legacy.

Based on the above, we propose option 2 which eliminates the chance of UE context release at the source NG RAN node before it has received the UE performance feedback.

Proposal 1: We propose that target NG RAN node sends the UE Context Release to the source NG RAN node after it sends the Data Collection Update in cases where Data Collection ID IE is included in the Handover Request.

From 38.423: The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep. Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. If the UE Context Release procedure is not initiated towards the source NG-RAN node from any prepared NG-RAN node before the expiry of the timer TXnRELOCoverall, the source NG-RAN node shall request the AMF to release the UE context.

To support the requirements of AIML feedback, this requires that the target NG RAN node send the Data Collection Update to the source NG RAN node before the expiry of the timer TXnRELOCoverall. And the source needs to maintain the UE context until the expiry of the timer or until it receives Data Collection Update from the target NG RAN node.

Observation 2: The target NG RAN node should send the Data Collection Update to the source NG RAN node before the expiry of the timer TXnRELOCoverall. The shource needs to maintain the UE context until the expiry of the timer or until it receives the Data Collection Update from the target NG RAN node.

The timing of transmission of Data Collection Update from the target NG RAN node depends on the value of the Collection Time Duration for UE Performance IE and the exit conditions which have been configured by the source NG RAN node. Hence, the source NG RAN node should configure the value of the timer TXnRELOCoverall taking into account the value of Collection Time Duration for UE Performance IE and the exit conditions. This will enable it to retain the UE context until it receives the Data Collection Update.

Proposal 2:  The source NG RAN node configures the value of the timer TXnRELOCoverall taking into account the value of Collection Time Duration for UE Performance IE and the exit conditions.

# Conclusion

We have the following observations and proposals:

Observation 1: To support Rel 18 AI/ML, use of the legacy mechanism, where the target NG RAN node sends a UE Context Release to the source NG RAN node on completion of a successful HO, might lead to a change in the behavior at the source NG RAN node on receiving the UE Context Release from the target NG RAN node.

Proposal 1: We propose that target NG RAN node sends the UE Context Release to the source NG RAN node after it sends the Data Collection Update in cases where Data Collection ID IE is included in the Handover Request.

Observation 2: The target NG RAN node should send the Data Collection Update to the source NG RAN node before the expiry of the timer TXnRELOCoverall. The shource needs to maintain the UE context until the expiry of the timer or until it receives the Data Collection Update from the target NG RAN node.

Proposal 2:  The source NG RAN node configures the value of the timer TXnRELOCoverall taking into account the value of Collection Time Duration for UE Performance IE and the exit conditions.

# Reference

[1] TS38.423 Xn application protocol (XnAP)