3GPP TSG-RAN WG3 Meeting #126	R3-247654

Orlando, USA, 18-22 Nov 2024

Agenda Item:	14.2

Source:	ZTE Corporation

Title:	(TP for TS 38.300) Discussion on NTN broadcast service supporting

Document for:	Discussions & Approval

- Introduction

The Work Item on how to support MBS broadcast in NR NTN has been agreed in RAN#102(RP-234078) with the following objective:

==== content in objective ====

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

==== content in objective ====

Additionally, the following agreement has been made at the end of previous RAN3 meeting:

Till now, the following progress has been made during previous meetings.

RAN3 starts work on MBS broadcast service.

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

The intention of this contribution is to further explain our views on how to support broadcast service in NR NTN.

- Discussion

The discussion on MBS supporting in regenerative payload has also been touched in previous meeting. based on the discussion progress in regenerative payload section, companies may believe that based on the movement of gNB on broad, the MBS broadcast session may be impacted. Moreover, how to handle the stored MBS broadcast session in the gNB on broad can be based on current mechanism at current stage.  Any other further enhancement on how to treat the MBS broadcast session in regenerative payload scenario will be triggered if the progress made in regeneration payload section may impact the NTN broadcast service supporting.

Proposal 1: It is proposed for RAN3 to confirm the current mechanism can treat the MBS broadcast session in regenerative payload scenario. Any other further enhancement on how to treat the MBS broadcast session will be triggered if necessary.

In addition, a TP to draft CR for TS 38.300 which is used to capture the RAN3 agreement is prepared in annex part and can be considered as baseline.

Proposal 2: Capture the RAN3 agreement and consider the TP to draft CR in annex as baseline.

- Conclusion

Proposal 1: It is proposed for RAN3 to confirm the current mechanism can treat the MBS broadcast session in regenerative payload scenario. Any other further enhancement on how to treat the MBS broadcast session will be triggered if necessary.

Proposal 2: Capture the RAN3 agreement and consider the TP to draft CR in annex as baseline.

- Annex TP to draft CR for TS38.300

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26].

Editor’s Note: The above paragraphs may be further reformulated.

NEXT CHANGE

INFORMATIVE: RAN2 MAY EXPECT RAN3 TO REVIEW THIS PART – NO CHANGE

MBS Broadcast Session Setup and MBS Broadcast Session Release procedure may be used for AMF to treat MBS broadcast session for gNB on satellite.