3GPP TSG-RAN WG3 Meeting #127-bis	R3-251603

Wuhan, China, 7-11 April, 2025

Title:	(TPs to TS 38.300 38.413 BL CRs) Architecture aspects and interface management procedures

Agenda Item:	16.2

Source:	Huawei

Document for:	other

# Introduction

In this contribution, we discuss the Architecture aspects and interface management procedures based on the progress achieved in last meeting, and provide the corresponding TPs to baseline CRs.

# 2	Discussion

## 2.1	General Architecture

In last meeting, the following WA was captured in chairman notes and the TS 38.300 BL CR:

WA: Take Option1 (Including AIoTF information in NGAP).

And in the LS R3-250905, this WA is further elaborated as:

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

The following protocol stacks shows how this solution works in direct communication case and indirect communication case. In direct communication, the AIOTF will encode/decode both the AIOTF Information Transfer IEs and the NGAP messages. In indirect communication, the AMF will encode/decode the NGAP messages, but the AIOTF Information Transfer IEs will be forwarded by the AMF transparently. The WA should be changed to agreement.

AIOTF Info in direct communication						AIOTF Info in indirect communication

Proposal 1: Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP.

## 2.2	Reader Location

The following agreements were made in last meeting:

- AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

- AIOTF may be aware of the location of reader via OAM configuration.

And in the LS, the following were listed:

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information.

From our understanding, OAM configuration is a default solution which should be available to provide the location of the reader to the AIOTF as well, there is no need to say “may”.

Proposal 2-1: AIoTF is aware of the location of reader, at least via OAM.

About the details of the location of reader, the following FFS was made:

- FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

Comparing with introducing new complex location defintions by RAN3, it is preferred to reuse existing ones which have aready well defined by 3GPP.

In TS 29.572, section 6.1.6.2.14, “Civic Address” is defined as follows, and it can indicate the street, the building, the floor, the room, etc.

And in TS 38.455, the following location information can also be considered:

- 9.2.10	NG-RAN Access Point Position

- 9.2.49	NG-RAN High Accuracy Access Point Position

To allow different implementations in different scenarios, maybe we can consider to support all of these three types of location, i.e., define the reader location as follows:

9.3.1.xxx		Reader Location

This IE provides the location information of the reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reader Location | M |  |  |  |
| >Civic Address |  |  |  |  |
| >>Civic Address | M |  | OCTET STRING | Includes the CivicAddress as defined in TS 29.572 [xx]. |
| >NG-RAN Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | OCTET STRING | Includes the NG-RAN Access Point Position IE as defined in TS 38.455 [19]. |
| >NG-RAN High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | OCTET STRING | Includes the NG-RAN High Accuracy Access Point Position IE as defined in TS 38.455 [19]. |

Proposal 2-2: The Reader Location can be indicated by Civic Address defined in TS 29.572, or NG-RAN Access Point Position or NG-RAN High Accuracy Access Point Position defined in TS 38.455.

## 2.3	Signalling of A-IoT RAN Information to AIOTF

As discussed above in 2.2 and 2.3, the AIOTF is able to know the flowing A-IoT RAN Information via OAM configuration:

- Supported Area(s)

- Served Reader List

- Location of the Reader

Although the above information will not change frequently, but they may change, providing such information in interface management procedures will not lead to high signalling load, it is therefore preferred to also provide them to the core network via NGAP signalling.

Proposal 3: Signalling the Supported Area(s), the Served Reader List and the Location of Reader in NG SETUP REQUEST message and the RAN CONFIGURATION UPDATE message, by introducing an A-IoT RAN Information Transfer IE.

## 2.4	“New A-IoT Area” or “TAI dedicated for A-IoT”

The following can be found in the LS R3-250905:

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

It should be understood that the “Area” in those statements is talking about the same thing. the OAM configures the supporte Area of the gNB to the AIOTF, or the gNB reports the supported Area to the AIOTF (FFS), and the AIOTF may provide the Area in INVENTORY REQUEST message to request the gNB to perform inventory within that Area.

In our view, both “New A-IoT Area” and “Tracking Area dedicated for A-IoT” are workable.

- Solution 1: Use A-IoT dedicated TAI, by reusing existing Supported TA List IE

  - the RAT Information IE ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) needs to be enhanced with new code point ‘A-IoT’.

  - the current Semantics description of TAC and PLMN Identity, i.e., “Broadcast TAC” and “Broadcast PLMN” should be changed, as the word “Broadcast’ is not proper.

  - the slicing related mandatory sub-IEs need to be ignored.

- Solution 2: Use A-IoT dedicated TAI, as sub-IE of the new A-IoT RAN Information Transfer IE

  - the slicing related IEs are not there, not proper text “broadcast’ will not happen.

  - no need to indicate RAT information additionally.

- Solution 3: Introduce new A-IoT Area

  - a brand-new identifier will be introduced, which can be similar to the TAI, e.g., an A-IoT Area Identity = A-IoT Area Code + List of Supported PLMNs.

  - need to coordinate with SA2/SA5 on the definition of A-IoT Area Code.

Comparing with these three solutions, seems solution 2 is the simplest and cleanest way to go.

Proposal 4: Further discuss whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT. It is slightly preferred to use A-IoT dedicated TAI as sub-IE of the new A-IoT RAN Information Transfer IE

## 2.5	Handling of requested service area information

The following agreements were achieved in last meeting:

- AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

  - Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

  - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

  - WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

  - FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

And the LS says:

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

About the third sub bullet WA, upon receiving only the reader list in the Inventory Request, there is no reason for the gNB to further down select readers among the indicated ones, therefore, it is better to change the WA to agreement.

Proposal 5-1: Upon receiving only the reader list in Inventory Request, the gNB performs inventory in the readers indicated by the reader list.

For the fourth sub bullet, upon receiving both the area and the reader list in Inventory Request, the proper way is to perform inventory in both the indicated area and the indicated readers.

Proposal 5-2: Upon receiving both the area and the reader list in Inventory Request, the gNB performs inventory in the readers within the indicated area and the readers within the reader list.” to agreement.

## 2.6	Interface Management procedures

About Interface management procedures, the following FFSs were achieved last meeting:

- FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

- FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

- FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

In indirect communication, the gNB will initiate the NG Setup procedure towards the AMF, existing NG interface management procedures should be reused/enhanced.

Note that, in case the gNB reports some A-IoT RAN Information to the AMF in e.g., NG SETUP REQUEST message, the AIOTF(s) will be able to get the information based on the related service provided by the AMF, and the interaction between the AMF and AIOTF(s) is out of RAN3 scope.

Proposal 6-1: Existing NG interface management procedures are used in indirect communication between the gNB and the AMF.

In direct communication, the gNB needs to establish NG interface with the AIOTF, in order to have unified handling at gNB side for both direct communication and indirect path communication, we slightly preferred to also use existing NG interface management procedures between the gNB and the AIOTF.

In current NGAP specification, there are following interface management procedures:

- NG Setup

- RAN Configuration Update

- NG Reset

- Error Indication

- AMF Configuration Update

- AMF Status Indication

- Overload Start

- Overload Stop

The procedure 5) ~ 8) are AMF specific procedures and not applicable to AIOTF, for direct path communication, we can reuse/enhance the procedure 1) ~ 4) between the gNB and the AIOTF.

Proposal 6-2: Existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are reused in direct communication between the gNB and the AIOTF.

When the gNB initiates the NG Setup, if the gNB only supports A-IoT radio, there are some mandatory IEs not applicable to A-IoT, e.g., Default Paging DRX IE. One possible solution is to introduce an optional IE e.g., Only Support A-IoT IE, and in such case the AMF/AIOTF will ignore such not applicable mandatory IE(s).

Option 1: Explicitly introduce optional IE to ignore not applicable mandatory IE(s)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | This IE is ignored if the Only Support A-IoT IE is included. | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Only Support A-IoT | O |  | ENUMERATED (true, ...) | This IE indicates that the NG-RAN node only provides A-IoT radio. | YES | ignore |

On the other hand, in case the AMF/AIOTF is able to know that the gNB only supports A-IoT via OAM configuration, we can also add semantics description direct without the new IE.

Option 2: ignore not applicable mandatory IE(s) based on deployment known via OAM

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | In case the NG-RAN node only provides A-IoT radio, this IE is ignored. | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |

From our view, option 1 is the tranditonal way to ignore not applicable mandatory IEs and much cleaner comparing with option 2.

Proposal 6-3: Ignoring the not applicable mandatory IE(s) based on new explicitly optional IE.

# 3	Conclusion

Based on the discussion in this paper, we got the following proposals:

About General Arch

Proposal 1: Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP.

About Reader Location

Proposal 2-1: AIoTF is aware of the location of reader. FFS on whether OAM or signalling or both.

Proposal 2-2: The Reader Location can be indicated by Civic Address defined in TS 29.572, or NG-RAN Access Point Position or NG-RAN High Accuracy Access Point Position defined in TS 38.455.

About Signalling of A-IoT RAN Information to AIOTF

Proposal 3: Signalling the Supported Area(s), the Served Reader List and the Location of Reader in NG SETUP REQUEST message and the RAN CONFIGURATION UPDATE message, by introducing an A-IoT RAN Information Transfer IE.

About “New A-IoT Area” or “TAI dedicated for A-IoT”

Proposal 4: Further discuss whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT. It is slightly preferred to use A-IoT dedicated TAI as sub-IE of the new A-IoT RAN Information Transfer IE

About Handling of requested service area information

Proposal 5-1: Upon receiving only the reader list in Inventory Request, the gNB performs inventory in the readers indicated by the reader list.

Proposal 5-2: Upon receiving both the area and the reader list in Inventory Request, the gNB performs inventory in the readers within the indicated area and the readers within the reader list.” to agreement.

About Interface Management procedures

Proposal 6-1: Existing NG interface management procedures are used in indirect communication between the gNB and the AMF.

Proposal 6-2: Existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are reused in direct communication between the gNB and the AIOTF.

Proposal 6-3: Ignoring the not applicable mandatory IE(s) based on new explicitly optional IE.

The corresponding TPs are provided in section 4 and 5.

# 4	TP to 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In case the gNB communicates with AIOTF indirectly via AMF, the AIOTF containers are transparent to the AMF.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

### 16.xx.x3	OAM Requirements

OAM configures the following A-IoT RAN Information of a gNB to the AIOTF:

- The supported Tracking Area(s) dedicated for A-IoT;

- The served reader list;

- The location of the reader, using the format of Civic Address defined in TS 29.572, or NG-RAN Access Point Position IE or NG-RAN High Accuracy Access Point Position IE defined in TS 38.455.

Editor’s Note:	It is FFS whether to introduce new “A-IoT Area” or use “A-IoT dedicated Tracking Area”.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request may include information concerning one or multiple A-IoT devices, and it may contain the requested service area information (A-IoT Areas and/or Reader List).

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

# 5	TP to 38.413 BL CR

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ACL functionality: as defined in TS 36.413 [16].

A-IoT CN node: as defined in TS 38.300 [8].

CAG cell: as defined in TS 38.300 [8].

DAPS Handover: as defined in TS 38.300 [8].

Elementary Procedure: NGAP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several NGAP EPs together or together with EPs from other interfaces is specified in stage 2 specifications (e.g., TS 38.401 [2], TS 38.410 [3] and TS 38.300 [8]).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 5	NGAP Services

NGAP provides the signalling service between the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF, that is required to fulfil the NGAP functions described in TS 38.410 [3]. NGAP services are divided into two groups:

Non UE-associated services:	They are related to the whole NG interface instance between the NG-RAN node and AMF, or between the NG-RAN node and the AIOTF, utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. NGAP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.7	Interface Management Procedures

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF, to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation with AMF

Figure 8.7.1.2-2: NG setup: successful operation with AIOTF

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF or the AIOTF. The AMF or the AIOTF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

//skip unchanged part

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

Upon receiving the Only Support A-IoT IE in the NG SETUP REQUEST message, the core network shall, if supported, consider that the NG-RAN node only provides A-IoT radio, and ignore the not applicable IEs as specified in clause 9.2.6.1.

Upon receiving AIOTF Identifier IE in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, ignore the IEs not applicable for AIOTF as specified in clause 9.2.6.2.

In case the NG-RAN node provides A-IoT radio, the NG-RAN node shall, if supported, inlcude the A-IoT RAN Information Transfer IE in the NG SETUP REQUEST message. Upon receiving the A-IoT RAN Information Transfer IE, the core network shall, if supported, use this information as specified in TS 23.xxx [xx].

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation with AMF

Figure 8.7.1.3-2: NG setup: unsuccessful operation with AIOTF

If the AMF or the AIOTF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

If the NG SETUP FAILURE message includes the Time to Wait IE, the NG-RAN node shall wait at least for the indicated time before reinitiating the NG Setup procedure towards the same AMF.

#### 8.7.1.4	Abnormal Conditions

If the AMF does not identify any of the PLMNs/SNPNs indicated in the NG SETUP REQUEST message, it shall reject the NG Setup procedure with an appropriate cause value.

If none of the RATs indicated by the NG-RAN node in the NG SETUP REQUEST message is supported by the AMF or the AIOTF, then the AMF or the AIOTF shall fail the NG Setup procedure with an appropriate cause value.

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF, or the NG-RAN node and the AIOTF, to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation with AMF

Figure 8.7.2.2-2: RAN configuration update: successful operation with AIOTF

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF or the AIOTF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF or the AIOTF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF or the AIOTF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

//skip unchanged part

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

Upon receiving the Only Support A-IoT IE in the RAN CONFIGURATION UPDATE message, the core network shall, if supported, consider that the NG-RAN node only provides A-IoT radio, and ignore the the not applicable IEs as specified in clause 9.2.6.4.

If the A-IoT RAN Information Transfer IE in the RAN CONFIGURATION UPDATE message, the core network shall overwrite all the previously stored A-IoT RAN information, if any.

#### 8.7.2.3	Unsuccessful Operation

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation with AMF

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation with AIOTF

If the AMF or the AIOTF cannot accept the update, it shall respond with a RAN CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node shall wait at least for the indicated time before reinitiating the RAN Configuration Update procedure towards the same AMF.

#### 8.7.2.4	Abnormal Conditions

If the NG-RAN node, after initiating the RAN Configuration Update procedure, receives neither a RAN CONFIGURATION UPDATE ACKOWLEDGE nor a RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node may reinitiate a further RAN Configuration Update procedure towards the same AMF or the same AIOTF, provided that the content of the new RAN CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged RAN CONFIGURATION UPDATE message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.7.4	NG Reset

#### 8.7.4.1	General

The purpose of the NG Reset procedure is to initialise or re-initialise the RAN, or part of RAN NGAP UE-related contexts, in the event of a failure in the 5GC or vice versa. This procedure does not affect the application level configuration data exchanged during, e.g., the NG Setup procedure. The procedure uses non-UE associated signalling.

#### 8.7.4.2	Successful Operation

##### 8.7.4.2.1	NG Reset initiated by the AMF or the AIOTF

Figure 8.7.4.2.1-1: NG reset initiated by the AMF: successful operation

Figure 8.7.4.2.1-2: NG reset initiated by the AIOTF: successful operation

In the event of a failure at the AMF or the AIOTF which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the NG-RAN node.

//skip unchanged part

Interactions with other procedures:

If the NG RESET message is received, any other ongoing procedure (except for another NG Reset procedure) on the same NG interface related to a UE association, indicated explicitly or implicitly in the NG RESET message, shall be aborted.

##### 8.7.4.2.2	NG Reset initiated by the NG-RAN node

Figure 8.7.4.2.2-1: NG reset initiated by the NG-RAN node: successful operation with AMF

Figure 8.7.4.2.2-2: NG reset initiated by the NG-RAN node: successful operation with AIOTF

In the event of a failure at the NG-RAN node which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the AMF and the AIOTF.

At reception of the NG RESET message the AMF shall release all allocated resources on NG related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the NGAP ID for the indicated UE associations.

At reception of the NG RESET message the AIOTF shall release all allocated NG resources.

After the AMF has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the AMF shall respond with the NG RESET ACKNOWLEDGE message.

//skip unchanged part

##### 8.7.4.4.3	Crossing of NG RESET Messages

If an NG Reset procedure is ongoing in the NG-RAN node and the NG-RAN node receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the NG-RAN node shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.1.

If an NG Reset procedure is ongoing in the AMF and the AMF receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the AMF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

If an NG Reset procedure is ongoing in the AIOTF and the AIOTF receives an NG RESET message from the peer entity on the same NG interface, the AIOTF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

### 8.7.5	Error Indication

#### 8.7.5.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE-associated signalling, then the Error Indication procedure uses UE-associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: Error indication initiated by the AMF

Figure 8.7.5.2-2: Error indication initiated by the NG-RAN nodeto the AMF

Figure 8.7.5.2-3: Error indication initiated by the AIOTF

Figure 8.7.5.2-4: Error indication initiated by the NG-RAN node to the AIOTF

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE-associated signalling the AMF UE NGAP ID IE and the RAN UE NGAP ID IE shall be included in the ERROR INDICATION message. If one or both of the AMF UE NGAP ID IE and the RAN UE NGAP ID IE are not correct, the cause shall be set to an appropriate value, e.g., "Unknown local UE NGAP ID" or "Inconsistent remote UE NGAP ID".

#### 8.7.5.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.6	Interface Management Messages

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF, NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. This IE is ignored if the Only Support A-IoT IE is included. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | This IE is ignored if the Only Support A-IoT IE is included. | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Only Support A-IoT | O |  | ENUMERATED (true, …) | This IE indicates that the NG-RAN node only provides A-IoT radio. | YES | reject |
| A-IoT RAN Information Transfer | O |  | OCTET STRING | Containing the A-IoT RAN Information Transfer IE specified in subclause 9.3.x.1. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF or the AIOTF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 | This IE is ignored if the AIOTF Identifier IE is included. | YES | reject |
| Served GUAMI List |  | 1 |  | This IE is ignored if the AIOTF Identifier IE is included. | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |
| AIOTF Identifier | O |  | 9.3.3.xx2 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

#### 9.2.6.3	NG SETUP FAILURE

This message is sent by the AMF or the AIOTF to indicate NG setup failure.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF, NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Only Support A-IoT | O |  | ENUMERATED (true, …) | This IE indicates that the NG-RAN node only provides A-IoT radio. | YES | reject |
| A-IoT RAN Information Transfer | O |  | OCTET STRING | Containing the A-IoT RAN Information Transfer IE specified in subclause 9.3.x.1. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

#### 9.2.6.5	RAN CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the AMF or the AIOTF to acknowledge the NG-RAN node transfer of updated information for an NG-C interface instance.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.6	RAN CONFIGURATION UPDATE FAILURE

This message is sent by the AMF or the AIOTF to indicate RAN configuration update failure.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.11	NG RESET

This message is sent by both the NG-RAN node and the AMF to request that the NG interface, or parts of the NG interface, be reset.

In case the gNB communicates with the AIOTF directly, this message is also sent by both the NG-RAN node and the AIOTF to request that the NG interface be reset.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >NG interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all, …) |  | - |  |
| >Part of NG interface |  |  |  |  |  |  |
| >>UE-associated Logical NG-connection List | M |  | 9.3.3.25 |  | - |  |

#### 9.2.6.12	NG RESET ACKNOWLEDGE

This message is sent by both the NG-RAN node and the AMF, or both the NG-RAN node and the AIOTF, as a response to an NG RESET message.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| UE-associated Logical NG-connection List | O |  | 9.3.3.25 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.13	ERROR INDICATION

This message is sent by both the NG-RAN node and the AMF, or both the NG-RAN node and the AIOTF, to indicate that some error has been detected in the node.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | O |  | 9.3.3.2 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aa1	Reader Index

This IE represents identifier of a Reader within a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Identity | M |  | BIT STRING (SIZE(16, …)) |  |

#### 9.3.1.dd2	Reader Location Information

This IE indicates the location information of the A-IoT Reader provided by the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reader Location | M |  |  |  |
| >Civic Address |  |  |  |  |
| >>Civic Address | M |  | OCTET STRING | Includes the CivicAddress as defined in TS 29.572 [xx]. |
| >NG-RAN Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | OCTET STRING | Includes the NG-RAN Access Point Position IE as defined in TS 38.455 [19]. |
| >NG-RAN High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | OCTET STRING | Includes the NG-RAN High Accuracy Access Point Position IE as defined in TS 38.455 [19]. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.xx2	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identitifer | M |  | OCTET STRING | Encoded as defined in TS 23.xxx [xx]. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x.x	A-IoT RAN Information Transfer

This IE provides the A-IoT RAN Information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Served Reader List |  | 1 |  |  |
| >Served Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Reader Location Information | O |  | 9.3.1.dd2 |  |
| Supported TA List [FFS] |  | 1 |  | Supported TAs dedicated for A-IoT in the NG-RAN node. |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  |
| >>TAC | M |  | 9.3.3.10 |  |
| >>PLMN List |  | 1 |  |  |
| >>>PLMN Item |  | 1..<maxnoofPLMNs> |  |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofInventoryAreas | Maximum no. of Inventory Areas supported by a gNB. Value is 256. |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

Editor’s Note:	It is FFS whether to introduce new “A-IoT Area” or use “A-IoT dedicated Tracking Area”.

-----------------Start of the Next  Change-----------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AIOTFIdentifier,

AIoT-OnlySupportAIoT,

AIoT-RANInformationTransfer,

AllowedNSSAI,

AMFName,

AMFSetID,

//skip unchanged part

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-A2X-PC5-QoS-Parameters,

id-AerialUEsubscriptionInformation,

id-AIOTFIdentifier,

id-AIoT-OnlySupportAIoT,

id-AIoT-RANInformationTransfer,

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

//skip unchanged part

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	}|

{ ID id-AIoT-OnlySupportAIoT		CRITICALITY reject	TYPE AIoT-OnlySupportAIoT			PRESENCE optional	}|

{ ID id-AIoT-RANInformationTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING AIoT-RANInformationTransfer)		PRESENCE optional	},

...

}

-- **************************************************************

--

###### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		}|

{ ID id-IAB-Supported				CRITICALITY ignore	TYPE IAB-Supported				PRESENCE optional		}|

{ ID id-Extended-AMFName			CRITICALITY ignore	TYPE Extended-AMFName			PRESENCE optional		}|

{ ID id-MobileIAB-Supported			CRITICALITY ignore	TYPE MobileIAB-Supported    	PRESENCE optional		}|

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier    		PRESENCE optional		},

...

}

//skip unchanged part

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}|

{ ID id-AIoT-OnlySupportAIoT					CRITICALITY reject	TYPE AIoT-OnlySupportAIoT							PRESENCE optional	}|

{ ID id-AIoT-RANInformationTransfer				CRITICALITY reject	TYPE OCTET STRING (CONTAINING AIoT-RANInformationTransfer)		PRESENCE optional	},

...

}

//skip unchanged part

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//skip unchanged part

maxnoofPLMNs,

maxnoofPLMNforQMC,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofReaders,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

//skip unchanged part

AdditionalCancelledlocationReportingReferenceIDItem ::= SEQUENCE {

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID,

iE-Extensions		ProtocolExtensionContainer { { AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFIdentifier ::= OCTET STRING

AIoT-OnlySupportAIoT ::= ENUMERATED {true, ...}

AIoT-PLMNList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF AIoT-PLMNItem

AIoT-PLMNItem ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

iE-Extensions		ProtocolExtensionContainer { { AIoT-PLMNItem-ExtIEs} } OPTIONAL,

...

}

AIoT-PLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-RANInformationTransfer ::= SEQUENCE {

globalgNB-ID				GlobalGNB-ID,

servedReaderList			AIoT-ServedReaderList,

supportedTAList				AIoT-supportedTAList,

iE-Extensions				ProtocolExtensionContainer { { AIoT-RANInformationTransfer-ExtIEs} }		OPTIONAL,

...

}

AIoT-RANInformationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-ReaderIndex ::= BIT STRING (SIZE(16, ...))

AIoT-ReaderLocation ::= CHOICE {

civicAddress								OCTET STRING,

nG-RANAccessPointPosition					OCTET STRING,

nG-RANHighAccuracyAccessPointPosition		OCTET STRING,

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-ReaderLocation-ExtIEs} }

}

AIoT-ReaderLocation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AIoT-ServedReaderList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ServedReaderItem

AIoT-ServedReaderItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

readerLocation				AIoT-ReaderLocation,

iE-Extensions		ProtocolExtensionContainer { { AIoT-ServedReaderItem-ExtIEs} } OPTIONAL,

...

}

AIoT-ServedReaderItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-supportedTAList ::= SEQUENCE (SIZE(1..maxnoofTACs)) OF AIoT-supportedTAItem

AIoT-supportedTAItem ::= SEQUENCE {

tAC							TAC,

pLMNList					AIoT-PLMNList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-supportedTAItem-ExtIEs} } OPTIONAL,

...

}

AIoT-supportedTAItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

//skip unchanged part

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofReaders								INTEGER ::= 65535

//skip unchanged part

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-InventoryRequestTransfer							ProtocolIE-ID ::= 993 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 994 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 995 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 996 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 997 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 998 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 999 --to be allocated

id-AIOTFIdentifier										ProtocolIE-ID ::= 881 --to be allocated

id-AIoT-OnlySupportAIoT									ProtocolIE-ID ::= 882 --to be allocated

id-AIoT-RANInformationTransfer						ProtocolIE-ID ::= 883 --to be allocated

-----------------End of the Changes-----------------