3GPP TSG-RAN WG3 Meeting #127-bis	R3-251684

Wuhan, China, 7th – 11st April 2025

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

In RAN3#126 meeting, one new class 1 procedure for providing UL WUS Configuration from NES Cell gNB to Cell A gNB and one new class 2 procedure for informing UL WUS configuration broadcast stop from Cell A gNB to NES Cell gNB were agreed.

In RAN3#127 meeting, we further agreed the followings:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

All the agreements are reflected in the BLCR[1].

This contribution further discusses the open issues and the corresponding RAN3 impacts of OD-SIB1.

# Discussion

## UL WUS configuration provision

Regarding the new defined class 1 procedure and the new class 2 procedure, one FFS is the names of the procedures and messages.

According to the purpose of the class 1 procedure, i.e. providing UL WUS Configuration from NES Cell gNB to Cell A gNB, it is suitable to name it as “UL WUS Configuration Provision” procedure. The three corresponding messages should be named as

- UL WUS Configuration Provision Request

- UL WUS Configuration Provision Acknowledge

Since the 2nd message is used by Cell A to acknowledge that it can broadcast UL WUS Configuration for the requested NES cell, “Acknowledge” is more appropriate than “Response”.

- UL WUS Configuration Provision Reject

Since the 3rd message is used by Cell A to inform that it cannot broadcast UL WUS Configuration for the requested NES cell, “Reject” is more appropriate than “Failure”.

Proposal 1: To name the class 1 procedure as “UL WUS Configuration Provision” procedure, and the three corresponding messages as “UL WUS Configuration Provision Request”, “UL WUS Configuration Provision Acknowledge” and “UL WUS Configuration Provision Reject” (to remove FFS in [1]).

Regarding the new class 2 procedure, according to the agreement made in last meeting, “A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB”, therefore, we suggest to directly name the procedure as “UL WUS Configuration Broadcast Stop Indication”.

Proposal 2: To rename the class 2 procedure and the corresponding message as “UL WUS Configuration Broadcast Stop Indication”.

Another open issue in last meeting is how to address the role of the NG-RAN node1 and NG-RAN node2 in the procedure description in [1]. Since each newly defined UL WUS Configuration related message is with a specific direction, it will cause confusion if a general node name, e.g. NG-RAN node1 and NG-RAN node2, is used in the procedure description. We think it should clearly identify the NG-RAN node1 that serves NES cell as NES-NG-RAN node and the NG-RAN node2 that serves Cell A as Cell A-NG-RAN node.

Proposal 3: To replace NG-RAN node1 with NES-NG-RAN node, NG-RAN node2 with Cell A-NG-RAN node in [1].

As agreed in RAN#105 meeting, for Rel-19 NES, support Case-2 on OD-SIB1:

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

There is one possible scenario for Case 2 that, for one NES Cell, multiple Cell A(s) are applicable. In this case, the NES Cell can select which Cell A(s) to broadcast UL WUS configuration for it, and it can select only one or several or all Cell A(s) from the applicable Cell A(s). Then the UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time. Afterwards, the NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1. This can all be up to implementation.

Proposal 4: In case of multiple Cell A(s) appliable for one NES Cell, the NES Cell can select which Cell A(s) for UL WUS configuration provision.

Proposal 5: UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time, which can be up to implementation.

Proposal 6: NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1, which can be up to implementation.

In addition, if a cell, based on its capability, can work as a Cell A, which means it can support UL WUS configuration broadcasting for NEC cell(s), it can exchange its UL WUS configuration broadcasting capability with neighbouring gNB in advance, e.g. during Xn setup procedure. This can greatly reduce the UL WUS Configuration Provision request failure rate.

Proposal 7: Cell A can exchange its UL WUS configuration broadcasting capability with neighbouring gNB during Xn setup procedure.

## OD-SIB1 activation/deactivation

The following three options for OD-SIB1 activation was proposed and discussed:

- Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.

- Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES Cell goes to OD-SIB Mode upon reception of an explicit “UL config provided” message from Cell A gNB.

- Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. NES Cell goes to OD-SIB Mode and Cell A gNB broadcasts received UL WUS configuration via its SIB upon transmission/reception of an explicit “UL WUS configuration broadcast indication” message from NES Cell gNB.

Option 1 was agreed to adopt in last meeting, here we further discuss Option 2 and Option 3. Comparing to Option 1, the difference for Option 2 and Option 3 is, after UL WUS Configuration Provision procedure, whether Cell A or NES Cell needs another indication to start UL WUS configuration broadcast or to activate OD-SIB1.

After the NES Cell provide its UL WUS configuration to cell A, the NES Cell can update Cell A if the current UL WUS configuration is changed or released. Meanwhile, the NES Cell can activate or deactivate (switch to legacy way) OD-SIB1 transmission at any time. If the NES Cell is in SIB1 broadcast, the UE can receive SIB1 in legacy way; if the NES Cell is in OD-SIB1, the UE will request cell A for the UL WUS configuration first and then trigger the NES Cell to send OD-SIB1. Besides, RAN2 already agreed that “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”. Therefore, we think it is no need to synchronize between OD-SIB1 in NES Cell and UL WUS configuration broadcast in Cell A, i.e., as stated in Option 1, after the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell can go to OD-SIB1 Mode at any time and Cell A can start to broadcast UL WUS configuration via its SIB at any time.

If NES Cell already goes to OD-SIB1 mode but Cell A has not started to broadcast UL WUS configuration, the NES Cell is regarded as barred for UE. As normally NES Cell is just used for capacity boost and there is always another coverage cell for UE, there will have not much impact on UE even if the NES Cell is not applicable for a period of time.

If Cell A has started to broadcast UL WUS configuration but NES Cell is still in legacy SIB1 broadcast mode, as RAN2 agreed, UE will check the NES Cell first and use the legacy SIB1 broadcasted from NES Cell without requesting OD-SIB1. Considering RAN2 agreed that “NES Cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration)”, the new SIB for UL WUS configuration can include UL WUS configuration of multiple NES Cells, so the signaling waste for broadcasting UL WUS configuration for a NES Cell which is not in OD-SIB1 mode is negligible.

Therefore, we think no further activation indication for OD-SIB1 after no matter from Cell A or NES Cell is needed.

Proposal 8: A further activation indication after UL WUS Configuration Provision procedure no matter from Cell A or NES Cell is not needed.

Besides, considering the OD-SIB1 activation/deactivation change can be dynamic and frequent, NES Cell may not need to inform cell A about the OD-SIB1 activation or deactivation.

Proposal 9: NES Cell does not need to inform cell A about the OD-SIB1 activation or deactivation.

If NES Cell needs to inform Cell A about the UL WUS configuration update or release, UL WUS Configuration Provision procedure can be reused to send an updated UL WUS configuration or a release indication.

Proposal 10: UL WUS Configuration Provision procedure can be used for UL WUS configuration update or release.

In specific, UL WUS Configuration Provision Request message can be used to send a new/updated UL WUS configuration or to inform a UL WUS configuration release. Therefore, besides UL WUS configuration, a Configuration Release Indication needs to be added in the UL WUS Configuration Provision Request message. The structure of this message can be as below:

#### 9.1.1.1	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NES-NG-RAN node to the Cell A-NG-RAN node to request the UL WUS configuration broadcasting for specific NES cell(s).

Direction: NES-NG-RAN node  Cell A-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| List of NES Cells | M | 1 .. <maxnoof CellsinNG -RAN node> |  |  | YES | reject |
| >NES Cell ID | M |  | 9.2.2.7 |  | YES | reject |
| >CHOICE UL WUS Configuration Provision | M |  |  |  | YES | reject |
| >>Configuration Provision |  |  |  |  |  |  |
| >>>UL WUS Configuration | M |  | 9.2.3.x |  |  |  |
| >> Configuration Release |  |  |  |  |  |  |
| >>>Configuration Release Indication | M |  | ENUMERATED (true, ...) |  |  |  |

Proposal 11: To add a Configuration Release Indication in the UL WUS Configuration Provision Request message for UL WUS configuration release.

As descripted above, there can be a scenario i.e. multiple NES Cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES Cells under that Cell A can activate OD-SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 12: Cell A can provide guidance to assist NES Cell on OD-SIB1 activation/deactivation.

In split architecture, considering gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES Cells and neighbor cells, we prefer gNB-CU to decide OD-SIB1 activation/deactivation of a NES Cell, or at least gNB-CU should provide guidance to assist gNB-DU on OD-SIB1 activation/deactivation of a NES Cell.

Proposal 13: gNB-CU decides or assists gNB-DU on the OD-SIB1 activation/deactivation of a NES Cell, and sends the request or assistance information to gNB-DU.

# Proposals

In this contribution, we addressed the open issues for OD-SIB1 of a NES Cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: To name the class 1 procedure as “UL WUS Configuration Provision” procedure, and the three corresponding messages as “UL WUS Configuration Provision Request”, “UL WUS Configuration Provision Acknowledge” and “UL WUS Configuration Provision Reject” (to remove FFS in [1]).

Proposal 2: To rename the class 2 procedure and the corresponding message as “UL WUS Configuration Broadcast Stop Indication”.

Proposal 3: To replace NG-RAN node1 with NES-NG-RAN node, NG-RAN node2 with Cell A-NG-RAN node in [1].

Proposal 4: In case of multiple Cell A(s) appliable for one NES Cell, the NES Cell can select which Cell A(s) for UL WUS configuration provision.

Proposal 5: UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time, which can be up to implementation.

Proposal 6: NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1, which can be up to implementation.

Proposal 7: Cell A can exchange its UL WUS configuration broadcasting capability with neighbouring gNB during Xn setup procedure.

Proposal 8: A further activation indication after UL WUS Configuration Provision procedure no matter from Cell A or NES Cell is not needed.

Proposal 9: NES Cell does not need to inform cell A about the OD-SIB1 activation or deactivation.

Proposal 10: UL WUS Configuration Provision procedure can be used for UL WUS configuration update or release.

Proposal 11: To add a Configuration Release Indication in the UL WUS Configuration Provision Request message for UL WUS configuration release.

Proposal 12: Cell A can provide guidance to assist NES Cell on OD-SIB1 activation/deactivation.

Proposal 13: gNB-CU decides or assists gNB-DU on the OD-SIB1 activation/deactivation of a NES Cell, and sends the request or assistance information to gNB-DU.

# Reference

[1] R3-250923, Introduction of Network Energy Saving Enhancement