3GPP TSG-RAN WG3 Meeting #127-bis	R3-252032

Wuhan, CN, 7 - 11 April, 2025

Agenda Item:	31.2

Source:	Ericsson

Title:	Discussion on E-CID new cell ID indication

Document for:	Discussion, Decision

# 1	Discussion

In Rel-17 TEI, the new CGI IE was added in the positioning Information Update message to support the case of SDT with partial anchor relocation during positioning.

In Rel-18 Positioning WI, with positioning with SRS in Validity Area, it was discussed that the LMF can be made aware by source gNB directly about the new cell ID where UE has resumed after RRC_INACTIVE mobility. When the old gNB receives the RETRIEVE UE CONTREXT RETRIEVE message and just before (fully or partially) relocating the context, it can indicate to LMF the cell ID of the new gNB, so that we save on latency, and don’t have to wait for LMF to know from AMF.

Currently TS 38.305 stipulates that all positioning methods can be supported during RRC_INACTIVE state:

7.9 Positioning in RRC_INACTIVE state Positioning may be performed when a UE is in RRC_INACTIVE state. Any uplink LCS or LPP message can be transported in RRC_INACTIVE state.

Therefore, for E-CID positioning, the old gNB should also be able to include New CGI when it closes the NRPPa E-CID session to LMF, and save on signalling. Below a signalling flow to illustrate:

- UE is in RRC_CONNECTED in serving gNB, it is CM_CONNECTED in CN

- gNB decides to put UE to RRC_INACTIVE, UE remains CM_CONNECTED in CN

- gNB receives E-CID MEASUREMENT INITIATION REQUEST message (can be periodic reporting request) from LMF.

- The UE in RRC_INACTIVE resumed to another gNB

- The receiving gNB triggers the Retrieve UE Context procedure towards the last serving gNB.

- The last serving gNB sends the NRPPa E-CID MEASUREMENT INITIAITON FAILURE (E-CID MEASUREMENT FAILURE INDICATION message in case of periodic) to the LMF.

Observation: For E-CID, the old gNB should also be able to include New CGI when it terminates the E-CID measurement reporting (on-demand or periodic) when the UE leaves the old serving cell and knows the new serving cell from the new gNB. This will improve the NRPPa feedback to LMF.

Proposal: Addition of New Cell Identity IE in the E-CID MEASUREMENT INITIATION FAILURE and E-CID MEASUREMENT FAILURE INDICATION messages.

A CR to NRPPa is avialable in [1].

# 3	Conclusions

Observation: For E-CID, the old gNB should also be able to include New CGI when it terminates the E-CID measurement reporting (on-demand or periodic) when the UE leaves the old serving cell and knows the new serving cell from the new gNB. This will improve the NRPPa feedback to LMF.

Proposal: Addition of New Cell Identity IE in the E-CID MEASUREMENT INITIATION FAILURE and E-CID MEASUREMENT FAILURE INDICATION messages.

# 4	References

- R3-252045, Indication of new cell ID for E-CID positioning [E-CID-enh2], Ericsson, CATT, ZTE, Polaris Wireless, NTT Docomo, Xiaomi