3GPP TSG-RAN WG3 #127bis	                                        R3-252089

Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1476 | Rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Xn support for inter-CU L1/L2 Triggered Mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Ltd (JPL) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Release 18 introduced L1/L2 Triggered Mobility (LTM) to significantly reduce handover interruption time by using lower-layer (Layer 1/2) signaling. However, the Release-18 solution only supports LTM when the source and target cells are controlled by the same gNB Central Unit (gNB-CU). In scenarios where a UE needs to perform an LTM-based handover to a cell under a different gNB-CU (i.e., inter-CU LTM), the current Xn interface procedures are insufficient. To fully realize the mobility and latency benefits of LTM across multi-gNB deployments, enhancements to the XnAP protocol are required. These changes will enable the source gNB-CU and target gNB-CU to coordinate L1/L2-triggered handovers with minimal signaling delay. In particular, new Xn signaling is needed to prepare the target gNB for a lower-layer triggered handover and to notify the target when the UE execution is triggered at Layer 1/2. Without these additions, the LTM feature cannot be utilized across gNB boundaries, limiting its applicability and performance gains in heterogeneous network scenarios (e.g., different CUs). This CR implements the necessary protocol support on Xn to facilitate inter-CU LTM, reflecting the agreements reached under RAN3#127 (Agenda Item 13.2) for this work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces a new XnAP procedure and associated Information Elements (IEs) to support inter-CU L1/L2 triggered mobility: New XnAP “LTM Cell Switch Notification” procedure: A class-2 (unsolicited) XnAP message is added for the source gNB-CU to notify the target gNB-CU when an LTM execution is triggered. This message carries the identity of the target cell and related configuration (e.g., target beam/TCI state) that the UE will use, allowing the target to complete the handover seamlessly at Layer 1/2. No explicit acknowledgment message is required for this notification (consistent with a class 2 procedure). Enhancements to Xn Handover Preparation: The existing XnAP Handover Request/Acknowledgement messages are extended to support LTM-specific information. A new cause value is introduced (e.g., “ltm-triggered”) to indicate that a handover is being prepared due to an L1/L2 trigger. This informs the target gNB that the preparation is for an LTM procedure, so it can handle the context accordingly (e.g., maintain UE context without immediate RRC reconfiguration). Additionally, the Handover Acknowledge is augmented to include the Target TCI State ID (Transmission Configuration Indicator state identifier) assigned by the target cell for the UE. This allows the source gNB to direct the UE’s beam/channel synchronization to the target cell prior to the final switch. ASN.1 updates: New identifiers and IE definitions are added to the ASN.1 module of 38.423 for the LTM Cell Switch Notification message and the associated IEs (such as the target cell ID and TCI state ID). The Cause enumeration is updated with the new cause value for LTM. Placeholders for new procedure and IE IDs are included, to be assigned by MCC. In summary, these changes equip the Xn interface with the necessary signaling to coordinate inter-gNB lower-layer triggered handovers. The source gNB-CU can prepare one or multiple target gNB-CUs for a possible LTM handover and, when the L1/L2 trigger occurs, promptly notify the chosen target, resulting in a fast handover execution with minimal interruption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In practice, UEs moving from a cell under one gNB-CU to a cell under another gNB-CU would be unable to benefit from the reduced interruption time provided by L1/L2 triggered mobility. The network would have to fall back to traditional layer-3 handover procedures in these cases, incurring higher latency (possibly up to tens of milliseconds more interruption) and higher signaling overhead. This would undermine the gains of the LTM feature in multi-vendor or multi-CU deployments and could lead to performance inconsistencies: UEs may experience very fast handovers within a gNB but slower, less efficient handovers across gNBs. In the worst case, lack of support could cause handover failures or radio link failures in scenarios where a rapid LTM-based mobility is expected (for example, in ultra-dense networks or industrial use-cases requiring very low latency mobility). Approving this change is therefore critical to ensure a consistent and optimized mobility performance across all gNB boundaries in Release-19 networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.7, 8.3.8, 9.1.4.15–9.1.4.19, 9.1.8.4, 9.1.8.5, 9.2.1 (new IE defs), 10 (ASN.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

A2X	Aircraft-to-Everything

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LBT	Listen-Before-Talk

LTM	L1/L2 Triggered Mobility (Lower Layer Triggered Mobility)

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

MT-SDT	Mobile Terminated Small Data Transmission

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

RSPP	Ranging/SL Positioning Protocol

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-CPAC	Subsequent CPAC

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

-------------------------------------------Next changes-------------------------------------------

# 9	Elements for XnAP Communication

## 9.0	General

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the XnAP protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [5].

NOTE1:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [6].

NOTE2:  When a source gNB-CU decides to initiate a handover based on an L1/L2 trigger (Lower Layer Triggered Mobility) towards a cell served by a different gNB-CU, it shall indicate this in the XnAP Handover Request message. In this case, the Handover Request includes a cause value set to “ltm-triggered” to inform the target gNB-CU that the handover is L1/L2-triggered. The target gNB-CU, upon receiving a Handover Request with cause “ltm-triggered”, shall prepare the UE context and necessary resources for the target cell, but shall not immediately expect UE relocation via RRC. Instead, the target gNB-CU stores the context and awaits an explicit LTM execution notification from the source gNB-CU (as defined below) before completing the handover. The target gNB-CU may also provide information to facilitate lower-layer synchronization (e.g., a target cell Transmission Configuration Indicator state) in its Handover Acknowledge message, as described in the next subclause.

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. NOTE: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| >NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| >LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.2.3.108 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. | YES | reject |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. | YES | reject |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED(authorized, not authorized, ...) | This IE indicates the authorization status of the IAB node. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) | This IE indicates that information on DL LBT Failures occurring at the target gNB that results in mobility failure is requested | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.2.3.175 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.2.3.176 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.2.3.177 |  | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.2.3.178 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.180 | This IE contains information about cells that a UE is predicted to be connected to. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| Candidate Relay UE Info List | O |  | 9.2.3.188 |  | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Mobile IAB Authorization Status | O |  | 9.2.2.105 |  | YES | reject |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Cause | M |  |  | Indicates the reason for initiating the handover (ltm-triggered) |  |  |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| Target TCI State ID | O | Integer (value range [TBD]) |  | Identifier of the TCI state (beam ID) in the target cell to be used for LTM pre-synchronization. This IE is included if the handover was triggered by L1/L2 (ltm-triggered). |  | ignore |

-------------------------------------------Next changes-------------------------------------------

#### 9.X	LTM Cell Switch Notification procedure

When the L1/L2 execution condition is met for an inter-gNB-CU handover (after prior preparation of the UE context at the target gNB-CU via Handover Request/Ack), the source gNB-CU shall trigger the LTM execution towards the target cell and inform the target gNB-CU that the UE is switching to the target cell at lower layers. This is done by sending an LTM Cell Switch Notification message over the Xn interface. The LTM Cell Switch Notification is an XnAP initiating message (class 2 procedure) sent by the source gNB-CU to the target gNB-CU, with the following Information Elements:

Source gNB-CU UE XnAP ID – Identifies the UE context at the source gNB (as allocated in the earlier Handover Request). (M)

Target gNB-CU UE XnAP ID – Identifies the UE context at the target gNB (as allocated in the Handover Acknowledge). (M)

Target Cell ID – The NR CGI (NR Cell Global Identity) of the target cell to which the UE is performing LTM handover. (M)

Target TCI State ID – The Transmission Configuration Indicator state that was used for downlink synchronization with the target cell. (M)

Cause – [Optional] If the LTM execution fails or cannot be completed as intended, a cause value may be included (e.g., to indicate resource shortage). (O, conditional)

Upon reception of the LTM Cell Switch Notification, the target gNB-CU shall consider that the UE has now effectively moved to the target cell (Layer 1/2 handover completed). The target gNB-CU can then proceed with the remaining steps of handover completion, such as initiating the path switch towards the core network (NG Path Switch Request) and confirming UE context release from the source. The source gNB-CU, after sending the notification, shall await either an Xn UE Context Release from the target or an NG RAN release from the core, as per existing handover completion procedures. Overall message flow: The source gNB-CU uses the normal Handover Request/Acknowledge procedure to prepare the target gNB-CU with UE context (with cause “ltm-triggered”). When the UE is commanded (via lower layers or RRC) to perform the LTM handover, the source gNB-CU sends LTM Cell Switch Notification. No response is expected for this message. Standard Xn UE context release and data forwarding procedures (e.g., Xn SN Status Transfer) apply thereafter as in a normal handover, with the timing aligned to the LTM execution.

| Information Element | M/O | Criticality | Type / Reference | Notes |
| --- | --- | --- | --- | --- |
| Source UE XnAP ID | M | reject | UE XnAP ID (source) | Identifies the UE at the source gNB-CU (as assigned in preparation phase). |
| Target UE XnAP ID | M | reject | UE XnAP ID (target) | Identifies the UE at the target gNB-CU (as assigned in preparation phase). |
| Target Cell ID | M | ignore | NRGCI (Global Cell ID) | NR CGI of the target cell for the LTM handover execution. |
| Target TCI State ID | M | ignore | Integer (0..maxTCI-1) [START EDIT][Note 1][END EDIT] | The index of the TCI state (beam ID) that the UE used for synchronization to the target cell. |
| Cause | O | ignore | Cause | If included, indicates reason for any abnormality (e.g., failure) in LTM execution. Not present for normal execution notification. |

-------------------------------------------Next changes-------------------------------------------

### 9.3.5	Information Element definitions

Cause ::= CHOICE {

radioNetwork		CauseRadioNetworkLayer,

transport			CauseTransportLayer,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-Single-Container { {Cause-ExtIEs} }

}

Cause-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CauseRadioNetworkLayer ::= ENUMERATED {

cell-not-available,

handover-desirable-for-radio-reasons,

handover-target-not-allowed,

invalid-AMF-Set-ID,

no-radio-resources-available-in-target-cell,

partial-handover,

ltm-triggered,               -- L1/L2 triggered mobility handover

reduce-load-in-serving-cell,

resource-optimisation-handover,

time-critical-handover,

tXnRELOCoverall-expiry,

tXnRELOCprep-expiry,

unknown-GUAMI-ID,

unknown-local-NG-RAN-node-UE-XnAP-ID,

inconsistent-remote-NG-RAN-node-UE-XnAP-ID,

encryption-and-or-integrity-protection-algorithms-not-supported,

not-used-causes-value-1,

multiple-PDU-session-ID-instances,

unknown-PDU-session-ID,

unknown-QoS-Flow-ID,

multiple-QoS-Flow-ID-instances,

switch-off-ongoing,

not-supported-5QI-value,

tXnDCoverall-expiry,

tXnDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

resources-not-available-for-the-slice-s,

ue-max-IP-data-rate-reason,

cP-integrity-protection-failure,

uP-integrity-protection-failure,

slice-not-supported-by-NG-RAN,

mN-Mobility,

sN-Mobility,

count-reaches-max-value,

unknown-old-NG-RAN-node-UE-XnAP-ID,

pDCP-Overload,

drb-id-not-available,

unspecified,

...,

ue-context-id-not-known,

non-relocation-of-context,

cho-cpc-resources-tobechanged,

rSN-not-available-for-the-UP,

npn-access-denied,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

ue-power-saving,

not-existing-NG-RAN-node2-Measurement-ID,

insufficient-ue-capabilities,

normal-release,

value-out-of-allowed-range,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission,

ssb-not-available,

lTM-triggered,

no-Backhaul-Resource,

mIAB-node-not-authorized,

iAB-not-authorized

}

-- Definition of new IE: Target TCI State ID

TargetTCIStateID ::= INTEGER (0..255)

-- Range TBD based on max number of configured TCI states.

id-TargetTCIStateID OBJECT IDENTIFIER ::= { xnap-IEs [START EDIT][TBD_Id1][END EDIT] }

-- Procedure Code for LTM Cell Switch Notification

id-LTMCellSwitchNotification INTEGER ::= [START EDIT][TBD_Id2][END EDIT]  -- (value to be assigned for this procedure)

LTMCellSwitchNotification ::= SEQUENCE {

protocolIEs    ProtocolIE-Container {{LTMCellSwitchNotification-IEs}},

...

}

-- Define the container of IEs for LTMCellSwitchNotification

LTMCellSwitchNotification-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-SourceUE_XnAP_ID        CRITICALITY reject  TYPE UE_XnAP_ID                                               PRESENCE mandatory } |

{ ID id-TargetUE_XnAP_ID        CRITICALITY reject  TYPE UE_XnAP_ID                                               PRESENCE mandatory } |

{ ID id-NRCellIdentifier        CRITICALITY ignore  TYPE NRCellIdentifier                                        PRESENCE mandatory } |

{ ID id-TargetTCIStateID        CRITICALITY ignore  TYPE TargetTCIStateID                                        PRESENCE mandatory } |

{ ID id-Cause                   CRITICALITY ignore  TYPE Cause                                                   PRESENCE optional } ,

...

}

-- Add the procedure to the list of XnAP initiated procedures (initiating message only, no response expected)

XNAP-PROCEDURES ::= {

...

{ INITIATING MESSAGE      LTMCellSwitchNotification

PROCEDURE CODE          id-LTMCellSwitchNotification

CRITICALITY             ignore

} ,

...

}

-------------------------------------------End of changes-------------------------------------------