3GPP TSG-RAN WG3 #127-bis					 				                                  R3-252243

Wuhan, China, 7-11 April 2025

Title: 	(TP to BL CR 38.300, 38.410) New Ambient IoT procedures

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

The TPs are based on the following proposals.

Proposal 1: Introduce AIoT service cancel procedure.

Proposal 2: Introduce AIoT reader Status Reporting procedure.

- TP to TS38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

16.xx.xa	AIoT reader Status reporting procedure

Figure 16.xx.xa-1 depicts the basic communication between the gNB and the A-IoT CN node for the AIoT reader Status reporting procedure.

Editor’s Note x:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: AIoT reader Status reporting procedure

1.	The A-IoT CN node initiates the AIoT reader Status reporting procedure over NG-C by sending the AIoT reader Status Request message to the gNB.

Editor’s Note 4:	Specification of further parameterisation of the AIoT reader Status Request message needs further work.

2.	The gNB confirms the request from the A-IoT CN node by replying with the AIoT reader Status Response message.

NOTE 2:	If the gNB is not able to perform the AIoT reader Status reporting procedure, it rejects the request and sends an AIoT reader Status Failure message to the A-IoT CN node.

3./4.	The gNB sends the AIoT reader Status reporting message to the A-IoT CN node. If the AIoT reader Status reporting procedure concerns e.g., reporting periodicity, multiple AIoT reader Status Reports may be sent to the A-IoT CN node.

16.xx.xb	AIoT service cancellation procedure

Figure 16.xx.xb-1 depicts the basic communication between the gNB and the A-IoT CN node for the AIoT service cancellation procedure.

Editor’s Note 5:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.xb-1: AIoT service cancellation procedure

1.	The A-IoT CN node initiates the AIoT service cancellation procedure over NG-C by sending the AIoT service Cancel message to the gNB.

3.	The gNB releases the usage of A-IoT radio resources and performs the AIoT service cancellation procedure over the A-IoT radio interface.

4.	The gNB replies to the AIoT service cancel with the AIoT service cancel acknowledge message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- TP to TS38.410

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.xx1	AIoT procedures

The following procedures are used for AIoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	AIoT reader Status;

-	AIoT reader Status Report;

-	AIoT service Cancellation.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>