3GPP TSG-RAN WG3 Meeting #127	R3-250797

Athens, Greece, 17 - 21 February 2025

Agenda Item:	12.3

Source:	Huawei, Lenovo, Nokia

Title:	(TP for BL CR for TS 38.300) Functional of AMF for NR Femto

Document for:	Discussion

1	Introduction

This paper provides the TP for capturing the functional supported by the AMF.

Annex 1——TP for BL CR for TS 38.300

Start of Change

4	Overall Architecture and Functional Split

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

4.X.2	Functional Split

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE: NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-	The TAC and PLMN ID(s) used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer messages towards NR Femto GWs based on the Selected TAI contained in these messages.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.

End of Change