| TSG-RAN Working Group3 meeting#14 | TSGR3#14(00)1979 |
| --- | --- |
| Helsinki, 3-7 July 2000 |  |
| Agenda Item: 21.3 |  |
| Source: Motorola |  |

Changes to 25.435 required to support the signaling of the ‘DSCH DL signaling frame’ from the CRNC to the Node B

# Introduction

This paper describes the changes on 25.435 required to support the signaling of TFCI2 from the Drift CRNC to the Node B.  An overview of all the required changes is given in [1], whilst [2] provides the related change requests on NBAP.

# References

[1] TSGR3#14(00)1699, Motorola, 'Configuration of transport bearers to carry FDD mode TFCI2 information '  3-7 July 2000

[2] TSGR3#14(00)1701, Motorola, 'Changes to NBAP to support the signaling of the ‘DSCH DL signaling frame’ from the CRNC to the Node B’'  3-7 July 2000

| 3GPP TSG-RAN Meeting #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | RP-000 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 25.435 |  |  |  |  |  | CR |  |  | 022 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | TSG-RAN#9 |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | x | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Motorola |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | June 2000 |  |  |  |  |  |
| Subject: |  |  | Enhancement to 25.435 to support FDD mode DSCH scheduling from the DRNC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | The CR provides the changes necessary to support the scheduling of the DSCH at the DRNC. A description of the DSCH DL signalling control procedure and frame format is included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clauses affected: |  |  |  |  | - 5, 5.4, 6.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Other specs |  | - Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  | -  List of CRs: |  |  |  |  |  |  | - 25.433 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - affected: |  | - Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  | -  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  | -  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  | -  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- <--------- double-click here for help and instructions on how to create a CR.

## 5.4	DL Timing Adjustment

Timing Adjustment procedure is used to indicate for the CRNC the incorrect arrival time of downlink data to node B.

Timing adjustment procedure is initiated by the node B if a DL frame arrives outside of the defined arrival window.

If the DL frame has arrived before the ToAWS or after the ToAWE nodeB includes the ToA and the target CFN as message parameters for TIMING ADJUSTMENT Control Frame.

The arrival window and the time of arrival are defined as follows:

-	Time of Arrival Window Endpoint (ToAWE): ToAWE represents the time point by which the DL data shall arrive to the node B from Iub. The ToAWE is defined as the amount of milliseconds before the last time point from which a timely DL transmission for the identified CFN would still be possible taking into account the node B internal delays. ToAWE is set via control plane. If data does not arrive before ToAWE a Timing Adjustment Control Frame shall be sent by node B.

-	Time of Arrival Window Startpoint (ToAWS): ToAWS represents the time after which the DL data shall arrive to the node B from Iub. The ToAWS  is defined as the amount of milliseconds from the ToAWE. ToAWS is set via control plane. If data arrives before ToAWS a Timing Adjustment Control Frame shall be sent by node B.

Time of Arrival (ToA): ToA is the time difference between the end point of the DL arrival window (ToAWE) and the actual arrival time of DL frame for a specific CFN. A positive ToA means that the frame is received before the ToAWE, a negative ToA means that the frame is received after the ToAWE.

[FDD - For the FACH, PCH and DSCH the timing adjustment is made relative to the Cell Frame Number whilst for the DSCH DL signalling control frame adjustment is made relative to the Connection Frame Number on the associated DCH.]

The general overview on the timing adjustment procedure is reported in [2].

Figure 9: FACH, PCH, DSCH and [FDD - DSCH TFCI signalling] Timing Adjustment procedure

## 5.5	[TDD – Dynamic PUSCH assignment]

Procedure for dynamic allocation of physical resources to uplink shared channels (USCH) in the NodeB. The control frame includes a parameter “PUSCH Set Id” which is a pointer to a preconfigured table of PUSCH Sets in the NodeB.

When this control frame is sent via a certain Iub USCH data port, then it applies to that USCH and in addition to any other USCH channel which is multiplexed into the same CCTrCH in the NodeB.

The time limitation of the PUSCH allocation is expressed with the parameters “Activation CFN” and “Duration”.

NodeB behaviour: When the NodeB receives the control frame “Dynamic PUSCH assignment” from the CRNC in the USCH frame protocol over an Iub USCH data port within a Traffic Termination Point, it shall behave as follows:

1)	The NodeB shall extract the PUSCH Set Id.

2)	It shall extract the parameters “Activation CFN” and “Duration” which identify the allocation period of that physical channel.

3)	It shall retrieve the PUSCH Set which is referred to by the PUSCH Set Id.

4)	It shall identify the CCTrCH to which the USCH is multiplexed, and hence the TFCS which is applicable for the USCH.

5)	Within the time interval indicated by Activation CFN and Duration, the NodeB shall make the specified PUSCH Set available to the CCTrCH.

Figure 10: Dynamic PUSCH assignment procedure

## 5.6	DSCH TFCI Signalling [FDD]

This procedure is used in order to signal to the node B the TFCI (field 2). This allows the node B to build the TFCI word(s) which have to be transmitted on the DPCCH.

The procedure consists in sending the DSCH TFCI signalling control frame from the CRNC to the node B. The frame contains the TFCI (field 2) and the correspondent Connection Frame Number. The DSCH TFCI signalling frame is sent once every Uu frame interval (10 ms) for as long as there is DSCH data for that UE to be transmitted in the associated PDSCH Uu frame. In the event that the node B does not receive a DSCH TFCI signalling control frame then the node B shall infer that no DSCH data is to be transmitted to the UE on the associated PDSCH Uu frame and will build the TFCI word(s) accordingly.

Figure 11: DSCH TFCI Signalling

#### 6.3.3.6	[TDD – Dynamic PUSCH assignment]

##### 6.3.3.6.1	Payload structure

The payload of the Dynamic PUSCH Assignment control frames is shown in the figure below:

##### 6.3.3.6.2	PUSCH Set Id

Description: Identifies a PUSCH Set from the collection of PUSCH Sets which have been preconfigured in the NodeB, for the respective cell in which the USCH exists. The PUSCH Set Id is unique within a cell.

Value range: 0...255.

Field length: 8 bits.

##### 6.3.3.6.3	Activation CFN

Description: Activation CFN, specifies the Connection Frame Number where the allocation period of that PUSCH Set starts.

Value range: Integer (0...255).

Field length: 8 bits.

##### 6.3.3.6.4	Duration

Description: Indicates the duration of the activation period of the PUSCH Set, in radio frames.

Value range: 0 ... 255 means: 0 to 255 radio frames, i.e. 0 to 2550 msec.

Field length: 8 bits.

#### 6.3.3.7	[FDD - DSCH TFCI signalling]

##### 6.3.3.7.1	Payload structure

The figure below shows the structure of the payload when the control frame is used for signalling TFCI (field 2) bits. The TFCI (field 2) bits are used by the node B to create the TFCI word(s) for transmission on the DPCCH.

Figure 11: [FDD - Structure of the payload for the DSCH TFCI signalling control frame]

##### 6.3.3.7.2	TFCI (field 2)

Description: TFCI (field 2) is as described in [6], it takes the same values as the TFCI (field 2) which is transmitted over the Uu interface.

Value range:   {0 - 1023}

Field length:  10 bits

##### 6.3.3.7.3	Spare Extension

The Spare Extension is described in subclause 6.2.7.19