| 3GPP TSG RAN WG3 Meeting #14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | R3-001980 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Helsinki, Finland, 3rd July – 7th July 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TS 25.423 |  |  |  |  |  |  |  | CR |  |  | 147r2 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | RAN#9 |  |  |  |  |  | For approval |  |  |  |  |  |  | X |  |  | Strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  |  | For information |  |  |  |  |  |  |  |  |  |  |  |  | Non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | InterDigital |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 07/2000 |  |  |  |  |  |
| Subject: |  |  | Shared Channel Signalling Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| Shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| With an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | The shared channel UE or Cell based TFS handling is signalled when a new radio link is established, however in Radio Link addition and Synchronised Radio Link Reconfiguration a DSCH/USCH can be established for the first time for this user in a new cell so these procedures must also signal the TFS handling. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 9.1.7, 9.1.12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  | X |  List of CRs: |  |  |  |  |  |  | 25.331 CR 416r1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.1.7	RADIO LINK ADDITION RESPONSE

#### 9.1.7.1	FDD Message

| IE/Group Name | Presence |  | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  |  |  | 9.2.1.59 |  | – |  |
| RL Information Response |  |  | 1..<maxnoofRLs-1> |  |  |  | EACH | ignore |
| >RL ID | M |  |  |  | 9.2.1.49 |  | – |  |
| >RL Set ID | M |  |  |  | 9.2.2.35 |  | – |  |
| >SAI | M |  |  |  | 9.2.1.52 |  | – |  |
| >Cell GAI | O |  |  |  |  |  | – |  |
| >UTRAN Access Point Position | O |  |  |  |  |  | – |  |
| >UL Interference Level | M |  |  |  | 9.2.1.68 |  | – |  |
| >Secondary CCPCH Info |  |  | 0..1 |  |  |  | – |  |
| >>FDD S-CCPCH Offset | M |  |  |  | 9.2.2.15 | Corresponds to: S-CCPCH,k , see ref. [8] | – |  |
| >>DL Scrambling Code | M |  |  |  | 9.2.2.8 |  | – |  |
| >>FDD DL Channelisation Code Number | M |  |  |  | 9.2.2.14 |  | – |  |
| >>TFCS | M |  |  |  | 9.2.1.63 | For the DL. | – |  |
| >>Secondary CCPCH Slot Format | M |  |  |  | 9.2.2.38 |  | – |  |
| >>TFCI presence | C - SlotFormat |  |  |  | 9.2.1.55 |  | – |  |
| >>Multiplexing Position | M |  |  |  | 9.2.2.26 |  | – |  |
| >>STTD Indicator | M |  |  |  | 9.2.2.44 |  | – |  |
| >>FACH/PCH Information |  |  | 1 .. <maxFACHcount+1> |  |  |  | – |  |
| >>>TFS |  |  |  |  | 9.2.1.64 | For each FACH, and the PCH when multiplexed on the same Secondary CCPCH | – |  |
| >>Scheduling Information |  |  | 1 |  |  |  | – |  |
| >>>IB_SG_EP | M |  |  |  | 9.2.2.21 |  | – |  |
| >>>Segment Information |  |  | 1.. <maxIBSEG> |  |  |  | – |  |
| >>>>IB_SG_POS | M |  |  |  | 9.2.2.20 |  | – |  |
| >DL Code Information |  |  | 1..<maxnoofDLCodes> |  |  |  | GLOBAL | ignore |
| >>DL Scrambling Code | M |  |  |  | 9.2.2.8 |  | – |  |
| >>FDD DL Channelisation Code Number | M |  |  |  | 9.2.2.14 |  | – |  |
| >>Transmission Gap Pattern Sequence Information Response | O |  |  |  |  |  | – |  |
| >Diversity Indication | M |  |  |  | 9.2.2.7 |  | YES | ignore |
| >CHOICE diversity indication |  |  |  |  |  |  |  |  |
| >>Combining |  |  |  |  |  |  | YES | ignore |
| >>>RL ID | M |  |  |  | 9.2.1.49 | Reference RL-Id | – |  |
| >>Non combining |  |  |  |  |  |  | YES | ignore |
| >>>DCH Information Response |  |  | 1..<maxnoofDCHs> |  |  | Only one DCH per set of co-ordinated DCHs shall be included. | – |  |
| >>>>DCH ID | M |  |  |  | 9.2.1.16 |  | – |  |
| >>>>Binding ID | M |  |  |  | 9.2.1.3 |  | – |  |
| >>>>Transport Layer Address | M |  |  |  | 9.2.1.62 |  | – |  |
| >SSDT Support Indicator | M |  |  |  | 9.2.2.43 |  | – |  |
| >Minimum Uplink SIR | M |  |  |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Maximum Uplink SIR | M |  |  |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Closed loop timing adjustment mode |  | O |  |  |  |  | - |  |
| >Maximum Allowed UL Tx Power |  | M |  |  | 9.2.1.35 |  | – |  |
| >Neighbouring Cell Information |  |  |  | 0..<maxnoofneighbouringRNCs> |  |  | EACH | ignore |
| >>RNC-Id |  | M |  |  | 9.2.1.50 |  | – |  |
| >>CN PS Domain Identifier |  | O |  |  | 9.2.1.12 |  | – |  |
| >>CN CS Domain Identifier |  | O |  |  | 9.2.1.11 |  | – |  |
| >>Per FDD Cell Information |  |  |  | 0..<maxnoofFDDneighbours> |  |  | – |  |
| >>>C-Id |  | M |  |  | 9.2.1.6 |  | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nu in ref. [6] | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nd in ref. [6] | – |  |
| >>>Frame Offset |  | O |  |  | 9.2.1.30 |  | – |  |
| >>>Primary Scrambling Code |  | M |  |  | 9.2.1.45 |  | – |  |
| >>>Primary CPICH Power |  | O |  |  | 9.2.1.44 |  | – |  |
| >>>Cell Individual Offset |  | O |  |  | 9.2.1.7 |  | – |  |
| >>>Tx Diversity Indicator |  | M |  |  | 9.2.2.50 |  | – |  |
| >>>STTD Support Indicator |  | O |  |  | 9.2.2.45 |  | – |  |
| >>>Closed Loop Mode1 Support Indicator |  | O |  |  | 9.2.2.2 |  | – |  |
| >>>Closed Loop Mode2 Support Indicator |  | O |  |  | 9.2.2.3 |  | – |  |
| >>Per TDD Cell Information |  |  |  | 0..<maxnoofTDDneighbours> |  |  | – |  |
| >>>C-Id |  | M |  |  | 9.2.1.6 |  | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nt in ref. [7] | – |  |
| >>>Frame Offset |  | O |  |  | 9.2.1.30 |  | – |  |
| >>>Cell Parameter ID |  | M |  |  | 9.2.1.8 |  | – |  |
| >>>Sync Case |  | M |  |  | 9.2.1.54 |  | – |  |
| >>>Time Slot |  | C-Case1 |  |  | 9.2.1.56 |  | – |  |
| >>>SCH Time Slot |  | C-Case2 |  |  | 9.2.1.51 |  | – |  |
| >>>Block STTD Indicator |  | M |  |  |  |  | – |  |
| >>>Cell Individual Offset |  | O |  |  | 9.2.1.7 |  | – |  |
| >>>DPCH Constant Value |  | O |  |  | 9.2.1.23 |  | – |  |
| >>>PCCPCH Power |  | O |  |  | 9.2.1.43 |  | – |  |
| Criticality Diagnostics |  | O |  |  | 9.2.1.13 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2 | This IE is present only if Sync Case = Case2. |
| SlotFormat | This IE is present only if the Secondary CCPCH Slot Format is equal to any of the value 8 to 17 |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of dedicated channels on one RL |
| MaxnoofRLs | Maximum number of radio links for one UE |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDNeighbours | Maximum number of neighbouring FDD cells for one cell |
| MaxnoofTDDNeighbours | Maximum number of neighbouring TDD cells for one cell |
| MaxnoofDLCodes | Maximum number of DL code information |
| MaxFACHCount | Maximum number of FACH's mapped onto secondary CCPCH's |
| MaxIBSEG | Maximum number of segments for one Information Block |

#### 9.1.7.2	TDD Message

| IE/Group Name | Presence |  | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  |  |  | 9.2.1.59 |  | – |  |
| RL Information Response |  |  | 1 |  |  |  | YES | ignore |
| >RL ID | M |  |  |  | 9.2.1.49 |  | – |  |
| >SAI | M |  |  |  | 9.2.1.52 |  | – |  |
| >Cell GAI | O |  |  |  |  |  | – |  |
| >UTRAN Access Point Position | O |  |  |  |  |  | – |  |
| >UL Interference per Time Slot |  |  | 1 .. <maxnoofULts> |  |  | Interference Level for each UL time slot within the Radio Link | – |  |
| >>Time Slot | M |  |  |  | 9.2.1.56 |  | – |  |
| >>UL Interference Level | M |  |  |  | 9.2.1.68 |  | – |  |
| >UL CCTrCH Information |  |  | 0..<maxnoof CCTrCHs> |  |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  |  |  | 9.2.3.2 |  | – |  |
| >>UL DPCH Information |  |  | 1..<maxnoOfDPCHs> |  |  |  | EACH | ignore |
| >>>DPCH ID | M |  |  |  | 9.2.3.3 |  | – |  |
| >>>TDD Channelisation Code | M |  |  |  | 9.2.3.8 |  | – |  |
| >>>Burst Type | M |  |  |  | 9.2.3.1 |  | – |  |
| >>>Midamble Shift | M |  |  |  | 9.2.3.4 |  | – |  |
| >>>Time Slot | M |  |  |  | 9.2.1.56 |  | – |  |
| >>>TDD Physical Channel Offset | M |  |  |  | 9.2.3.9 |  | – |  |
| >>>Repetition Period | M |  |  |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  |  |  | 9.2.3.6 |  | – |  |
| >>>TFCI Presence | M |  |  |  | 9.2.1.55 |  | – |  |
| >DL CCTrCH Information |  |  | 0..<maxnoof CCTrCHs> |  |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  |  |  | 9.2.3.2 |  | – |  |
| >>DL DPCH Information |  |  | 1..<maxnoOfDPCHs> |  |  |  | EACH | ignore |
| >>>DPCH ID | M |  |  |  | 9.2.3.3 |  | – |  |
| >>>TDD Channelisation Code | M |  |  |  | 9.2.3.8 |  | – |  |
| >>>Burst Type | M |  |  |  | 9.2.3.1 |  | – |  |
| >>>Midamble Shift | M |  |  |  | 9.2.3.4 |  | – |  |
| >>>Time Slot | M |  |  |  | 9.2.1.56 |  | – |  |
| >>>TDD Physical Channel Offset | M |  |  |  | 9.2.3.9 |  | – |  |
| >>>Repetition Period | M |  |  |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  |  |  | 9.2.3.6 |  | – |  |
| >>>TFCI Presence | M |  |  |  | 9.2.1.55 |  | – |  |
| >Diversity Indication | M |  |  |  | 9.2.2.7 |  | YES | ignore |
| >CHOICE diversity indication |  |  |  |  |  |  |  |  |
| >>Combining |  |  |  |  |  |  | YES | ignore |
| >>>RL ID | M |  |  |  | 9.2.1.49 | Reference RL | – |  |
| >>Non combining |  |  |  |  |  |  | YES | ignore |
| >>>DCH Information Response |  |  | 1..<maxnoofDCHs> |  |  | Only one DCH per set of co-ordinated DCHs shall be included. | – |  |
| >>>>DCH ID | M |  |  |  | 9.2.1.16 |  | – |  |
| >>>>Binding ID | M |  |  |  | 9.2.1.3 |  | – |  |
| >>>>Transport Layer Address | M |  |  |  | 9.2.1.62 |  | – |  |
| >Minimum Uplink SIR | M |  |  |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Maximum Uplink SIR | M |  |  |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Maximum Allowed UL Tx Power |  | M |  |  | 9.2.1.35 |  | – |  |
| >DSCH Information Response |  |  |  | 0 .. <Maxnoof DSCHs> |  |  | GLOBAL | ignore |
| >>DSCH ID |  | M |  |  |  |  | – |  |
| >>Transport Format Management |  | M |  |  |  |  | – |  |
| >>Priority Indicator |  |  |  | 1..16 |  | Provide Information for each priority class used | – |  |
| >>>Scheduling Priority Indicator |  | M |  |  |  | DSCH priority indicator | – |  |
| >>>MAC-c/sh SDU Length |  |  |  | 1..<MaxNbMAC-c/shSDULength> |  |  | – |  |
| >>>>MAC-c/sh SDU Length |  | M |  |  |  |  | – |  |
| >>CHOICE Diversity Indication |  |  |  |  |  |  | – |  |
| >>>Non combining |  |  |  |  |  |  | – |  |
| >>>>BindingID |  | M |  |  |  |  | – |  |
| >>>>Transport Layer Address |  | M |  |  |  |  | – |  |
| >USCH Information Response |  |  |  | 0 .. <Maxnoof USCHs> |  |  | GLOBAL | ignore |
| >>USCH ID |  | M |  |  |  |  | – |  |
| >>Transport Format Management |  | M |  |  |  |  | – |  |
| >>CHOICE Diversity Indication |  |  |  |  |  |  | – |  |
| >>>Non combining |  |  |  |  |  |  | – |  |
| >>>>BindingID |  | M |  |  |  |  | – |  |
| >>>>Transport Layer Address |  | M |  |  |  |  | – |  |
| >Neighbouring Cell Information |  |  |  | 0..<maxnoofneighbouringRNCs> |  |  | EACH | ignore |
| >>RNC-Id |  | M |  |  | 9.2.1.50 |  | – |  |
| >>CN PS Domain Identifier |  | O |  |  | 9.2.1.12 |  | – |  |
| >>CN CS Domain Identifier |  | O |  |  | 9.2.1.11 |  | – |  |
| >>Per FDD Cell Information |  |  |  | 0..<maxnoofFDDneighbours> |  |  | – |  |
| >>>C-Id |  | M |  |  | 9.2.1.6 |  | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nu in ref. [6] | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nd in ref. [6] | – |  |
| >>>Frame Offset |  | O |  |  | 9.2.1.30 |  | – |  |
| >>>Primary Scrambling Code |  | M |  |  | 9.2.1.45 |  | – |  |
| >>>Primary CPICH Power |  | O |  |  | 9.2.1.44 |  | – |  |
| >>>Cell Individual Offset |  | O |  |  | 9.2.1.7 |  | – |  |
| >>>Tx Diversity Indicator |  | M |  |  | 9.2.2.50 |  | – |  |
| >>>STTD Support Indicator |  | O |  |  | 9.2.2.45 |  | – |  |
| >>>Closed Loop Mode1 Support Indicator |  | O |  |  | 9.2.2.2 |  | – |  |
| >>>Closed Loop Mode2 Support Indicator |  | O |  |  | 9.2.2.3 |  | – |  |
| >>Per TDD Cell Information |  |  |  | 0..<maxnoofTDDneighbours> |  |  | – |  |
| >>>C-Id |  | M |  |  | 9.2.1.6 |  | – |  |
| >>>UARFCN |  | M |  |  | 9.2.1.66 | Corresponds to Nt in ref. [7] | – |  |
| >>>Frame Offset |  | O |  |  | 9.2.1.30 |  | – |  |
| >>>Cell Parameter ID |  | M |  |  | 9.2.1.8 |  | – |  |
| >>>Sync Case |  | M |  |  | 9.2.1.54 |  | – |  |
| >>>Time Slot |  | C-Case1 |  |  | 9.2.1.56 |  | – |  |
| >>>SCH Time Slot |  | C-Case2 |  |  | 9.2.1.51 |  | – |  |
| >>>Block STTD Indicator |  | M |  |  |  |  | – |  |
| >>>Cell Individual Offset |  | O |  |  | 9.2.1.7 |  | – |  |
| >>>DPCH Constant Value |  | O |  |  | 9.2.1.23 |  | – |  |
| >>>PCCPCH Power |  | O |  |  | 9.2.1.43 |  | – |  |
| Criticality Diagnostics |  | O |  |  | 9.2.1.13 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1 |
| Case2 | This IE is present only if Sync Case = Case2. |

| Range Bound | Explanation |  |
| --- | --- | --- |
| MaxnoofDCHs | Maximum number of dedicated channels on one RL |  |
| MaxnoofDSCHs | Maximum number of DSCHs for one UE. |  |
| MaxnoofUSCHs | Maximum number of USCHs for one UE. |  |
| MaxNbMAC-c/shSDULength | Maximum number of different MAC-c/sh SDU lengths |  |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |  |
| MaxnoofFDDNeighbours | Maximum number of neighbouring FDD cells for one cell |  |
| MaxnoofTDDNeighbours | Maximum number of neighbouring TDD cells for one cell |  |
| MaxnoofDLCodes | Maximum number of DL code information |  |
| MaxnoOfDPCHs |  | Maximum number of DPCH in one CCTrCH |
| MaxnoofCCTrCHs |  | number of CCTrCH for one UE. |
| MaxnoofULts |  | Maximum number of Uplink time slots per Radio Link |

### 9.1.12	RADIO LINK RECONFIGURATION READY

#### 9.1.12.1	FDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  | 9.2.1.59 |  | – |  |
| RL Information Response |  | 0..<maxnoofRLs> |  |  | EACH | ignore |
| >RL ID | M |  | 9.2.1.49 |  | – |  |
| >Maximum Uplink SIR | O |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Minimum Uplink SIR | O |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Secondary CCPCH Info |  | 0..1 |  |  | – |  |
| >>FDD S-CCPCH Offset | M |  | 9.2.2.15 | Corresponds to: S-CCPCH,k , see ref. [8] | – |  |
| >>DL Scrambling Code | M |  | 9.2.2.8 |  | – |  |
| >>FDD DL Channelisation Code Number | M |  | 9.2.2.14 |  | – |  |
| >>TFCS | M |  | 9.2.1.63 | For the DL. | – |  |
| >>Secondary CCPCH Slot Format | M |  | 9.2.2.38 |  | – |  |
| >>TFCI Presence | C - SlotFormat |  | 9.2.1.55 |  | – |  |
| >>Multiplexing Position | M |  | 9.2.2.26 |  | – |  |
| >>STTD Indicator | M |  | 9.2.2.44 |  | – |  |
| >>FACH/PCH Information |  | 1 .. <maxFACHcount+1> |  |  | – |  |
| >>>TFS |  |  | 9.2.1.64 | For each FACH, and the PCH when multiplexed on the same Secondary CCPCH | – |  |
| >>Scheduling Information |  | 1 |  |  | – |  |
| >>>IB_SG_REP | M |  | 9.2.2.21 |  | – |  |
| >>>Segment Information |  | 1.. <maxIBSEG> |  |  | – |  |
| >>>>IB_SG_POS | M |  | 9.2.2.20 |  | – |  |
| >Downlink Code Information |  | 0..<maxnoofDLCodes> |  |  | GLOBAL | ignore |
| >>DL Scrambling Code | M |  | 9.2.2.8 |  | – |  |
| >>FDD DL Channelisation Code Number | M |  | 9.2.2.14 |  | – |  |
| >>Transmission Gap Pattern Sequence Information Response | O |  |  |  | – |  |
| >DCH Information Response |  | 0..<maxnoofDCHs> |  | Only one DCH per set of co-ordinated DCHs shall be included. The IE group shall be included only once per DCH per set of combined RLs. | GLOBAL | ignore |
| >>DCH ID | M |  | 9.2.1.16 |  | – |  |
| >>Binding ID | M |  | 9.2.1.3 |  | – |  |
| >>Transport Layer Address | M |  | 9.2.1.62 |  | – |  |
| >DSCH to be Added or Modified |  | 0..1 |  |  | YES | ignore |
| >>DSCH Information |  | 1 .. <Maxnoof DSCHs> |  |  | – |  |
| >>>DSCH ID | M |  |  |  | – |  |
| >>>Priority Indicator |  | 1..16 |  | Provide Information for each priority class used | – |  |
| >>>>Scheduling Priority Indicator | M |  |  | DSCH priority indicator | – |  |
| >>>>MAC-c/sh SDU Length |  | 1..<MaxNbMAC-c/shSDULength> |  |  | – |  |
| >>>>>MAC-c/sh SDU Length | M |  |  |  | – |  |
| >>>Binding ID | M |  |  |  | – |  |
| >>>Transport Layer Address | M |  |  |  | – |  |
| >>PDSCH code mapping | M |  |  | PDSCH code mapping to be used | – |  |
| Criticality Diagnostics | O |  | 9.2.1.13 |  | YES | ignore |

| Condition |  | Explanation |
| --- | --- | --- |
| SlotFormat | This IE is present only if the Secondary CCPCH Slot Format is equal to any of the value 8 to 17 |  |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of DCHs. |
| MaxNbMAC-c/shSDULength | Maximum number of different MAC-c/sh SDU lengths |
| MaxnoofDSCHs | Maximum number of DSCHs for one UE. |
| MaxnoofRLs | Maximum number of RLs for a UE. |
| MaxnoofDLCodes | Maximum number of Downlink Channelisation Codes. |
| MaxFACHCount | Maximum number of FACH's mapped onto secondary CCPCH's |
| MaxIBSEG | Maximum number of segments for one Information Block |

#### 9.1.12.2	TDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  | 9.2.1.59 |  | – |  |
| RL Information Response |  | 0..1 |  |  | YES | ignore |
| >RL ID | M |  | 9.2.1.49 |  | – |  |
| >Maximum Uplink SIR | O |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Minimum Uplink SIR | O |  | Uplink SIR 9.2.1.69 |  | – |  |
| >UL CCTrCH Information |  | 0..<maxnoofCCTrCHs> |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  | 9.2.3.2 |  | – |  |
| >>UL DPCH to be added |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  | 9.2.3.3 |  | – |  |
| >>>TDD Channelisation Code | M |  | 9.2.3.8 |  | – |  |
| >>>Burst Type | M |  | 9.2.3.1 |  | – |  |
| >>>Midamble Shift | M |  | 9.2.3.4 |  | – |  |
| >>>Time Slot | M |  | 9.2.1.56 |  | – |  |
| >>>TDD Physical Channel Offset | M |  | 9.2.3.9 |  | – |  |
| >>>Repetition Period | M |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  | 9.2.3.6 |  | – |  |
| >>>TFCI Presence | M |  | 9.2.1.55 |  | – |  |
| >>UL DPCH to be modified |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  |  |  | – |  |
| >>>TDD Channelisation Code | O |  |  |  | – |  |
| >>>Burst Type | O |  |  |  | – |  |
| >>>Midamble Shift | O |  |  |  | – |  |
| >>>Time Slot | O |  |  |  | – |  |
| >>>TDD Physical Channel Offset | O |  |  |  | – |  |
| >>>Repetition Period | O |  |  |  | – |  |
| >>>Repetition Length | O |  |  |  | – |  |
| >>>TFCI Presence | O |  |  |  | – |  |
| >>UL DPCH to be deleted |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  |  |  | – |  |
| >DL CCTrCH Information |  | 0..<maxnoofCCTrCHs> |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  | 9.2.3.2 |  | – |  |
| >>DL DPCH to be added |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  | 9.2.3.3 |  | – |  |
| >>>TDD Channelisation Code | M |  | 9.2.3.8 |  | – |  |
| >>>Burst Type | M |  | 9.2.3.1 |  | – |  |
| >>>Midamble Shift | M |  | 9.2.3.4 |  | – |  |
| >>>Time Slot | M |  | 9.2.1.56 |  | – |  |
| >>>TDD Physical Channel Offset | M |  | 9.2.3.9 |  | – |  |
| >>> Repetition Period | M |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  | 9.2.3.6 |  | – |  |
| >>>TFCI Presence | M |  | 9.2.1.55 |  | – |  |
| >>DL DPCH to be modifed |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  |  |  | – |  |
| >>>TDD Channelisation Code | O |  |  |  | – |  |
| >>>Burst Type | O |  |  |  | – |  |
| >>>Midamble Shift | O |  |  |  | – |  |
| >>>Time Slot | O |  |  |  | – |  |
| >>>TDD Physical Channel Offset | O |  |  |  | – |  |
| >>> Repetition Period | O |  |  |  | – |  |
| >>>Repetition Length | O |  |  |  | – |  |
| >>>TFCI Presence | O |  |  |  | – |  |
| >>DL DPCH to be deleted |  | 0..<maxnoofDPCHs> |  |  | GLOBAL | ignore |
| >>>DPCH ID | M |  |  |  | – |  |
| >DCH Information Response |  | 0..<maxnoofDCHs> |  | Only one DCH per set of co-ordinated DCHs shall be included. The IE group shall be included only once per DCH per set of combined RLs. | GLOBAL | ignore |
| >>DCH ID | M |  | 9.2.1.16 |  | – |  |
| >>Binding ID | M |  | 9.2.1.3 |  | – |  |
| >>Transport Layer Address | M |  | 9.2.1.62 |  | – |  |
| >DSCH to be Added or Modified |  | 0 .. <Maxnoof DSCHs> |  |  | GLOBAL | ignore |
| >>DSCH ID | M |  |  |  | – |  |
| >>Transport Format Management | M |  |  |  | – |  |
| >>Priority Indicator |  | 1..16 |  | Provide Information for each priority class used | – |  |
| >>>Scheduling Priority Indicator | M |  |  | DSCH priority indicator | – |  |
| >>>MAC-c/sh SDU Length |  | 1..<MaxNbMAC-c/shSDULength> |  |  | – |  |
| >>>>MAC-c/sh SDU Length | M |  |  |  | – |  |
| >>Binding ID | M |  |  |  | – |  |
| >>Transport Layer Address | M |  |  |  | – |  |
| >USCH to be Added or Modified |  | 0 .. <Maxnoof USCHs> |  |  | GLOBAL | ignore |
| >>USCH ID | M |  |  |  | – |  |
| >>Transport Format Management | M |  |  |  | – |  |
| >>Binding ID | M |  |  |  | – |  |
| >>Transport Layer Address | M |  |  |  | – |  |
| Criticality Diagnostics | O |  | 9.2.1.13 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of DCHs for a UE. |
| MaxnoofDSCHs | Maximum number of DSCHs for one UE. |
| MaxnoofUSCHs | Maximum number of USCHs for one UE. |
| MaxNbMAC-c/shSDULength | Maximum number of different MAC-c/sh SDU lengths |
| MaxnoofCCTrCHs | Maximum number of CCTrCHs for a UE. |
| Maxnoof DPCHs | Maximum number of DPCHs in one CCTrCH. |

-- **************************************************************

--

-- RADIO LINK ADDITION RESPONSE TDD

--

-- **************************************************************

RadioLinkAdditionResponseTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkAdditionResponseTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkAdditionResponseTDD-Extensions}}                   OPTIONAL,

...

}

RadioLinkAdditionResponseTDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-RL-InformationResponse-RL-AdditionRspTDD

CRITICALITY ignore	TYPE RL-InformationResponse-RL-AdditionRspTDD 	PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponse-RL-AdditionRspTDD ::= SEQUENCE {

rL-ID								RL-ID,

sAI									SAI,

gA-Cell								GA-Cell		OPTIONAL,

gA-AccessPointPosition				GA-AccessPointPosition	OPTIONAL,

ul-InteferencePerTimeslot			UL-InterferenceList-RL-AdditionRspTDD,

ul-CCTrCHInformation				UL-CCTrCHInformationList-RL-AdditionRspTDD		OPTIONAL,

dl-CCTrCHInformation				DL-CCTrCHInformationList-RL-AdditionRspTDD		OPTIONAL,

diversityIndication					DiversityIndication-RL-AdditionRspTDD,

-- This IE represents both the Diversity Indication IE and the choice based on the diversity indication as described in

-- the tabular message format in subclause 9.1.

minUL-SIR							UL-SIR,

maxUL-SIR							UL-SIR,

maximumAllowedULTxPower				MaximumAllowedULTxPower,

dSCH-InformationResponse			DSCH-InformationResponse-RL-AdditionRspTDD		OPTIONAL,

uSCH-InformationResponse			USCH-InformationResponse-RL-AdditionRspTDD		OPTIONAL,

neighbouring-CellInformationList	Neighbouring-CellInformationList-RL-AdditionRsp 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {RL-InformationResponse-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

RL-InformationResponse-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-InterferenceList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfULTs)) OF UL-InterferenceItem-RL-AdditionRspTDD

UL-InterferenceItem-RL-AdditionRspTDD ::= SEQUENCE {

timeSlot					TimeSlot,

ul-InterferenceLevel		UL-InterferenceLevel,

iE-Extensions				ProtocolExtensionContainer { { UL-InterferenceItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-InterferenceItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-CCTrCHInformationList-RL-AdditionRspTDD ::= ProtocolIE-Container {{UL-CCTrCHInformationListIEs-RL-AdditionRspTDD}}

UL-CCTrCHInformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-CCTrCH-InformationListIE-RL-AdditionRspTDD   CRITICALITY ignore	TYPE UL-CCTrCHInformationListIE-RL-AdditionRspTDD		PRESENCE mandatory },

...

}

UL-CCTrCHInformationListIE-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF UL-CCTrCHInformationItem-RL-AdditionRspTDD

UL-CCTrCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

ul-DPCH-Information				UL-DPCH-InformationList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { {UL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-DPCH-InformationList-RL-AdditionRspTDD ::= DPCH-IE-ContainerList { {UL-DPCH-InformationListIEs-RL-AdditionRspTDD} }

UL-DPCH-InformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-DPCH-InformationItem-RL-AdditionRspTDD		CRITICALITY ignore	TYPE UL-DPCH-InformationItem-RL-AdditionRspTDD  PRESENCE mandatory	},

...

}

UL-DPCH-InformationItem-RL-AdditionRspTDD ::= SEQUENCE {

dPCH-ID							DPCH-ID,

tDD-ChannelisationCode			TDD-ChannelisationCode,

burstType						BurstType,

midambleShift					MidambleShift,

timeSlot						TimeSlot,

tDD-PhysicalChannelOffset		TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-CCTrCHInformationList-RL-AdditionRspTDD ::= ProtocolIE-Container {{DL-CCTrCHInformationListIEs-RL-AdditionRspTDD}}

DL-CCTrCHInformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-CCTrCH-InformationListIE-RL-AdditionRspTDD   CRITICALITY ignore	TYPE DL-CCTrCHInformationListIE-RL-AdditionRspTDD		PRESENCE mandatory },

...

}

DL-CCTrCHInformationListIE-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF DL-CCTrCHInformationItem-RL-AdditionRspTDD

DL-CCTrCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

dl-DPCH-Information				DL-DPCH-InformationList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { {DL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DPCH-InformationList-RL-AdditionRspTDD ::= DPCH-IE-ContainerList { {DL-DPCH-InformationListIEs-RL-AdditionRspTDD} }

DL-DPCH-InformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-DPCH-InformationItem-RL-AdditionRspTDD		CRITICALITY ignore	TYPE DL-DPCH-InformationItem-RL-AdditionRspTDD  PRESENCE mandatory	},

...

}

DL-DPCH-InformationItem-RL-AdditionRspTDD ::= SEQUENCE {

dPCH-ID							DPCH-ID,

tDD-ChannelisationCode			TDD-ChannelisationCode,

burstType						BurstType,

midambleShift					MidambleShift,

timeSlot						TimeSlot,

tDD-PhysicalChannelOffset		TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DiversityIndication-RL-AdditionRspTDD ::= ProtocolIE-Container {{DiversityIndicationIE-RL-AdditionRspTDD}}

DiversityIndicationIE-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DiversityIndicationItem-RL-AdditionRspTDD   CRITICALITY ignore	TYPE DiversityIndicationItem-RL-AdditionRspTDD  PRESENCE mandatory },

...

}

DiversityIndicationItem-RL-AdditionRspTDD		::= CHOICE {

combining		Combining-RL-AdditionRspTDD,

nonCombining	NonCombining-RL-AdditionRspTDD,

...

}

Combining-RL-AdditionRspTDD	::= 	ProtocolIE-Container {{CombiningIE-RL-AdditionRspTDD}}

CombiningIE-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-CombiningItem-RL-AdditionRspTDD   CRITICALITY ignore	TYPE CombiningItem-RL-AdditionRspTDD  PRESENCE mandatory },

...

}

CombiningItem-RL-AdditionRspTDD	::= SEQUENCE {

rL-ID						RL-ID,

iE-Extensions				ProtocolExtensionContainer { { CombiningItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

CombiningItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

NonCombining-RL-AdditionRspTDD	::= 	ProtocolIE-Container {{NonCombiningIE-RL-AdditionRspTDD}}

NonCombiningIE-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-NonCombiningItem-RL-AdditionRspTDD   CRITICALITY ignore	TYPE NonCombiningItem-RL-AdditionRspTDD  PRESENCE mandatory },

...

}

NonCombiningItem-RL-AdditionRspTDD ::= SEQUENCE {

dCH-InformationResponse-RL-AdditionRspTDD		DCH-InformationResponseList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { { NonCombiningItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

NonCombiningItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DCH-InformationResponseList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-AdditionRspTDD

DCH-InformationResponseItem-RL-AdditionRspTDD ::= SEQUENCE {

dCH-ID							DCH-ID,

bindingID						BindingID,

transportLayerAddress			TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

DCH-InformationResponseItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DSCH-InformationResponse-RL-AdditionRspTDD ::= ProtocolIE-Container {{DSCH-InformationListIEs-RL-AdditionRspTDD}}

DSCH-InformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DSCH-InformationListIE-RL-AdditionRspTDD	CRITICALITY ignore	TYPE DSCH-InformationListIE-RL-AdditionRspTDD		PRESENCE mandatory },

...

}

DSCH-InformationListIE-RL-AdditionRspTDD ::= SEQUENCE (SIZE(0..maxNoOfDSCHs)) OF DSCHInformationItem-RL-AdditionRspTDD

DSCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

dsch-ID					DSCH-ID,

transportFormatManagement	TransportFormatManagement,

priorityIndicator		PriorityIndicator-RL-AdditionRspTDD,

diversityIndication		DiversityIndication-RL-AdditionRspTDD2 OPTIONAL,

-- diversityIndication present, if CHOICE = nonCombining

iE-Extensions			ProtocolExtensionContainer { {DSCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

DSCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

PriorityIndicator-RL-AdditionRspTDD ::=  SEQUENCE (SIZE(1..16)) OF PriorityIndicatorItem-RL-AdditionRspTDD

PriorityIndicatorItem-RL-AdditionRspTDD ::= SEQUENCE {

schedulingPriorityIndicator		SchedulingPriorityIndicator,

mAC-c-sh-SDU-Lengths			MAC-c-sh-SDU-LengthList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { {PriorityIndicatorItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

PriorityIndicatorItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

MAC-c-sh-SDU-LengthList-RL-AdditionRspTDD ::= SEQUENCE(SIZE(1..maxNrOfMACcshSDU-Length)) OF MAC-c-sh-SDU-Length

DiversityIndication-RL-AdditionRspTDD2 ::= SEQUENCE {

bindingID				BindingID,

transportLayerAddress	TransportLayerAddress,

iE-Extensions			ProtocolExtensionContainer { {DiversityIndication-RL-AdditionRspTDD2-ExtIEs} } OPTIONAL,

...

}

DiversityIndication-RL-AdditionRspTDD2-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

USCH-InformationResponse-RL-AdditionRspTDD ::= ProtocolIE-Container {{USCH-InformationListIEs-RL-AdditionRspTDD}}

USCH-InformationListIEs-RL-AdditionRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-USCH-InformationListIE-RL-AdditionRspTDD	CRITICALITY ignore	TYPE USCH-InformationListIE-RL-AdditionRspTDD		PRESENCE mandatory },

...

}

USCH-InformationListIE-RL-AdditionRspTDD ::= SEQUENCE (SIZE(0..maxNoOfUSCHs)) OF USCHInformationItem-RL-AdditionRspTDD

USCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

uSCH-ID					USCH-ID,

transportFormatManagement	TransportFormatManagement,

diversityIndication		DiversityIndication-RL-AdditionRspTDD2 OPTIONAL,

-- diversityIndication present, if CHOICE = nonCombining

iE-Extensions			ProtocolExtensionContainer { {USCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

USCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

RadioLinkAdditionResponseTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- RADIO LINK RECONFIGURATION READY TDD

--

-- **************************************************************

RadioLinkReconfigurationReadyTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkReconfigurationReadyTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkReconfigurationReadyTDD-Extensions}}                   OPTIONAL,

...

}

RadioLinkReconfigurationReadyTDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-RL-InformationResponse-RL-ReconfReadyTDD

CRITICALITY ignore	TYPE RL-InformationResponse-RL-ReconfReadyTDD	PRESENCE optional	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponse-RL-ReconfReadyTDD ::= SEQUENCE {

rL-ID							RL-ID,

max-UL-SIR						UL-SIR			OPTIONAL,

min-UL-SIR						UL-SIR			OPTIONAL,

ul-CCTrCH-Information			UL-CCTrCH-InformationList-RL-ReconfReadyTDD 	OPTIONAL,

dl-CCTrCH-Information			DL-CCTrCH-InformationList-RL-ReconfReadyTDD	OPTIONAL,

dCHsInformationResponseList		DCH-InformationResponseList-RL-ReconfReadyTDD	OPTIONAL,

dSCHsToBeAddedOrModified		DSCHToBeAddedOrModified-RL-ReconfReadyTDD	OPTIONAL,

uSCHsToBeAddedOrModified		USCHToBeAddedOrModified-RL-ReconfReadyTDD	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponse-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

RL-InformationResponse-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-CCTrCH-InformationList-RL-ReconfReadyTDD 		::= ProtocolIE-Container {{UL-CCTrCHInformationListIEs-RL-ReconfReadyTDD}}

UL-CCTrCHInformationListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-CCTrCH-InformationListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE UL-CCTrCHInformationListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

UL-CCTrCHInformationListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfCCTrCHs)) OF UL-CCTrCH-InformationItem-RL-ReconfReadyTDD

UL-CCTrCH-InformationItem-RL-ReconfReadyTDD ::= SEQUENCE {

cCTrCH-ID						CCTrCH-ID,

ul-DPCH-AddInformation			UL-DPCH-InformationAddList-RL-ReconfReadyTDD			OPTIONAL,

ul-DPCH-ModifyInformation		UL-DPCH-InformationModifyList-RL-ReconfReadyTDD				OPTIONAL,

ul-DPCH-DeleteInformation		UL-DPCH-InformationDeleteList-RL-ReconfReadyTDD				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UL-CCTrCH-InformationItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

UL-CCTrCH-InformationItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-DPCH-InformationAddList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{UL-DPCH-InformationAddListIEs-RL-ReconfReadyTDD}}

UL-DPCH-InformationAddListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-DPCH-InformationAddListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE UL-DPCH-InformationAddListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

UL-DPCH-InformationAddListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF UL-DPCH-InformationAddItem-RL-ReconfReadyTDD

UL-DPCH-InformationAddItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationAddList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationAddList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-DPCH-InformationModifyList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{UL-DPCH-InformationModifyListIEs-RL-ReconfReadyTDD}}

UL-DPCH-InformationModifyListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE UL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

UL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF UL-DPCH-InformationModifyItem-RL-ReconfReadyTDD

UL-DPCH-InformationModifyItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode			OPTIONAL,

burstType					BurstType				OPTIONAL,

midambleShift					MidambleShift				OPTIONAL,

timeSlot					TimeSlot				OPTIONAL,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset		OPTIONAL,

repetitionPeriod				RepetitionPeriod			OPTIONAL,

repetitionLength				RepetitionLength			OPTIONAL,

tFCI-Presence					TFCI-Presence				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationModifyList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationModifyList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-DPCH-InformationDeleteList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{UL-DPCH-InformationDeleteListIEs-RL-ReconfReadyTDD}}

UL-DPCH-InformationDeleteListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE UL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

UL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF UL-DPCH-InformationDeleteItem-RL-ReconfReadyTDD

UL-DPCH-InformationDeleteItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationDeleteList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationDeleteList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-CCTrCH-InformationList-RL-ReconfReadyTDD 		::= ProtocolIE-Container {{DL-CCTrCHInformationListIEs-RL-ReconfReadyTDD}}

DL-CCTrCHInformationListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-CCTrCH-InformationListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE DL-CCTrCHInformationListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

DL-CCTrCHInformationListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfCCTrCHs)) OF DL-CCTrCH-InformationItem-RL-ReconfReadyTDD

DL-CCTrCH-InformationItem-RL-ReconfReadyTDD ::= SEQUENCE {

cCTrCH-ID						CCTrCH-ID,

dl-DPCH-AddInformation				DL-DPCH-InformationAddList-RL-ReconfReadyTDD				OPTIONAL,

dl-DPCH-ModifyInformation				DL-DPCH-InformationModifyList-RL-ReconfReadyTDD			OPTIONAL,

dl-DPCH-DeleteInformation				DL-DPCH-InformationDeleteList-RL-ReconfReadyTDD			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DL-CCTrCH-InformationItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DL-CCTrCH-InformationItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DPCH-InformationAddList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{DL-DPCH-InformationAddListIEs-RL-ReconfReadyTDD}}

DL-DPCH-InformationAddListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-DPCH-InformationAddListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE DL-DPCH-InformationAddListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

DL-DPCH-InformationAddListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF DL-DPCH-InformationAddItem-RL-ReconfReadyTDD

DL-DPCH-InformationAddItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationAddList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationAddList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DPCH-InformationModifyList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{DL-DPCH-InformationModifyListIEs-RL-ReconfReadyTDD}}

DL-DPCH-InformationModifyListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE DL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

DL-DPCH-InformationModifyListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF DL-DPCH-InformationModifyItem-RL-ReconfReadyTDD

DL-DPCH-InformationModifyItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode			OPTIONAL,

burstType					BurstType				OPTIONAL,

midambleShift					MidambleShift				OPTIONAL,

timeSlot					TimeSlot				OPTIONAL,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset		OPTIONAL,

repetitionPeriod				RepetitionPeriod			OPTIONAL,

repetitionLength				RepetitionLength			OPTIONAL,

tFCI-Presence					TFCI-Presence				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationModifyList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationModifyList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DPCH-InformationDeleteList-RL-ReconfReadyTDD ::= ProtocolIE-Container {{DL-DPCH-InformationDeleteListIEs-RL-ReconfReadyTDD}}

DL-DPCH-InformationDeleteListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD   CRITICALITY ignore	TYPE DL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD		PRESENCE mandatory },

...

}

DL-DPCH-InformationDeleteListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDPCHs)) OF DL-DPCH-InformationDeleteItem-RL-ReconfReadyTDD

DL-DPCH-InformationDeleteItem-RL-ReconfReadyTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationDeleteList-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationDeleteList-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DCH-InformationResponseList-RL-ReconfReadyTDD				::= ProtocolIE-Container { {DCH-InformationResponseListIEs-RL-ReconfReadyTDD} }

DCH-InformationResponseListIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DCH-InformationResponseListIE-RL-ReconfReadyTDD		CRITICALITY ignore	TYPE DCH-InformationResponseListIE-RL-ReconfReadyTDD		PRESENCE mandatory	},

...

}

DCH-InformationResponseListIE-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-ReconfReadyTDD

DCH-InformationResponseItem-RL-ReconfReadyTDD ::= SEQUENCE {

dCH-ID							DCH-ID,

bindingID						BindingID,

transportLayerAddress			TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DCH-InformationResponseItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DSCHToBeAddedOrModified-RL-ReconfReadyTDD			::= ProtocolIE-Container { {DSCHToBeAddedOrModifiedIEs-RL-ReconfReadyTDD} }

DSCHToBeAddedOrModifiedIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DSCHToBeAddedOrModifiedList-RL-ReconfReadyTDD	CRITICALITY ignore	TYPE DSCHToBeAddedOrModifiedList-RL-ReconfReadyTDD		PRESENCE mandatory	},

...

}

DSCHToBeAddedOrModifiedList-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNoOfDSCHs)) OF DSCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD

DSCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD ::= SEQUENCE {

dsch-ID					DSCH-ID,

transportFormatManagement	TransportFormatManagement,

priorityIndicator		PriorityIndicator-RL-ReconfReadyTDD,

bindingID				BindingID,

transportLayerAddress	TransportLayerAddress,

iE-Extensions			ProtocolExtensionContainer { {DSCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

DSCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

PriorityIndicator-RL-ReconfReadyTDD ::=  SEQUENCE (SIZE(1..16)) OF PriorityIndicatorItem-RL-ReconfReadyTDD

PriorityIndicatorItem-RL-ReconfReadyTDD ::= SEQUENCE {

schedulingPriorityIndicator		SchedulingPriorityIndicator,

mAC-c-sh-SDU-Lengths			MAC-c-sh-SDU-LengthList-RL-ReconfReadyTDD,

iE-Extensions					ProtocolExtensionContainer { {PriorityIndicatorItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

PriorityIndicatorItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

MAC-c-sh-SDU-LengthList-RL-ReconfReadyTDD ::= SEQUENCE(SIZE(1..maxNrOfMACcshSDU-Length)) OF MAC-c-sh-SDU-Length

USCHToBeAddedOrModified-RL-ReconfReadyTDD			::= ProtocolIE-Container { {USCHToBeAddedOrModifiedIEs-RL-ReconfReadyTDD} }

USCHToBeAddedOrModifiedIEs-RL-ReconfReadyTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-USCHToBeAddedOrModifiedList-RL-ReconfReadyTDD	CRITICALITY ignore	TYPE USCHToBeAddedOrModifiedList-RL-ReconfReadyTDD		PRESENCE mandatory	},

...

}

USCHToBeAddedOrModifiedList-RL-ReconfReadyTDD ::= SEQUENCE (SIZE (0..maxNoOfUSCHs)) OF USCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD

USCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD ::= SEQUENCE {

uSCH-ID					USCH-ID,

transportFormatManagement	TransportFormatManagement,

bindingID				BindingID,

transportLayerAddress	TransportLayerAddress,

iE-Extensions			ProtocolExtensionContainer { {USCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD-ExtIEs} } OPTIONAL,

...

}

USCHToBeAddedOrModifiedItem-RL-ReconfReadyTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

RadioLinkReconfigurationReadyTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}