TSG-RAN Working Group 3 Meeting #18                                                                     R3-010173

Lidingö, Sweden, 15th –19th January 2001

Agenda Item: 	rb-12.6

Source: 	LG Electronics, Inc.

Title: 	Text Proposal for TFCI Power Control Improvement

Document for:	Discussion and Decision

The text proposal on the signalling support for TFCI Power Control Improvement is proposed to be added in the chapter 6 of 25.849.

# Impacts to TSG RAN WG 3 specifications

For the Iub/Iur (NBAP/RNSAP) specifications [4] & [3] the power offset parameters for the secondary case with respect to the power value given per each frame in the FP in [5] and the averaging window parameters for the primary/secondary status determination from the SSDT commands sent in the uplink FBI (Feedback Information) field, would need to be added.

For the Iub/Iur the following IEs would be needed to indicate improved DSCH PC activation which is very similar with the way of SSDT activation. (e.g. Enhanced DSCH PC Indication IE & Enhanced DSCH PC Support Indication IE)

For the averaging window parameters at Node B the range [1..10] frames and [1..50] primary SSDT commands are proposed. (The latter range is larger as there can be from 1to 5 updates of SSDT status per frame depending on the SSDT ID length)

## Impacts on RNSAP (TS25.423)

### 9.1.11	RADIO LINK RECONFIGURATION PREPARE

#### 9.1.11.1	FDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  | 9.2.1.59 |  | – |  |
| Allowed Queuing Time | O |  | 9.2.1.2 |  | YES | reject |
| UL DPCH Information |  | 0..1 |  |  | YES | reject |
| >UL Scrambling Code | O |  | 9.2..2.53 |  | – |  |
| >UL SIR Target | O |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Min UL Channelisation Code Length | O |  | 9.2.2.25 |  | – |  |
| >Max Number of UL DPDCHs | C – CodeLen |  | 9.2.2.24 |  | – |  |
| >Puncture Limit | O |  | 9.2.1.46 | For the UL. | – |  |
| >TFCS | O |  | 9.2.1.63 | TFCS for the UL. | – |  |
| >UL DPCCH Slot Format | O |  | 9.2.2.52 |  | – |  |
| >Diversity mode | O |  | 9.2.2.8 |  | – |  |
| >SSDT Cell Identity Length | O |  | 9.2.2.41 |  | – |  |
| >Enhanced DSCH PC Wnd | O |  | 9.2.2.xx |  | – |  |
| >S-Field Length | O |  | 9.2.2.36 |  | – |  |
| DL DPCH Information |  | 0..1 |  |  | YES | reject |
| >TFCS | O |  | 9.2.1.63 | TFCS for the DL. | – |  |
| >DL DPCH Slot Format | O |  | 9.2.2.9 |  | – |  |
| >Number of DL Channelisation Codes | O |  | 9.2.2.26A |  | – |  |
| >TFCI Signalling Mode | O |  | 9.2.2.46 |  | – |  |
| >TFCI Presence | C- SlotFormat |  | 9.2.1.55 |  | – |  |
| >Multiplexing Position | O |  | 9.2.2.26 |  | – |  |
| > Enhanced primary TFCI Power Offset | O |  | 9.2.2.xx |  |  |  |
| > Enhanced non-primary TFCI Power Offset | O |  | 9.2.2.xx |  |  |  |
| >Limited Power Increase | O |  | 9.2.1.33 |  | – |  |
| DCHs to Modify | O |  | FDD DCHs to Modify 9.2.2.14C |  | YES | reject |
| DCHs to Add | O |  | DCH FDD Information 9.2.2.4A |  | YES | reject |
| DCHs to Delete |  | 0..<maxnoofDCHs> |  |  | GLOBAL | reject |
| >DCH ID | M |  | 9.2.1.16 |  | – |  |
| DSCHs to Modify |  | 0..1 |  |  | YES | reject |
| >DSCH Info |  | 0..<maxnoofDSCHs> |  |  | – |  |
| >>DSCH ID | M |  | 9.2.1.26A |  | – |  |
| >>TrCh Source Statistics Descriptor | O |  | 9.2.1.65 |  | – |  |
| >>Transport Format Set | O |  | 9.2.1.64 | For DSCH | – |  |
| >>Allocation/ Retention Priority | O |  | 9.2.1.1 |  | – |  |
| >>Scheduling Priority Indicator | O |  | 9.2.1.51A |  | – |  |
| >>BLER | O |  | 9.2.1.4 |  | – |  |
| >>Transport Bearer Request Indicator | M |  | 9.2.1.61 |  | – |  |
| >PDSCH RL ID | O |  | RL ID 9.2.1.49 |  | – |  |
| >TFCS | O |  | 9.2.1.63 | For DSCH | – |  |
| DSCHs to Add | O |  | DSCH FDD Information 9.2.2.13A |  | YES | reject |
| DSCHs to Delete |  | 0..1 |  |  | YES | reject |
| >DSCH Info |  | 1..<maxnoofDSCHs> |  |  | – |  |
| >>DSCH ID | M |  | 9.2.1.26A |  | – |  |
| RL Information |  | 0..<maxnoofRLs> |  |  | EACH | reject |
| >RL ID | M |  | 9.2.1.49 |  | – |  |
| >SSDT Indication | O |  | 9.2.2.41 |  | – |  |
| >Enhanced DSCH PC Indication | O |  | 9.2.2.xx |  | – |  |
| >Enhanced DSCH Power Offset | O |  | 9.2.2.xx |  | – |  |
| >SSDT Cell Identity | C - SSDTIndON |  | 9.2.2.40 |  | – |  |
| >Transmit Diversity Indicator | C – Diversity mode |  | 9.2.2.50 |  | – |  |
| Transmission Gap Pattern Sequence Information | O |  | 9.2.2.47A |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| SSDTIndON | The IE may be present if the SSDT Indication IE is set to 'SSDT Active in the UE' or Enhanced DSCH PC Indication IE is set to 'Enhanced DSCH PC Active in the UE'. |
| CodeLen | This IE is present only if the Min UL Channelisation Code length IE equals to 4. |
| SlotFormat | This IE is only present if the DL DPCH Slot Format IE is equal to any of the values 12 to 16. |
| Diversity mode | This IE is present if Diversity Mode IE is present in the UL DPCH Information IE and is not equal to “none”. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of DCHs for a UE. |
| MaxnoofDSCHs | Maximum number of DSCHs for one UE. |
| MaxnoofRLs | Maximum number of RLs for a UE. |

#### 9.2.2.xx	Enhanced Primary TFCI Power Offset

The Enhanced Primary TFCI Power Offset parameter gives the power offset to be applied on TFCI when cell is decided to be primary.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Primary TFCI Power Offset |  |  | INTEGER(0..15) | step 1dB |

#### 9.2.2.xx	Enhanced Non-primary TFCI Power Offset

The Enhanced Non-primary TFCI Power Offset parameter gives the power offset to be applied on TFCI when cell is decided to be non-primary.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Non-primary TFCI Power Offset |  |  | INTEGER(0..15) | step 1dB |

## Impacts on NBAP (TS25.433)

### RADIO LINK RECONFIGURATION PREPARE message

Power offset parameter for the secondary case and averaging window parameter for the primiary/secondary status determination will be needed. And also the information whether UE supports Enhanced DSCH PC shall be indicated.

### 9.1.42	RADIO LINK RECONFIGURATION PREPARE

#### 9.1.42.1	FDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantic Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Discriminator | M |  | 9.2.1.45 |  | – |  |
| Message Type | M |  | 9.2.1.46 |  | YES | reject |
| Node B Communication Context ID | M |  | 9.2.1.48 | The reserved value “All NBCC” shall not be used. | YES | reject |
| Transaction ID | M |  | 9.2.1.62 |  | – |  |
| UL DPCH Information |  | 0..1 |  |  | YES | reject |
| >UL Scrambling code | O |  | 9.2.2.59 |  | – |  |
| >UL SIR Target | O |  | UL SIR 9.2.2.58 |  | – |  |
| >Min UL Channelistion Code Length | O |  | 9.2.2.22 |  | – |  |
| >Max Number of UL DPDCHs | C – CodeLen |  | 9.2.2.20 |  | – |  |
| >Puncture Limit | O |  | 9.2.1.50 | For UL | – |  |
| >TFCS | O |  | 9.2.1.58 |  | – |  |
| >UL DPCCH Slot Format | O |  | 9.2.2.57 |  | – |  |
| >Diversity mode | O |  | 9.2.2.9 |  | – |  |
| >SSDT Cell Identity Length | O |  | 9.2.2.45 |  | – |  |
| >Enhanced DSCH PC Wnd | O |  | 9.2.2.xx |  | – |  |
| >S-Field Length | O |  | 9.2.2.40 |  | – |  |
| DL DPCH Information |  | 0..1 |  |  | YES | reject |
| >TFCS | O |  | 9.2.1.58 |  | – |  |
| >DL DPCH Slot Format | O |  | 9.2.2.10 |  | – |  |
| >TFCI Signalling Mode | O |  | 9.2.2.50 |  | – |  |
| >TFCI presence | C-Slot Format |  | 9.2.1.57 |  | – |  |
| >Multiplexing Position | O |  | 9.2.2.23 |  | – |  |
| >Enhanced Primary TFCI Power Offset | O |  | 9.2.2.xx |  | – |  |
| >Enhanced Non-primary TFCI Power Offset | O |  | 9.2.2.xx |  | – |  |
| >PDSCH code mapping | O |  | 9.2.2.25 |  | – |  |
| >PDSCH RL ID | O |  | RL ID 9.2.1.53 |  | – |  |
| >Limited Power Increase | O |  | 9.2.2.18A |  | – |  |
| DCHs to Modify | O |  | DCHs FDD to Modify 9.2.2.4E |  | YES | reject |
| DCHs to Add | O |  | DCH FDD Information 9.2.2.4D |  | YES | reject |
| DCHs to Delete |  | 0..<maxnoofDCHs> |  |  | GLOBAL | reject |
| >DCH ID | M |  | 9.2.1.20 |  | – |  |
| DSCH to modify |  | 0..<maxnoofDSCHs> |  |  | YES | reject |
| >DSCH ID | M |  | 9.2.1.27 |  | – |  |
| >Transport Format Set | O |  | 9.2.1.59 | For the DL. | – |  |
| >Allocation/Retention Priority | O |  | 9.2.1.1A |  | – |  |
| >Frame Handling Priority | O |  | 9.2.1.30 |  | – |  |
| >ToAWS | O |  | 9.2.1.61 |  | – |  |
| >ToAWE | O |  | 9.2.1.60 |  | – |  |
| >Transport Bearer Request Indicator | M |  | 9.2.1.62A |  | – |  |
| DSCH to add | O |  | DSCH FDD Information 9.2.2.13B |  | YES | reject |
| DSCH to Delete |  | 0..<maxnoofDSCHs> |  |  | YES | reject |
| >DSCH ID | M |  | 9.2.1.27 |  | – |  |
| TFCI2 bearer specific information |  | 0..1 |  |  | YES | reject |
| >CHOICE TFCI2 bearer action | M |  |  |  | – |  |
| >>Add or modify |  |  |  |  | – |  |
| >>>ToAWS | M |  | 9.2.1.61 |  | – |  |
| >>>ToAWE | M |  | 9.2.1.60 |  | – |  |
| >>Delete |  |  | NULL |  | – |  |
| RL Information |  | 0..<maxnoofRLs> |  |  | EACH | reject |
| >RL ID | M |  | 9.2.1.53 |  | – |  |
| >DL Code Information | O |  | FDD DL Code Information 9.2.2.14A |  | – |  |
| >Maximum DL Power | O |  | DL Power 9.2.1.21 |  | – |  |
| >Minimum DL Power | O |  | DL Power 9.2.1.21 |  | – |  |
| >SSDT Indication | O |  | 9.2.2.47 |  | – |  |
| >Enhanced DSCH PC Indication | O |  | 9.2.2.xx |  | – |  |
| >Enhanced DSCH Power Offset | O |  | 9.2.2.xx |  | – |  |
| >SSDT Cell Identity | C– SSDTIndON |  | 9.2.2.44 |  | – |  |
| >Transmit Diversity Indicator | C – Diversity mode |  | 9.2.2.53 |  | – |  |
| Transmission Gap Pattern Sequence Information | O |  | 9.2.2.53A |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| SSDTIndON | The IE may be present if the SSDT Indication is set to 'SSDT Active in the UE' or Enhanced DSCH PC Indication IE is set to 'Enhanced DSCH PC Active in the UE'.. |
| CodeLen | This IE is present only if "Min UL Channelisation Code length" equals to 4. |
| SlotFormat | This IE is only present if the DL DPCH slot format is equal to any of the value 12 to 16. |
| SF/2 | This IE is present only if the Transmission Gap Pattern Sequence Information IE is included and the indicated Downlink Compressed Mode method for at least one of the included Transmission Gap Pattern Sequence is set to "SF/2'". |
| Diversity mode | This IE is present unless Diversity Mode IE in UL DPCH Information group, unless it is equal to “none” |

| Range Bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of DCHs for a UE. |
| MaxnoofDSCHs | Maximum number of DSCHs for a UE. |
| MaxnoofRLs | Maximum number of RLs for a UE. |

#### 9.2.2.xx	Enhanced Primary TFCI Power Offset

The Enhanced Primary TFCI Power Offset parameter gives the power offset to be applied on TFCI when cell is decided to be primary.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Primary TFCI Power Offset |  |  | INTEGER(0..15) | step 1dB |

#### 9.2.2.xx	Enhanced Non-primary Power Offset

The Enhanced Non-primary Power Offset parameter gives the power offset to be applied on TFCI when cell is decided to be non-primary.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Non-primary Power Offset |  |  | INTEGER(0..15) | step 1dB |