3GPP TSG-RAN3 Meeting #69 	R3-102477

# Madrid, Spain, 23rd  - 27th  August 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.467 | CR | 0106 | rev | - | Current version: | 9.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of HNB GW transparent HNB to HNB mobility procedures using HNBAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HNB-HENB_mob_enh |  |  |  |  |  |  | Date: |  | 15/08/2010 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Introduction of HNB GW transparent HNB to HNB mobility procedure into Stage 2. HNB to HNB mobility is realised by enhancing the HNBAP protocol using the HNB GW as HNBAP signalling proxy. Stage 2 descriptions of the new procedures are introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add stage 2 descriptions to describe intra HNB GW, inter HNB mobility procedure via HNBAP in a way that is transparent to the HNB GW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient HNB to HNB mobility in enterprise and mall environment. Inter HNB mobility not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.7.2, 5.7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 25.469 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Start of change

### 5.2.2	HNB Registration procedure

Figure 5.2.2-1. HNB Registration procedure.

1.	HNB initialization is performed to obtain HNB configuration from the HNB Management System (HMS). Similarly, HNB-GW discovery is performed to obtain the initial serving HNB-GW information.

2.	The HNB establishes a secure tunnel to the SeGW of the serving HNB-GW.

NOTE: 	This step may be omitted if the secure tunnel happens to be the same tunnel that is already established to contact the HMS.

3.	The HNB sets up an SCTP transport session to a well-defined port on the serving HNB-GW.

4.	The HNB then attempts to register with the serving HNB-GW using an HNB REGISTER REQUEST message. The message contains:

a.	HNB Location Information: The HNB provides location information via use of one or more of the following mechanisms:

i.	detected macro-cell coverage information (e.g. GERAN or UTRAN cell information)

ii.	geographical co-ordinates (e.g. via use of GPS, etc)

iii.	Internet connectivity information (e.g. IP address), provided, the resulting location information is at least as accurate as location determination based on macro-cell coverage information, whether or not there is macro cell-coverage available at the location of the HNB (e.g. as determined by point i above).

b.	HNB Identity: the HNB has a globally unique and permanent identity.

c.	HNB Operating Parameters: Such as the selected LAC, RAC, SAC, PLMN Id, Cell Id, etc.

d.	HNB operating mode (optional): HNB CSG-id or access mode (open, closed or hybrid).

If the HNB supports the HNBAP Relocation procedures and if the HNB has been configured to enable the use of such procedures the HNB REGISTER REQUEST shall include

- Enhanced Mobility Indicator: This IE indicates to the HNB GW that the NB is enabled to support HNBAP relocation procedures

- Neighbour Cell List: This is the list of all the HNB neighbour cells, sharing the same HNB GW, configured at the HNB.  The HNB GW needs this information to provide the HNB with an indication of which o fits neighbours support HNBAP relocation.

5a.	The HNB-GW may use the information from the HNB REGISTER REQUEST message to check whether the HNB registration can be accepted (e.g. to check whether a particular HNB is allowed to operate in a given location, etc). If the HNB-GW accepts the registration attempt it shall respond with a HNB REGISTER ACCEPT message. If the HNB-GW has capability to de-multiplex, the HNB-GW may include a mux port in the HNB REGISTER ACCEPT message.
If the HNB GW received the Enhanced Mobility Indicator IE and the Neighbour Cell List IE in the HNB REGISTER REQUEST message, the HNB REGISTER ACCEPT shall include the Neighbour Information List IE including CSG-ID, PLMN-ID, Cell-ID and Enhanced Mobility Indicator for each of the HNB neighbour cells.  The HNB will use this information to understand whether to start HNBAP relocation procedures towards any of its neighbours.

5b.	Alternatively, the HNB-GW may reject the registration request (e.g. due to network congestion, blacklisted HNB, unauthorized HNB location, etc). In this case, the HNB-GW shall respond with an HNB REGISTER REJECT indicating the reject cause.

NOTE:	The HNB shall start broadcasting only after successful registration with the HNB-GW.

# Next change

## 5.7	HNB to HNB Mobility

### 5.7.1	General

The following sub-sections describe the mechanism for handling the intra HNB-GW intra CSG mobility signaling.

### Connected mode mobility from one HNB to another HNB (Intra HNB-GW, Intra CSG)

### Figure 5.7.2-1 shows the message flows for intra HNB GW HNBAP relocation for Cell_DCH UEs.

S-HNB

UE

T-HNB

1.HNBAP Relocation Request

3. HNBAP Relocation Response

Ongoing CS/PS session

CN

Decision to relocate UE to T-HNB within same HNB-GW

8. RUA Disconnect

5. HNBAP Relocation Commit

CN

HNB-GW

2.HNBAP Relocation Request

7.HNBAP Relocation Complete

6. HNBAP Relocation Commit

4. HNBAP Relocation Response

Access Control/Membership Verification Procedure

RRC Conn. Reconf. Req (e.g. Physical Channel Reconfig req.)

RRC Conn. Reconfig. Comp. (e.g. Physical Channel Reconfiguration Complete)

Figure 5.7.2-1:Intra-HNB GW relocation via the HNB-GW in Cell_DCH

- Based on the information received during HNB Registration, the HNB is able to determine whether relocation towards a neighbour HNB cell shall be carried out via HNBAP or via RANAP signalling.  If the HNB chooses to trigger a HNBAP relocation it will send a HNBAP Relocation Request to the HNB GW. This message shall include source and target Cell Global Identities, UE Context for the relocated UE and all the information needed at the target to relocate the UE bearers.
The HNB GW shall ensure that HNBAP relocation is allowed towards the target HNB and if this is allowed it shall forward the message to the target HNB. The Relocation Information Request IE contained in the HNBAP Relocation Request shall be transparent to the HNB GW.

- The HNB GW forwards the HNBAP Relocation Request to the target HNB.  The HNB GW shall include in the message the uplink transport layer parameters to be used by the target HNB. The Relocation Request message will serve the purpose of implicit UE registration at the target HNB.

- If the relocation to the target is possible the target HNB shall send a HNBAP Relocation Response to the HNB GW.  This message shall include all the information needed at the source to successfully terminate the relocation preparation procedures.  This message shall include source and target Cell Global Identities, UE Context for the relocated UE and the downlink transport layer parameters to be used by the HNB GW for communication with the target HNB. The Relocation Information Response IE containing information about the relocated bearers shall be transparent to the HNB GW.

- The HNB GW forwards the HNBAP Relocation Response to the target HNB.  The HNB GW may start forwarding UE bearers data to the target HNB via the downlink TNL address and association received by the target in HNBAP Relocation Response.

- After completion of the HNBAP Relocation preparation procedures RRC reconfiguration procedures are started in order to move the UE connection to the target HNB.  When all necessary Iu-UP parameters indicating the latest Iu-UP state are available at the source HNB (e.g. Last UL Frame number, up to date RFCIs, Data Volume Reports), the HNBAP Relocation Commit message shall be sent from source HNB to HNB GW. This message shall include all the parameters needed at the target HNB to allow seamless relocation of the Iu-UP without signalling to the CN.  
After this message is sent, the source HNB shall consider the UE relocated to the target HNB unless an error message is received by the HNB. The message shall include details about the relocated RABs as part of the Relocation Information IE.  This IE shall be transparent to the HNB GW.

- The HNB GW forwards the HNBAP Relocation Commit to the target HNB. Once RRC Connection Reconfiguration procedures are completed between UE and target HNB and after reception of this message the target HNB can start deliverying uplink traffic to the HNB GW.  
If seamless mobility has to be guaranteed, the HNBAP Relocation Commit procedure needs to terminate before termination of the RRC Connection Reconfiguration procedures

- The target HNB sends a HNBAP Relocation Complete to the HNB GW.  This message informs the HNB GW of completion of the relocation and it provides information about the relocated bearers.

- Upon successful termination of the relocation procedures the HNB GW sends a RUA Disconnect message to the source HNB

### 5.7.x	UE Cell FACH mobility from one HNB to another HNB (Intra HNB-GW, Intra CSG)

Figure 5.7.3-1 shows the message flows for intra HNB GW HNBAP relocation for Cell_FACH UEs.

S-HNB

UE

T-HNB

Ongoing CS/PS session

CN

CN

HNB-GW

HNBAP Relocation Procedures

RRC Cell Update

1. HNBAP Context Retrieve Request

2. HNBAP Context Retrieve Request

Figure 5.7.3-1:Intra-HNB GW relocation via the HNB-GW in Cell_FACH

- Upon receiving the RRC Cell Update message and provided that Cell FACH mobility is allowed the target HNB shall initiate the HNBAP Context Retrieve Request. This message contains the target Cell Identity, the UE U-RNTI received from the UE with the Cell Update.

- Upon detection of the HNBAP Context Retrieve Request message from the target HNB, HNB-GW determines the source HNB where that UE context is stored. This is done via the U-RNTI parameter. Once resolved, HNB-GW sends the HNBAP Context Retrieve Request message to the source HNB. Amongst other parameters this message contains the UE Context ID in the source HNB and the target Cell Identity. Upon detection of this message, source HNB starts the relocation procedure as described in the Cell_DCH mobility section 5.7.2.
It shall be pointed out that for Cell FACH relocation signalling the HNBAP Relocation Request shall include the U-RNTI of the UE registered at the source HNB.  The HNB GW may modify the U-RNTI in the HNBAP Relocation Request before forwarding this message to the target HNB.

# End of change