3GPP TSG-RAN WG3 Meeting #77bis 	R3-122365

# Lecce, Italy, 8th-12th October 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | xxxx | rev | - | Current version: | 11.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new MDT measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Siemens Networks, Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDT_UMTSLTE-Core |  |  |  |  |  |  | Date: |  | 2012-10-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | New MDT measurements (M4, M5, M6 and M7) have been introduced by RAN2, and are detailed in the incoming LS R3-121502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Semantics of Measurements to Activate IE are modified to support signalling of measurements M4, M5, M6 and M7. For each of these measurements, a configuration IE is introduced. Impact Analysis This CR has isolated impact towards the previous version of the specification (same release). This CR has an impact under functional and protocol point of view. The impact can be considered isolated since it only affects the MDT functionality. Note: The Immediate MDT sequence was defined without a protocol extension container, and this is added in ASN.1 in order to allow extension of the sequence according to RAN3 specification guidelines. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for new MDT functionality in rel-11 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.97, 9.2.1.98, 9.2.1.99, 9.2.1.100, 9.2.1.aa, 9.2.1.bb, 9.2.1.cc, 9.2.1.dd, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS36.413 CRxx, TS36.423 CRxx |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.1.97	MDT Configuration

The purpose of the MDT Configuration IE is to provide configuration information for the MDT function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  |
| CHOICE MDT Area Scope | M |  |  |  |
| >Cell Based |  |  |  |  |
| >>Cell ID List |  | 1 to <maxnoofCellID> |  |  |
| >>>Cell-ID | M | INTEGER (0..268435455) |  | This information element identifies a cell uniquely within UTRAN and consists of RNC-ID and C-ID as defined in TS 25.401 [3]. |
| >LA Based |  |  |  |  |
| >>LAI List |  | 1 to <maxnoofLAIs> |  |  |
| >>>LAI | M |  | 9.2.3.6 |  |
| >RA Based |  |  |  |  |
| >>RAI List |  | 0 to <maxnoofRAIs> |  |  |
| >>>RAI |  |  |  | This element identifies an area in which the MDT Configuration applies. |
| >>>>LAI | M |  | 9.2.3.6 |  |
| >>>>RAC | M |  | 9.2.3.7 |  |
| >PLMN Area Based |  |  | NULL |  |
| CHOICE MDT mode |  |  |  |  |
| >immediate MDT |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [31]. First Bit = M1, Second Bit = M2, Third Bit = M3, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = M6 Seventh Bit = M7. Other bits are reserved for future use and are ignored if received. Value ‘1’ indicates “activate” and value ‘0’ indicates “do not activate”. |
| >>M1 Report | C-ifM1 |  | 9.2.1.98 | M1: FDD only: CPICH RSCP and CPICH Ec/No measurement by UE |
| >>M2 Report | C-ifM2 |  | 9.2.1.99 | M2: TDD only: P-CCPCH RSCP and Timeslot ISCP for UTRA 1.28 TDD |
| >>M4 Report | C-ifM4 |  | 9.2.1.aa | M4: UPH measurement by UE |
| >>M5 Report | C-ifM5 |  | 9.2.1.bb | M5: RTWP measurement by NodeB |
| >>M6 Report | C-ifM6 |  | 9.2.1.cc | M6:Data volume measurement by RNC |
| >>M7 Report | C-ifM7 |  | 9.2.1.dd | M7: Throughput measurement by RNC |
| >logged MDT |  |  |  |  |
| >>logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12,10.24,20.48, 30.72, 40.96 and 61.44, ...) | The unit of this IE is second. This IE is defined in TS 25.331[10]. |
| >>logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120, ...) | The unit of this IE is minute. This IE is defined in TS 25.331[10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellID | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofLAIs | Maximum no. of LAI subject for MDT scope. Value is 8. |
| maxnoofRAIs | Maximum no of RAI subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| ifM2 | This IE shall be present if the Measurements to Activate IE has the second bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to “1”. |

#### 9.2.1.98	M1 Report

This IE defines the parameters for M1 report, FDD report of UE radio measurements as specified in TS 32.422 [38].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE M1 Report trigger |  |  |  |  |
| >Periodic |  |  |  |  |
| >>MDT Report Parameters | M |  | 9.2.1.100 |  |
| >event1F |  |  |  |  |
| >>Measurement quantity | M |  | ENUMERATED( CPICH Ec/N0, CPICH RSCP, pathloss, ...) |  |
| >>threshold | M |  | INTEGER(-120…165) | Range used depends on measurement quantity. CPICH RSCP -120..-25 dBm CPICH Ec/No -24..0 dB Pathloss 30..165dB |

#### 9.2.1.99	M2 Report

This IE defines the parameters for a M2 report, TDD report of UE radio measurements as specified in TS 32.422 [38].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE M2 Report trigger |  |  |  |  |
| >Periodic |  |  |  |  |
| >>MDT Report Parameters | M |  | 9.2.1.100 |  |
| >Event1I |  |  |  |  |
| >>threshold | M |  | INTEGER(-120..-25) |  |

#### 9.2.1.100	MDT Report parameters

This IE defines the report parameters for MDT periodic reports of measurements M1 and M2 as specified in TS 32.422 [38].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Report interval | M |  | ENUMERATED ( ms250, ms500, ms1000, ms2000, ms3000, ms4000, ms6000, ms12000, ms16000, ms20000, ms24000, ms32000, ms64000, ...) | This IE is defined in TS 25.331[10]. |
| Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity, ...) |  |

#### 9.2.1.aa	M4 Report

This IE defines the parameters for UE Power Headroom (M4) measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Collection mode |  |  |  |  |
| >All |  |  |  | All available measurements are logged |
| >Periodic and event triggered periodic |  |  |  | One sample collected per period |
| >>M4 Collection period | M |  | ENUMERATED(ms100, ms250, ms500, ms1000, ms2000, ms3000, ms4000, ms6000, …) |  |
| >>M4 Threshold | O |  | INTEGER(0..31) | If included, logging shall be initiated when UPH value falls below the indicated threshold, and then for each period, until UPH value goes above the threshold. The mapping Is defined in TS25.123 (TDD) and TS25.133 (FDD). |

#### 9.2.1.bb	M5 Report

This IE defines the parameters for RTWP (M5) measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Collection mode |  |  |  |  |
| >When available |  |  |  | All available measurements are logged |
| >Periodic |  |  |  | One sample collected per period |
| >>M5 Collection period | M |  | ENUMERATED(ms100, ms250, ms500, ms1000, ms2000, ms3000, ms4000, ms6000, …) |  |

#### 9.2.1.cc	M6 Report

This IE defines the parameters for data volume (M6) measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Collection Period | M |  | INTEGER (1..64) | Unit:: [second] |

#### 9.2.1.dd	M7 Report

This IE defines the parameters for data volume (M7) measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..64) | Unit:: [second] |

#### >>>>>>>>>>>>>>>>>   NEXT CHANGE  <<<<<<<<<<<<<<<<<<<<<<<<

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

# RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

# DEFINITIONS AUTOMATIC TAGS ::=

# BEGIN

# IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

maxRAB-Subflows,

maxRAB-SubflowCombination,

maxNrOfLevels,

maxNrOfAltValues,

maxNrOfSNAs,

maxNrOfLAs,

maxNrOfPLMNsSN,

maxSet,

maxNrOfHSDSCHMACdFlows-1,

maxNrOfUEsToBeTraced,

maxNrOfInterfaces,

maxnoofMulticastServicesPerRNC,

maxMBMSSA,

maxMBMSRA,

maxnoofMulticastServicesPerUE,

maxNrOfEDCHMACdFlows-1,

maxGANSSSet,

maxNrOfEUTRAFreqs,

maxNrOfCellIds,

maxNrOfRAIs,

maxNrOfLAIs,

maxNrOfVol,

id-CN-DomainIndicator,

id-MessageStructure,

id-SRB-TrCH-Mapping,

id-TypeOfError,

id-hS-DSCH-MAC-d-Flow-ID,

id-SignallingIndication,

id-CellLoadInformationGroup,

id-TraceRecordingSessionInformation,

id-MBMSLinkingInformation,

id-AlternativeRABConfiguration,

id-AlternativeRABConfigurationRequest,

id-E-DCH-MAC-d-Flow-ID,

id-RAC,

id-Alt-RAB-Parameter-ExtendedGuaranteedBitrateInf,

id-Alt-RAB-Parameter-ExtendedMaxBitrateInf,

id-Ass-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-Ass-RAB-Parameter-ExtendedMaxBitrateList,

id-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-RAB-Parameter-ExtendedMaxBitrateList,

id-Requested-RAB-Parameter-ExtendedMaxBitrateList,

id-Requested-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-LAofIdleModeUEs,

id-newLAListofIdleModeUEs,

id-LAListwithNoIdleModeUEsAnyMore,

id-ExtendedRNC-ID,

id-GANSS-PositioningDataSet,

id-d-RNTI-for-NoIuCSUP,

id-UE-History-Information,

id-SubscriberProfileIDforRFP,

id-Alt-RAB-Parameter-SupportedGuaranteedBitrateInf,

id-Alt-RAB-Parameter-SupportedMaxBitrateInf,

id-Ass-RAB-Parameter-SupportedGuaranteedBitrateList,

id-Ass-RAB-Parameter-SupportedMaxBitrateList,

id-RAB-Parameter-SupportedGuaranteedBitrateList,

id-RAB-Parameter-SupportedMaxBitrateList,

id-Requested-RAB-Parameter-SupportedMaxBitrateList,

id-Requested-RAB-Parameter-SupportedGuaranteedBitrateList,

id-PSRABtobeReplaced,

id-SRVCC-Information,

id-CSG-Id,

id-CSFB-Information,

id-IRAT-Measurement-Configuration,

id-Management-Based-MDT-Allowed,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M4Report,

id-M5Report,

id-M6Report,

id-M7Report

# FROM RANAP-Constants

# >>>>>>>>>>>>>>>skip unchanged text in same section  <<<<<<<<<<<<<<<

-- I

IMEI					::= OCTET STRING (SIZE (8))

-- Reference: 23.003

# IMEIGroup	::= SEQUENCE {

iMEI				IMEI,

iMEIMask			BIT STRING (SIZE (7)),

iE-Extensions		ProtocolExtensionContainer { { IMEIGroup-ExtIEs} } OPTIONAL

}

# IMEIGroup-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

IMEIList	::= SEQUENCE (SIZE (1..maxNrOfUEsToBeTraced)) OF IMEI

IMEISV					::= OCTET STRING (SIZE (8))

-- Reference: 23.003

# IMEISVGroup	::= SEQUENCE {

iMEISV				IMEISV,

iMEISVMask			BIT STRING (SIZE (7)),

iE-Extensions		ProtocolExtensionContainer { { IMEISVGroup-ExtIEs} } OPTIONAL

}

# IMEISVGroup-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

IMEISVList	::= SEQUENCE (SIZE (1..maxNrOfUEsToBeTraced)) OF IMEISV

ImmediateMDT	::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1report					M1Report				OPTIONAL,

-- Included in case of event-triggered reporting for measurement M1

m2report					M2Report				OPTIONAL,

-- Included in case of event-triggered reporting for measurement M2

iE-Extensions		ProtocolExtensionContainer { { ImmediateMDT-ExtIEs} } OPTIONAL,

...

}

# ImmediateMDT-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

ID id-M4Report	CRITICALITY ignore	EXTENSION M4Report	PRESENCE optional,

-- Included in case that measurement M4 is activated

ID id-M5Report	CRITICALITY ignore	EXTENSION M5Report	PRESENCE optional,

-- Included in case that measurement M5 is activated

ID id-M6Report	CRITICALITY ignore	EXTENSION M6Report	PRESENCE optional,

-- Included in case that measurement M6 is activated

ID id-M7Report	CRITICALITY ignore	EXTENSION M7Report	PRESENCE optional,

-- Included in case that measurement M7 is activated

...

}

# >>>>>>>>>>>>>>>skip unchanged text in same section  <<<<<<<<<<<<<<<

-- M

M1Report	::= CHOICE {

periodic			MDT-Report-Parameters,

event1F				Event1F-Parameters,

...

}

M2Report	::= CHOICE {

periodic			MDT-Report-Parameters,

event1I				Event1I-Parameters,

...

}

M4Report	::= CHOICE {

all							NULL

m4-collection-parameters	M4-Collection-Parameters,

...

}

M4-Collection-Parameters ::= SEQUENCE {

m4-period		M4-Period,

m4-threshold	M4-Threshold	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { M4-Collection-Parameters-ExtIEs} } OPTIONAL,

...

}

M4-Collection-Parameters-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M4-Period		::=	ENUMERATED {ms100, ms250, ms500, ms1000, ms2000, ms3000, ms4000, ms6000, ...)

M4-Threshold ::= INTEGER (0..31)

M5Report	::= CHOICE {

when-available		NULL

m5-period			M5-Period,

...

}

M5-Period		::=	ENUMERATED {ms100, ms250, ms500, ms1000, ms2000, ms3000, ms4000, ms6000, ...)

M6Report ::= SEQUENCE {

m6-period		M6-Period,

iE-Extensions	ProtocolExtensionContainer { { M6Report-ExtIEs} } OPTIONAL,

...

}

M6Report-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M6-Period ::= INTEGER (1..16)

M7Report ::= SEQUENCE {

m7-period		M7-Period,

iE-Extensions	ProtocolExtensionContainer { { M7Report-ExtIEs} } OPTIONAL,

...

}

M7Report-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M7-Period ::= INTEGER (1..16)

Management-Based-MDT-Allowed ::= ENUMERATED {

allowed, ... }

#### >>>>>>>>>>>>>>>>>   NEXT CHANGE  <<<<<<<<<<<<<<<<<<<<<<<<

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

# RANAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-Constants (4) }

# DEFINITIONS AUTOMATIC TAGS ::=

# BEGIN

# >>>>>>>>>>>>>>>skip unchanged text in same section  <<<<<<<<<<<<<<<

# id-Relocation-SourceRNC-ID								INTEGER ::= 222

# id-Relocation-SourceExtendedRNC-ID						INTEGER ::= 223

id-EncryptionKey										INTEGER ::= 224

id-IntegrityProtectionKey								INTEGER ::= 225

id-SRVCC-HO-Indication									INTEGER ::= 226

id-SRVCC-Information									INTEGER ::= 227

id-SRVCC-Operation-Possible								INTEGER ::= 228

# id-CSG-Id-List											INTEGER ::= 229

id-PSRABtobeReplaced									INTEGER ::= 230

# id-E-UTRAN-Service-Handover								INTEGER ::= 231

# id-UE-AggregateMaximumBitRate							INTEGER ::= 233

# id-CSG-Membership-Status 								INTEGER ::= 234

# id-Cell-Access-Mode										INTEGER ::= 235

# id-IP-Source-Address									INTEGER ::= 236

# id-CSFB-Information										INTEGER ::= 237

id-PDP-TypeInformation-extension						INTEGER ::= 238

# id-MSISDN												INTEGER ::= 239

# id-Offload-RAB-Parameters								INTEGER ::= 240

# id-LGW-TransportLayerAddress							INTEGER ::= 241

# id-Correlation-ID										INTEGER ::= 242

# id-IRAT-Measurement-Configuration						INTEGER ::= 243

# id-MDT-Configuration									INTEGER ::= 244

# id-Priority-Class-Indicator								INTEGER ::= 245

# id-RNSAPRelocationParameters							INTEGER ::= 247

# id-RABParametersList									INTEGER ::= 248

# id-Management-Based-MDT-Allowed							INTEGER ::= 249

# id-HigherBitratesThan16MbpsFlag							INTEGER ::= 250

# id-Trace-Collection-Entity-IP-Addess					INTEGER ::= 251

# id-End-Of-CSFB											INTEGER ::= 252

# id-Time-UE-StayedInCell-EnhancedGranularity				INTEGER ::= 253

# id-Out-Of-UTRAN											INTEGER ::= 254

id-TraceRecordingSessionReference				INTEGER ::= 255

id-IMSI											INTEGER ::= 256

# id-HO-Cause												INTEGER ::= 257

# id-VoiceSupportMatchIndicator							INTEGER ::= 258

# id-M4Report												INTEGER ::= xxx

# id-M5Report												INTEGER ::= xxx

# id-M6Report												INTEGER ::= xxx

# id-M7Report												INTEGER ::= xxx

# END