3GPP TSG-RAN WG3#86	R3-142948

San Francisco, USA, 17-21 November, 2014

Agenda Item:	29.1

Source: 	Ericsson

Title:  	Tabular format issues for E-DCH-Non-Scheduled-Grant-Info in some initiating messages (TDD)

Document for:	Discussion and Approval

# 1	Introduction

During the rapporteur check of the Rel-12 version of RNSAP it was noted that there was a mis-alignment between the ASN.1 and the tabular format for the RADIO LINK RECONFIGURATION PREPARE message and the RADIO LINK RECONFIGURATION REQUEST message since two IE’s are missing in the tabular format.

# 2	Discussion

## 2.1	Background

The issue exist from version 7.2.0 when Enhanced Uplink was introduced for 3.84 Mcps and 7.68Mcps TDD with CR1219r1 in R3-061352.

The E-DCH Information IE and the E-DCH Information 7.68Mcps IE in the tabular format are represented by the “E-DCH-Information-Reconfig” and “E-DCH-768-Information-Reconfig” SEQUENCES in the ASN.1, and the ASN.1 sequence contains one additional IE (E-DCH-Non-Scheduled-Grant-Info) that is not shown in the Tabular format.

### 2.1.1	Release 12 Tabular format

### 9.1.11	RADIO LINK RECONFIGURATION PREPARE

#### 9.1.11.1	FDD Message

Unaffected parts omitted

#### 9.1.11.2	TDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Unaffected parts omitted |  |  |  |  |  |  |
| RL Information |  | 0..<maxNrOfRLs. |  |  | YES | ignore |
| >RL ID | M |  | 9.2.1.49 |  | – |  |
| >RL Specific DCH Information | O |  | 9.2.1.49A |  | – |  |
| Primary CCPCH RSCP Delta | O |  | 9.2.3.5a |  | YES | ignore |
| E-DCH Information |  | 0..1 |  | 3.84Mcps TDD only. | YES | reject |
| >E-PUCH Information | O |  | 9.2.3.36 |  | – |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |
| >E-DCH TDD Information | O |  | 9.2.3.40 |  | – |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |
| E-DCH Serving RL | O |  | RL ID 9.2.1.49 | 3.84Mcps TDD only. | YES | reject |
| E-DCH Information 7.68Mcps |  | 0..1 |  | 7.68Mcps TDD only. | YES | reject |
| >E-PUCH Information | O |  | 9.2.3.36 |  | – |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |
| >E-DCH TDD Information 7.68Mcps | O |  | 9.2.3.51 |  | – |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |
| E-DCH Information 1.28Mcps |  | 0..1 |  | 1.28Mcps TDD only. | YES | reject |
| >E-PUCH Information LCR | O |  | 9.2.3.36a |  | – |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |
| >E-DCH TDD Information LCR | O |  | 9.2.3.40a |  | – |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |
| Need for Idle Interval | O |  | ENUMERATED (True, False) | TDD only. | YES | ignore |
| Unaffected parts omitted |  |  |  |  |  |  |

Unaffected parts omitted

### 9.1.16	RADIO LINK RECONFIGURATION REQUEST

#### 9.1.16.1	FDD Message

Unaffected parts omitted

#### 9.1.16.2	TDD Message

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unaffected parts omitted |  |  |  |  |  |  |
| HS-PDSCH RL ID | O |  | RL ID 9.2.1.49 |  | YES | reject |  |
| E-DCH Information |  | 0..1 |  | 3.84Mcps TDD only. | YES | reject |  |
| >E-PUCH Information | O |  | 9.2.3.36 |  | – |  |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |  |
| >E-DCH TDD Information | O |  | 9.2.3.40 |  | – |  |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |  |
| E-DCH Serving RL | O |  | RL ID 9.2.1.49 | 3.84Mcps TDD only. | YES | reject |  |
| E-DCH Information 7.68Mcps |  | 0..1 |  | 7.68Mcps TDD only. | YES | reject |  |
| >E-PUCH Information | O |  | 9.2.3.36 |  | – |  |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |  |
| >E-DCH TDD Information 7.68Mcps | O |  | 9.2.3.51 |  | – |  |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |  |
| E-DCH Information 1.28Mcps |  | 0..1 |  | 1.28Mcps TDD only. | YES | reject |  |
| >E-PUCH Information LCR | O |  | 9.2.3.36a |  | – |  |  |
| >E-TFCS Information TDD | O |  | 9.2.3.37 |  | – |  |  |
| >E-DCH MAC-d Flows to Add | O |  | 9.2.3.38 |  | – |  |  |
| >E-DCH MAC-d Flows to Delete | O |  | 9.2.1.90 |  | – |  |  |
| >E-DCH TDD Information LCR | O |  | 9.2.3.40a |  | – |  |  |
| >E-DCH TDD Information to Modify | O |  | 9.2.3.42 |  | – |  |  |
| Need for Idle Interval | O |  | ENUMERATED (True, False) | TDD only. | YES | ignore |  |
| Unaffected parts omitted |  |  |  |  |  |  |

### 2.1.1	Release 12 ASN.1

-- **************************************************************

--

#### -- RADIO LINK RECONFIGURATION PREPARE TDD

--

-- **************************************************************

RadioLinkReconfigurationPrepareTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkReconfigurationPrepareTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkReconfigurationPrepareTDD-Extensions}}                   OPTIONAL,

...

}

Unrelated parts omitted

RadioLinkReconfigurationPrepareTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

{ ID id-PrimaryCCPCH-RSCP-RL-ReconfPrepTDD	CRITICALITY ignore	EXTENSION PrimaryCCPCH-RSCP				PRESENCE optional}|

{ ID id-DL-TimeSlot-ISCP-Info-RL-ReconfPrepTDD	CRITICALITY ignore	EXTENSION DL-TimeSlot-ISCP-Info		PRESENCE optional}|

{ ID id-DL-Timeslot-ISCP-LCR-Information-RL-ReconfPrepTDD	CRITICALITY ignore	EXTENSION DL-TimeSlot-ISCP-LCR-Information	PRESENCE optional}|

{ ID id-HSDSCH-TDD-Information				CRITICALITY reject	EXTENSION HSDSCH-TDD-Information		PRESENCE optional}|

{ ID id-HSDSCH-Information-to-Modify		CRITICALITY reject	EXTENSION HSDSCH-Information-to-Modify		PRESENCE optional}|

{ ID id-HSDSCH-MACdFlows-to-Add				CRITICALITY reject	EXTENSION HSDSCH-MACdFlows-Information		PRESENCE optional}|

{ ID id-HSDSCH-MACdFlows-to-Delete			CRITICALITY reject	EXTENSION HSDSCH-MACdFlows-to-Delete		PRESENCE optional}|

{ ID id-HSPDSCH-RL-ID						CRITICALITY reject	EXTENSION RL-ID								PRESENCE optional}|

{ ID id-PDSCH-RL-ID							CRITICALITY ignore	EXTENSION RL-ID							PRESENCE optional}|

{ ID id-UL-Synchronisation-Parameters-LCR	CRITICALITY ignore	EXTENSION UL-Synchronisation-Parameters-LCR	PRESENCE optional}|

-- Mandatory for 1.28Mcps TDD, Not Applicable to 3.84Mcps TDD or 7.68Mcps TDD

{ ID id-RL-Information-RL-ReconfPrepTDD		CRITICALITY ignore	EXTENSION RL-Information-RL-ReconfPrepTDD	PRESENCE optional}|

{ ID id-PrimaryCCPCH-RSCP-Delta				CRITICALITY ignore	EXTENSION PrimaryCCPCH-RSCP-Delta			PRESENCE optional}|

{ ID id-E-DCH-Information-Reconfig			CRITICALITY reject	EXTENSION E-DCH-Information-Reconfig		PRESENCE optional}|

{ ID id-E-DCH-Serving-RL-ID					CRITICALITY reject	EXTENSION RL-ID				PRESENCE optional}|

{ ID id-E-DCH-768-Information-Reconfig		CRITICALITY reject	EXTENSION E-DCH-768-Information-Reconfig	PRESENCE optional}|

{ ID id-E-DCH-LCR-Information-Reconfig		CRITICALITY reject	EXTENSION E-DCH-LCR-Information-Reconfig	PRESENCE optional}|

{ ID id-NeedforIdleInterval					CRITICALITY ignore	EXTENSION NeedforIdleInterval				PRESENCE optional}|

{ ID id-CPC-InformationLCR					CRITICALITY reject	EXTENSION CPC-InformationLCR				PRESENCE optional}|

{ ID id-RNTI-Allocation-Indicator			CRITICALITY ignore	EXTENSION  RNTI-Allocation-Indicator		PRESENCE optional}|

{ ID id-DCH-MeasurementType-Indicator		CRITICALITY reject	EXTENSION DCH-MeasurementType-Indicator		PRESENCE optional}|

{ ID id-Multi-Carrier-EDCH-Reconfigure		CRITICALITY reject	EXTENSION Multi-Carrier-EDCH-Reconfigure	PRESENCE optional}|

{ ID id-MU-MIMO-Indicator					CRITICALITY reject	EXTENSION MU-MIMO-Indicator					PRESENCE optional}|

{ ID id-UE-Support-of-non-rectangular-resource-allocation	CRITICALITY ignore	EXTENSION UE-Support-of-non-rectangular-resource-allocation		PRESENCE optional},

...

}

Unrelated parts omitted

-- **************************************************************

--

#### -- RADIO LINK RECONFIGURATION REQUEST TDD

--

-- **************************************************************

RadioLinkReconfigurationRequestTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkReconfigurationRequestTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkReconfigurationRequestTDD-Extensions}}                   OPTIONAL,

...

}

Unrelated parts omitted

RadioLinkReconfigurationRequestTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

{ ID id-RL-ReconfigurationRequestTDD-RL-Information	CRITICALITY ignore	EXTENSION Multiple-RL-ReconfigurationRequestTDD-RL-Information	PRESENCE optional}|

{ ID id-HSDSCH-TDD-Information				CRITICALITY reject	EXTENSION HSDSCH-TDD-Information		PRESENCE optional}|

{ ID id-HSDSCH-Information-to-Modify-Unsynchronised	CRITICALITY reject	EXTENSION HSDSCH-Information-to-Modify-Unsynchronised	PRESENCE optional}|

{ ID id-HSDSCH-MACdFlows-to-Add				CRITICALITY reject	EXTENSION HSDSCH-MACdFlows-Information		PRESENCE optional}|

{ ID id-HSDSCH-MACdFlows-to-Delete			CRITICALITY reject	EXTENSION HSDSCH-MACdFlows-to-Delete		PRESENCE optional}|

{ ID id-HSPDSCH-RL-ID							CRITICALITY reject	EXTENSION RL-ID							PRESENCE optional}|

{ ID id-E-DCH-Information-Reconfig			CRITICALITY reject	EXTENSION E-DCH-Information-Reconfig		PRESENCE optional}|

{ ID id-E-DCH-Serving-RL-ID					CRITICALITY reject	EXTENSION RL-ID				PRESENCE optional}|

{ ID id-E-DCH-768-Information-Reconfig		CRITICALITY reject	EXTENSION E-DCH-768-Information-Reconfig	PRESENCE optional}|

{ ID id-E-DCH-LCR-Information-Reconfig		CRITICALITY reject	EXTENSION E-DCH-LCR-Information-Reconfig	PRESENCE optional}|

{ ID id-CPC-InformationLCR					CRITICALITY reject	EXTENSION CPC-InformationLCR				PRESENCE optional}|

{ ID id-RNTI-Allocation-Indicator			CRITICALITY ignore	EXTENSION RNTI-Allocation-Indicator		PRESENCE optional}|

{ ID id-DCH-MeasurementType-Indicator		CRITICALITY reject	EXTENSION DCH-MeasurementType-Indicator		PRESENCE optional}|

{ ID id-Multi-Carrier-EDCH-Reconfigure		CRITICALITY reject	EXTENSION Multi-Carrier-EDCH-Reconfigure	PRESENCE optional}|

{ ID id-MU-MIMO-Indicator					CRITICALITY reject	EXTENSION MU-MIMO-Indicator					PRESENCE optional}|

{ ID id-UE-Support-of-non-rectangular-resource-allocation	CRITICALITY ignore	EXTENSION UE-Support-of-non-rectangular-resource-allocation		PRESENCE optional},

...

}

Unrelated parts omitted

E-DCH-Information-Reconfig ::= SEQUENCE {

e-PUCH-Information 						E-PUCH-Information									OPTIONAL,

e-TFCS-Information-TDD 					E-TFCS-Information-TDD							OPTIONAL,

e-DCH-MACdFlows-to-Add					E-DCH-MACdFlows-Information-TDD					OPTIONAL,

e-DCH-MACdFlows-to-Delete					EDCH-MACdFlows-To-Delete							OPTIONAL,

e-DCH-Non-Scheduled-Grant-Info 			E-DCH-Non-Scheduled-Grant-Info					OPTIONAL,

e-DCH-TDD-Information						E-DCH-TDD-Information								OPTIONAL,

e-DCH-TDD-Information-to-Modify			E-DCH-TDD-Information-to-Modify					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { E-DCH-Information-Reconfig-ExtIEs} }	OPTIONAL,

...

}

Unrelated parts omitted

E-DCH-768-Information-Reconfig ::= SEQUENCE {

e-PUCH-Information 						E-PUCH-Information									OPTIONAL,

e-TFCS-Information-TDD 					E-TFCS-Information-TDD							OPTIONAL,

e-DCH-MACdFlows-to-Add					E-DCH-MACdFlows-Information-TDD					OPTIONAL,

e-DCH-MACdFlows-to-Delete					EDCH-MACdFlows-To-Delete							OPTIONAL,

e-DCH-Non-Scheduled-Grant-Info768		E-DCH-Non-Scheduled-Grant-Info768				OPTIONAL,

e-DCH-TDD-Information768					E-DCH-TDD-Information768							OPTIONAL,

e-DCH-TDD-Information-to-Modify			E-DCH-TDD-Information-to-Modify					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { E-DCH-768-Information-Reconfig-ExtIEs} }	OPTIONAL,

...

}

Unrelated parts omitted

## 2.2	Potential problems caused by the issues

The current procedure text clearly explains when to include the E-DCH Non-scheduled Grant Information TDD IE, E-DCH Non-scheduled Grant Information LCR TDD IE and E-DCH Non-scheduled Grant Information7.68Mcps TDD IE in the RADIO LINK ADDITION RESPONSE, RADIO LINK ADDITION RESPONSE, RADIO LINK RECONFIGURATION READY and RADIO LINK RECONFIGURATION RESPONSE message.

The current procedure text however does not mention anything about the E-DCH Non-scheduled Grant Information xxx IE’s for any initiating messages, and the intention was probably not to enable inclusion of the E-DCH Non-scheduled Grant Information TDD IE and E-DCH Non-scheduled Grant Information7.68Mcps TDD IE in the request messages (RADIO LINK RECONFIGURATION PREPARE or RADIO LINK RECONFIGURATION REQUEST).

The tabular format should be aligned with the ASN.1, but this alignment would add IE’s that was not meant be used to some request messages. In order to be backwards compatible the correction therefore needs to clarify that the IE’s shall not be used.

According to the current RAN3 rules, the ASN.1 takes precedence of the tabular format, and the current specifications thus allows that the two IE’s are included in the request messages, and this may cause interoperability issues especially since the inherited criticality for the IE is reject.

## 2.3	Proposed solutions

It is proposed to not make any changes to the ASN.1, and to treat the issues as an alignment of the tabular with the current ASN.1.

Since the issues are very old, and assuming that it can be agreed to not change the ASN.1 it is also proposed to only correct Release 12. But this should probably be handled with a separate CR to make the correction traceable.

It is proposed to add the E-DCH Non-scheduled Grant Information TDD IE and the E-DCH Non-scheduled Grant Information7.68Mcps TDD IE to the tabular format of the RADIO LINK RECONFIGURATION PREPARE and RADIO LINK RECONFIGURATION REQUEST messages, and at the same time in the Semantics Description state that the IE shall be ignored if received.

# 3	Proposal

It is proposed to clarify the Rel-12 specification by:

- Adding the E-DCH Non-scheduled Grant Information TDD IE and the E-DCH Non-scheduled Grant Information7.68Mcps TDD IE in the tabular format of the RADIO LINK RECONFIGURATION PREPARE and RADIO LINK RECONFIGURATION REQUEST messages

- In the Semantics Description of the added IE’s state that they shall be ignored if received

A proposed Release 12 CR has been submitted in R3-142949