3GPP TSG RAN3 Meeting #87	R3-150435

Athens, Greece, 9 – 13 Feb, 2015

Agenda item:       11.2

Source:	NEC

Title:	eRNTP Signalling for Inter-eNB CoMP

Document for:		Discussion and Decision

1. Introduction

It has been agreed that eRNTP will be delivered with the Load Information message.

In the following we provide our views on   content of this message, as well as a text proposal.

2. Discussion

2.1	eRNTP exchange

One concern that has been raised during the Rel.12 discussions is that a receiving eNB does not have the means to differentiate the “meaning” of  the CoMP hypothesis. In particular, a potential issue is that the signaled hypothesis could be a “suggestion” by the sender or an “action” which implies that the pattern in the hypothesis will be applied. A proposed solution [1] to this issue is to introduce a new IE. Our view is that the need for a new IE can be avoided by using the eRNTP exchange in order to convey the “action”. The content of this exchange is captured in a corresponding text proposal attached in the end of this contribution.

The presented text proposal exploits that fact that the RNTP (i.e., downlink (DL) power restriction) for the first subframe (subframe #0) is always conveyed. Then, if no information about the DL power restriction on any subsequent subframe is conveyed, the one conveyed for the first subframe can be assumed to remain static (i.e., applicable over subsequent subframes).

3. Conclusion

In this contribution we discussed the necessary X2 message to support eRNTP exchange for  inter-eNB CoMP and presented a corresponding text proposal.

References

[1] R3-143018, “Hypothesis Exchange and Coordination”, Qualcomm, Samsung, AT&T

Text Proposal:

### 9.2.19	Relative Narrowband Tx Power (RNTP)

This IE provides an indication on DL power restriction per PRB in a cell and other information needed by a neighbour eNB for interference aware scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RNTP Per PRB | M |  | BIT STRING (6..110, …) | Each position in the bitmap represents a nPRB value (i.e. first bit=PRB 0 and so on), for which the bit value represents RNTP (nPRB), defined in TS 36.213 [11]. Value 0 indicates "Tx not exceeding RNTP threshold". Value 1 indicates "no promise on the Tx power is given". This IE is used to indicate DL power restriction per PRB for the first subframe. In case the DL power restriction is static, the indicated DL power restriction is maintained over the subsequent subframes. | – | – |
| RNTP Threshold | M |  | ENUMERATED (-∞, -11, -10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, …) | RNTPthreshold is defined in TS 36.213 [11]. | – | – |
| Number Of Cell-specific Antenna Ports | M |  | ENUMERATED (1, 2, 4, …) | P (number of antenna ports for cell-specific reference signals) defined in TS 36.211 [10] | – | – |
| P_B | M |  | INTEGER (0..3, …) | PB is defined in TS 36.213 [11]. | – | – |
| PDCCH Interference Impact | M |  | INTEGER (0..4, …) | Measured by Predicted Number Of Occupied PDCCH OFDM Symbols (see TS 36.211 [10]). Value 0 means "no prediction is available". | – | – |
| Extended RNTP Per PRB | O |  | BIT STRING (6..4290, …) | Each position in the bitmap represents a PRB in a subframe, for which value “1” indicates ‘no promise on the Tx power is given’ and value “0” indicates ‘Tx not exceeding RNTP threshold.’ The first bit corresponds to PRB 0 of the first subframe for which the extended RNTP per PRB IE is valid, the second bit corresponds to PRB 1 of the first subframe for which the extended RNTP per PRB IE is valid, and so on. The length of the bit string is an integer (maximum 39) multiple of . is defined in TS 36.211 [10]. The bit string may span across multiple contiguous subframes. The pattern across contiguous subframes (formed by RNTP per PRB and extended RNTP per PRB) is continuously repeated |  |  |
| RNTP per PRB start time |  | 0..1 |  |  |  |  |
| >Starting SFN | M |  | INTEGER (0..1023, …) | SFN of the radio frame containing the first subframe when the RNTP Per PRB IE is valid. |  |  |
| >Starting Subframe Index | M |  | INTEGER (0..9, …) | Subframe number, within the radio frame indicated by the Start SFN IE, of the first subframe when the RNTP Per PRB IE is valid. |  |  |