3GPP TSG-RAN WG3#89Bis	R3-152218

Sofia Antipolis, France, October 05th – 09th 2015

Agenda Item:	15.2.3

Source: 	CISCO SYSTEMS

Title:  	Xw-WT Identifier Format

Document for:	Discussion and Approval

# 1	Introduction

During the RAN3#89 meeting Xw interface aspects have been extensively discussed and significant progress has been made. Afterwards, during the email discussion [RAN3#89-01], stage-3 aspects for the Xw signalling discussed. This contribution discusses the format for WLAN Termination point Identifier that is an open point from the email discussion.

# 2	Discussion

2.1	WLAN Termination Identifiers on the Xw interface

According to the current LTE/WLAN aggregation architecture, a WLAN mobility set defined is a set of one or more WLAN APs identified by one or more BSSID/HESSID/SSIDs, within which WLAN mobility mechanisms apply while the UE is configured with bearers using WLAN for LWA, i.e., the UE may perform mobility between WLAN APs belonging to the mobility set without informing the eNB. A WLAN Logical Node is defined as WLAN Termination (WT) point and could be formed of group of WLAN Access points.

Figure 1 E-UTRAN Architecture for LWA

In the agreed architecture, all APs belonging to mobility set share a common WT as termination point for Xw-C and Xw-U. The WLAN identifiers belonging to a mobility set may be a subset of all WLAN identifiers associated to the WT. In the current technically endorsed Stage-3 BL CR for XwAP, below mentioned approach has been described for the WT Identifier on the Xw interface.

During the XwAP association setup, eNB sends the XwAP Xw SETUP REQUEST message to the WT. WT in response send the XwAP Xw SETUP RESPONSE including the WT Identification in the following format.

| IE/Group Name | Range |
| --- | --- |
| Message Type |  |
| WT ID |  |
| WLAN Identifier List | 1 |
| >WLAN Identifier Item | 1..<maxnoofBSSsperWT> |
| >>WLAN Information |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBSSsperWT | 4096 |

WTID is defined as BIT STRING (48) and further details are currently FFS.

There are four alternate options defined for the WLAN information IE as below.

Option 0:

IE/Group Name

BSSID

>>WLAN Band Information List

>>>WLAN Band Information Item

>>>>WLAN Band Information

SSID

HESSID

Option 1:

BSSID List

>BSSID List Item

>>BSSID

>>WLAN Band Information List

>>>WLAN Band Information Item

>>>>WLAN Band Information

SSID

HESSID

Option 2:

IE/Group Name

BSSID

SSID List

>SSID List Item

>>SSID

HESSID List

>HESSID List Item

>>HESSID

WLAN Band Information List

>WLAN Band Information Item

>>WLAN Band Information

Option 3:

IE/Group Name

BSSID List

>BSSID List Item

>>BSSID

>>WLAN Band Information List

>>>WLAN Band Information Item

>>>WLAN Band Information

SSID List

>SSID List Item

>>SSID

HESSID List

>HESSID List Item

>>HESSID

By definition from 802.11 specifications [3], BSSID is a unique identifier (MAC address) for an access point in a BSS network while SSID is the construct that allows logical separation of different WLAN.  It is possible for an AP to be configured with multiple SSIDs, such numbers are limited to few in the actual deployments. Hence, while we agree that all the above options for encoding these identities could work we think that option 1 would require the minimal signalling overhead (in terms of number of bytes) to the Xw interface considering the typical WLAN deployment scenario.

Proposal 1: RAN3 shall agree on Option 1 as the encoding choice for the WLAN Identifier.

The semantic description of WT ID is currently FFS. Also, whether to include the PLMN Id as part of the WT ID is currently marked as FFS. One important point to note that WT ID is defined on top of the actual WLAN IDs (HESSSID/BSSID/SSID) used in other WLAN protocols in order to make the Xw interface simple. Actually there is no mapping between the WTID and WLAN IDs (HESSSID/BSSID/SSID) outside the Xw interface unlike the Global eNB ID that is used on several interfaces.

Hence in our view the actual encoding mechanism of WT ID bitstring (48) shall be completely left to the deployment choice and without any restriction.

In addition, we think that WT ID shall be added in TS 23.003.

Proposal 2: Actual format of WT ID shall be left for the deployment choice without any restriction.

# 3	Proposals

Proposal 1: RAN3 shall agree on Option 1 as the encoding choice for the WLAN Identifier.

Proposal 2: Actual format of WT ID shall be left for the deployment choice without any restriction.

# 4	References

[1]	RP-151114	LTE-WLAN Radio Level Integration and Interworking Enhancement

[2]	R3-151828	Baseline CR for parameter exchange for LTE-WLAN interworking

[3]	IEEE 802.11 Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specification

[4]	TS 23.003	Numbering, addressing and identification