3GPP TSG-RAN WG3 #93	R3-162006

Gothenburg, Sweden, 22nd – 26th August 2016

Source: 	Ericsson

Title:	TP on clarifications on fronthaul bitrate requirements

Agenda Item:	10.5.5

Document for:	Discussion and Decision

# Introduction

This contribution contains a TP for TR38.801. The TP derives from discussions on document R3-161901 on which the following was agreed:

CB: # 57_ClarificationTableTNL

-  add maximum

- discuss the possibility of 2nd value in the table

(E///)

# Text Proposal

----------------------------------------------Start of Changes----------------------------------------------

#### 6.1.2.3	Transport network aspects

##### 6.1.2.3.1	General

This section summarizes transport network requirements of different functional splits.

NOTE 1:	It is understood that RAN3 has no intention to specify any transport network.

##### 6.1.2.3.2	Transport network requirements for an example RAN architecture

According to TR 38.913 [5], the NR shall support up to 1GHz system bandwidth, and up to 256 antennas. A calculation relative to one of several possible transport deployments applied to a possible RAN architecture example shows that transmission between base band part and radio frequency part requires a maximum bitrate over the transport network of about 614.4Mbps per 10MHz mobile system bandwidth per antenna port.

When the system bandwidth is increasing as well as the number of antenna ports, the required bitrate is linearly increasing. An example of maximum bitrates with rounded numbers is shown in the table below (values on the right).

Another example of bit rate calculation for the same transport deployment and RAN architecture is added in the table below. The calculations in this case use the following parameters values:

Sampling Frequency for 20MHz = 19.92 Mega Semples per second (MSps)

Bit Width = 24bits

With such settings the following bitrate requirements are calculated

Table 6.1.2.2.2-1 Examples of optimised and maximum required bitrate on a transmission link for one possible PHY/RF based RAN architecture split

| Number of Antenna Ports | Frequency System Bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 200 MHz | 1GHz |
| 2 | 0.5 Gbps 1Gbps | 1 Gbps, 2Gbps | 10 Gbps, 20Gbps | 50 Gbps, 100Gbps |
| 8 | 2 Gbps, 4Gbps | 4 Gbps, 8Gbps | 40 Gbps, 80Gbps | 200 Gbps, 400Gbps |
| 64 | 16 Gbps, 32Gbps | 32 Gbps, 64Gbps | 320 Gbps, 640Gbps | 1600 Gbps, 3200Gbps |
| 256 | 64 Gbps, 128Gbps | 128 Gbps, 256Gbps | 1280 Gbps, 2560Gbps | 6400 Gbps, 12800Gbps |

NOTE 1:	Bitrate requirement on a transmission link = Number of BS antenna elements * Sampling frequency (proportional to System bandwidth) * bit width (per sample) + overhead.

NOTE 2:	Although it is assumed that 5G systems may use up to 1GHz frequency bandwidth and up to 256 antenna ports, scenarios that make use of very high system bandwidths and very high antenna port numbers (e.g. case of 1GHz and 256 antenna ports) should not be taken as a main scenario of  reference

----------------------------------------------End of Changes----------------------------------------------