3GPP TSG-RAN WG3 Meeting #93-Bis	R3-162546

Sophia Antipolis, France, 10-14 October 2016

Source: 	Ericsson

Title:	Clarifications on fronthaul bitrate requirements

Agenda Item:	10.6.1.1

Document for:	Discussion and Decision

# Introduction

TR 38.801 includes a section on transport network requirements where an estimate of the required bit rate over a fronthaul interface between base band and radio frequency layers is provided. However, the estimated figures included in the TR only represent a theoretical maximum of the bitrate required.

This paper proposes to clarify that the figures currently in the TR are maximum theoretical values.

Notably, these considerations mirror only if traditional CPRI interconnects are being used. There are multiple possibilities to handle high bandwidths in optical fibres as eg WDM, as well as possibilities on network level.

# Clarifications on fronthaul bitrate requirements

TR38.801 reports the following table of required bitrates over a baseband to radio frequency layer fronthaul (namely, the architecture followed is the typical one used for the CPRI interface):

Table 6.1.2.2.2-1 Examples of required bitrate on a transmission link for one possible PHY/RF based RAN architecture split

| Number of Antenna Ports | Frequency System Bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 200 MHz | 1GHz |
| 2 | 1Gbps | 2Gbps | 20Gbps | 100Gbps |
| 8 | 4Gbps | 8Gbps | 80Gbps | 400Gbps |
| 64 | 32Gbps | 64Gbps | 640Gbps | 3200Gbps |
| 256 | 128Gbps | 256Gbps | 2560Gbps | 12800Gbps |

NOTE 1:	Peak bitrate requirement on a transmission link = Number of BS antenna elements * Sampling frequency (proportional to System bandwidth) * bit width (per sample) + overhead.

It is understood that the Sampling Frequency and Bit Width used for the calculation in the table above are maximum theoretical values of what these parameters could be set at. In particular, the calculations in the table seem to assume a sampling frequency of 30.72 Mega Sample per second (MSps) for each 20MHz while the Bit Width is set to 30. These values are maximum figures for these parameters. 
It should also be pointed out that, despite it is true that TR 38.913 contemplates system bandwidths of up to 1GHz and antenna ports of up to 256, it is rather unrealistic to think that an implementation will use a system bandwidth of 1GHz in conjunction with a maximum number of antenna ports equal to 256. For this reason, the figures in the table of section 6.1.2.3 of TR38.801 should be interpreted with caution. In order to guide the TR reader it should be pointed out that extreme scenarios where the use of a very high system bandwidth is combined with the use of a high number of antenna ports (e.g. 1GHz and 256 antenna ports) should not be taken as main scenarios of reference.

It has therefore to be made clear in TR38.801 that firstly the shown figures are absolute maximum rather than the bit rates that can be achieved by a careful choice of sampling frequency and bit width. Secondly, it has to be made clear that extreme cases of very high system bandwidth and very high number of antenna ports should not be taken as main scenarios of reference.

It is therefore proposed to agree to the TP in the following sections where clarifications on the currently reported bitrate estimates are made and where a new example showing bitrate requirements for a fronthaul with optimised settings are shown.

# Conclusions

In this paper clarifications are made on the bitrate requirements reported in TR38.801 for a fronthaul interface based on a baseband to RF split, namely based on a CPRI model.

Proposal: it is proposed to agree to the TP in this paper, which clarifies under what conditions the reported bitrate requirements in the TR are calculated.

# Text Proposal

----------------------------------------------Start of Changes----------------------------------------------

#### 6.1.2.3	Transport network aspects

##### 6.1.2.3.1	General

This section summarizes transport network requirements of different functional splits.

NOTE 1:	It is understood that RAN3 has no intention to specify any transport network.

##### 6.1.2.3.2	Transport network requirements for an example RAN architecture

According to TR 38.913 [5], the NR shall support up to 1GHz system bandwidth, and up to 256 antennas. A calculation relative to one of several possible transport deployments applied to a possible RAN architecture example shows that transmission between base band part and radio frequency part requires a theoretical maximum bitrate over the transport network of about 614.4Mbps per 10MHz mobile system bandwidth per antenna port.

When the system bandwidth is increasing as well as the number of antenna ports, the required bitrate is linearly increasing. An example with rounded numbers is shown in the following table. Note that the figures in Table 6.1.2.2.2-1 are a maximisation of the needed bandwidth per number of antenna ports and frequency bandwidth.

Table 6.1.2.2.2-1 Examples of maximum required bitrate on a transmission link for one possible PHY/RF based RAN architecture split

| Number of Antenna Ports | Frequency System Bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 200 MHz | 1GHz |
| 2 | 1Gbps | 2Gbps | 20Gbps | 100Gbps |
| 8 | 4Gbps | 8Gbps | 80Gbps | 400Gbps |
| 64 | 32Gbps | 64Gbps | 640Gbps | 3200Gbps |
| 256 | 128Gbps | 256Gbps | 2560Gbps | 12800Gbps |

NOTE 1:	Peak bitrate requirement on a transmission link = Number of BS antenna elements * Sampling frequency (proportional to System bandwidth) * bit width (per sample) + overhead. The calculation is made for sampling frequency of 30.72 Mega Sample per second for each 20MHz and for a Bit Width equal to 30

----------------------------------------------End of Changes----------------------------------------------