3GPP TSG RAN WG3 Meeting #94	R3-163056

Reno, Nevada, USA, 14th – 18th November 2016

3GPP TSG-RAN WG2 #95bis	Tdoc R2-167313

# Kaohsiung, Taiwan, 10th – 14th October 2016

Title:	LS on RAN2 agreements for NR DL-based mobility and Cell definition

Response to:	-

Release:	-

Work Item:	FS_NR_newRAT

Source:	RAN2

To:	RAN1

Copy: 	RAN3, RAN4

Contact Person:

Name:	Icaro Leonardo da Silva

- E-mail Address:	icaro.leonardo.da.silva@ericsson.com

Attachments:	None

# Overall description

In RAN2#95bis, RAN1 provided an LS with updates regarding initial access and mobility [1]. RAN1 has requested RAN2 updates concerning the cell definition and RRC driven mobility. In RAN2#95, RAN2 has discussed DL-based mobility in RRC_CONNECTED mode and the following has been agreed:

Agreements for DL-based mobility in RRC_CONNECTED mode (optimized for data transmission, at least for network-controlled mobility) mobility with RRC involvement, concerning beams and the relation to the NR cell definition:

1. UE at least measures one or more individual beams and gNB should have mechanisms to consider those beams to perform HO. Note: This is necessary at least to trigger inter-gNB handovers and to optimize HO ping-pongs / HO failures.

–	FFS: whether UE report individual and/or combined quality of multiple beams

2. UE should be able to distinguish between the beams from its serving cell and beams from non-serving cells for RRM measurements in active mobility. UE should be able to determine if a beam is from its serving cell.

–	FFS whether serving/non serving cell may be termed 'serving/non serving set of beam)

–	FFS: whether the UE is informed via dedicated signalling or implicitly detected by the UE based on some broadcast signals.

-	FFS how the cell in connected relates to the cell in idle

3. 	Study how to derive a cell quality based on measurements from individual beams

4. 	In connected mode, intra-cell mobility can be handled by mobility without RRC involvement.

-FFS whether there may be cases that do require RRC involvement.

5	UE should be able to identify a beam. FFS how beams are identified (to be defined by RAN1)

Agreement

1	For connected mode, cell level signal quality for RRM measurement can be derived from multiple beams, if multiple beams can be detected (this does not preclude RRM measurement made on a single beam)

FFS how to combine beam measurements to a cell level single quality

Agreements for IDLE

1	In IDLE mode, UE performs cell selection and reselection on NR Cells.

2	Study how to derive a cell quality based on measurements.

Actions

RAN2 respectfully asks RAN1 to take these updates into account in the design of reference signals and synchronization signals, definition of identifiers, initial access and mobility..

As it can be seen from the RAN2 agreements, one of the open issues was how the cell in connected relates to the cell in Idle. A specific issue that was identified as relevant:

- What are the RAN1 assumptions on whether the signals (e.g. reference and/or synchronisation signals) supporting idle or connected mode operations are transmitted in the same manner or not such as in terms of beamforming configuration (e.g. single beam, multi-beam)?"

Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #96	November 14-18, 2016		       Reno, Nevada

TSG RAN WG2 Meeting AdHoc	January 17-19, 2016		       US

# References

R2-166005, LS on RAN1 agreements for NR initial access and mobility RAN1, Ericsson, RAN2#95bis, October 2016