3GPP TSG-RAN WG3 Meeting #95bis	R3-170943

Spokane, USA, April 3rd – 7th, 2017

Agenda item:		10.12.2

Source:	LG Electronics Inc.

Title:	Solution Evaluation for Path switch over NG-C

Document for:		Decision

1. Introduction

In TR 38.801, we have captured two candidate solutions for Xn based intra-system handover procedures. This paper is to compare the two solutions with the comparison index, based on which the down selection is performed. The corresponding proposal is provided.

2. Discussion

2.1 Evaluation on the two candidate solutions

During the study phase, we have agreed the principle on intra-system mobility given as follows:

- Principle: The LTE X2 handover procedure as in TS 36.300 Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO is taken as a baseline for intra-system mobility i.e. intra RAT (gNB <-> gNB; eLTE eNB <-> eLTE eNB) and inter-RAT (eLTE-eNB <-> gNB).

This means that the LTE handover procedure in TS 36.300 is a baseline.

In addition, another variant with in-band path switch was also adopted into the TR 38.801.

Fig. 1. Intra-new RAN Handover using in-band Path Switch over NG-U.

In the following section, we are going to investigate some aspects on this solution and the baseline solution.

First, in the in-band path switch solution gNB2 triggers the path switch to U-GW in step 2, which sends end marker directly to source gNB1 side. But for the baseline solution, NG-CP receives the path switch request and forwards this message to UP-GW, which sends the end marker at that time. Therefore on the latency optimization, the benefit of in-band path switch solution is about one message faster from the timing of sending end marker point of view.

Secondly, another advantage claimed on in-band path switch solution is that the U-GW may transmit packets to target gNB2 as early as possible. This is under the assumption that the U-GW is not changed by NG-CP. However, when step 4 arrives at NG-CP, which may decide to change the U-GW for this PDU session based on the load situation among the U-GW. The path switch has to be done again. Thus the path switch is performed twice from target gNB point of view.

Thirdly, it is to analyze the potential issue from the security point of view. For the in-band path switch solution, U-GW transmits the packets as early as possible after receiving step 2. Some downlink packets may arrive at the target gNB2 before it receives the updated security context in step 4. Thus the packets have to be transmitted without security guaranteed.

Another issue of the in-band path switch solution should be clarified, that is, whether a user plane update request/response is necessary from NG-CP to U-GW after the NG-CP receives the step 4. This is also important to judge on whether in-band path switch solution can reduce the signaling or not.

As mentioned in [3], two more issues should be also be solved for the in-band path switch solution.

- UL data packets always exist during handover

- How to do path swith for CP based NB-IoT since no user plane exists.

Based on the analysis above, the following comparison table is given for the two solutions:

|  | In-band Path Switch Solution | CP based Path Switch Solution |
| --- | --- | --- |
| Latency Optimization on path switch | About one message faster compared to the baseline CP solution | Normal |
| Number of path switch in case of U-GW Relocation needed: target gNB2 point of view | Twice | Once |
| Security | Not guaranteed for the early switched packets | Guaranteed for the switched packets |
| Signaling Reducing | Not clear | Normal |
| In cast that UL data packets does not exist during Handover | Not clear | Normal |
| CP based NB-IoT | Not clear | Normal |

Based on the analysis above, the in-band path switch solution has no clear advantages compared with the baseline CP based solution. The following proposal is suggested to RAN3:

Proposal 1): To select the baseline CP based solution as the solution for for intra-system mobility i.e. intra RAT (gNB <-> gNB; eLTE eNB <-> eLTE eNB) and inter-RAT (eLTE-eNB <-> gNB).

3. Conclusion

In this contribution, the comparison on the two solutions for Xn based intra-RAT mobility procedures was performed and the corresponding conclusion was also drawn. The following proposal is suggested to RAN3:

Proposal 1): To select the baseline CP based solution as the solution for for intra-system mobility i.e. intra RAT (gNB <-> gNB; eLTE eNB <-> eLTE eNB) and inter-RAT (eLTE-eNB <-> gNB).

4. References

- RP-160576, “New SID Proposal: Study on New Radio Access Technology”

- TR 38.801

- R3-170136, “Path Switch Alternatives”, NEC

- R3-170493, “TP on Evaluation and Conclusion for Intra-system mobility procedures”, LG Electronics