3GPP TSG-RAN WG3 Meeting #95bis	R3-171179

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	31.1

Source:	Ericsson

Title:	QoE SRNS Relocation Enhancement

Document for:	Discussions & Approval

# 1	Background

At RAN3#95 meeting the work item QoE [1] was completed. Related to SRNS relocation, it has been implemented that the QoE measurement would continue.

This paper discusses how the SRNS relocation could be enhanced in the QoE measurement feature.

# 2	Discussion

During SRNS Relocation, the source RNC would include in the Relocation Required message the “UE Application Layer Measurement” if there is such configured and started. This information is further sent from CN to the target RNC. The UE will then after the SRNS relocation, send the QoE measurement result to the target SRNS. The QoE measurement result will be packed in a DASH container with a size of 60kb before compression.

In the case that the target SRNS wish to support QoE measurement, one enhancement would be that the target SRNS feedback to the source RNC during the SRNS relocation procedure, so that the source SRNC understands the QoE measurement will be continued. Otherwise the source SRNS may stop the QoE measurement before the actual hand over.

When the QoE measurement is stopped by the source RNC, the big sized QoE measurement result would be skipped to send from the UE to the target.

Proposal 1: RAN3 to agree to include a feedback indication from the target SRNC to the source SRNC during SRNS relocation.

CR is submitted in [2].

# 3	Proposals

Proposal 1: RAN3 to agree to include a feedback indication from the target SRNC to the source SRNC during SRNS relocation.

# 4	Reference

- [1] 	RP-161917	New WI proposal: Quality of Experience (QoE) Measurement Collection for streaming services in UTRAN, China Unicom

- [2] 	R3-171180	QoE enhancement during SRNS relocation, Ericsson