3GPP TSG-RAN WG3 Meeting #95bis	R3-171183

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	31.2

Source:	Ericsson

Title:	Flexible eNB ID correction

Document for:	Discussions & Approval

# 1	Background

At RAN3#95 meeting the work item Flexible eNB ID and Cell ID was completed.

This paper is to propose the correction needed in the specification.

# 2	Discussion

## 2.1	Correction in RRC Resume ID

We have discussed and agreed that the introduction of different length of the Macro eNB-ID would not impact how the resume ID is used.

The RRC Resume ID is 40 bits long. That is not to be impacted. All the functions related the use of Resume ID would be performed under the condition that RAN would know how to interpret the resume ID via configuration.

Pre Rel 14, the RRC Resume ID consistes of 40 bits while 20 bits of MSB represent the Macro eNB-ID. With the flexible eNB ID feature, the descriptioin should be updated to reflect that the Macro eNB-ID could be 18 bits or 21bits in the RRC Resume ID.

## 2.2	Correction in ECGI

We have discussed and agreed that the introduction of different length of the Macro eNB-ID would not impact how the resume ID is used.

Proposal 1: RAN3 to agree that the correction included in [1] and [2].

# 3	Proposals

Proposal 1: RAN3 to agree that the correction included in [1] and [2].

# 4	Reference

- [1] 	R3-171184	Correction of eNB ID part in Resume ID not truncated, Ericsson

- [2] 	R3-171185	Correction of eNB ID part in E-UTRAN CGI, Ericsson