3GPP TSG-RAN WG3 Meeting #95bis	R3-171185

# Spokane, USA, 3rd -7th  April 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.444 | CR | 0077 | rev |  | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eNB ID part in E-UTRAN CGI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_FNBID-Core |  |  |  |  |  |  | Date: |  | 2017-03-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The semantic description for “E-UTRAN CGI” needs correction after the introduction of 21bits and 18 bits Macro eNB-ID in WI LTE_FNBID-Core |  |  |  |  |  |  |  |  |
| Summary of change: |  | Semantic Description for E-UTRAN CGI is corrected to reflects the 18bits, 20bits and 21bits Macro eNB-ID length.The reference is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect semantic description in 9.2.1.12 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR …CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.1.12	E-UTRAN CGI

This information element is used to globally identify a cell (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.7 |  |
| Cell Identity | M |  | BIT STRING (28) | The 18, 20 or 21 leftmost bits of the Cell Identity correspond to the eNB ID (Global eNB ID IE defined in section TS 36.413 [11]). |