3GPP TSG-RAN WG3 Meeting #95bis	R3-171290

# Spokane, USA, 3rd -7th  April 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1321 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE Application Layer Measurement Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UMTS_QMC_Streaming-Core |  |  |  |  |  |  | Date: |  | 2017-03-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | RAN needs to notify the CN if the UE is capable to support the UE Application Layer Measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE Application Layer Measurement Capability IE is added in Initial UE Message. Revision 1: Change the UE Application Layer Measurement Capability IE from OCTET STRING to BIT STRING. Rebase on v14.0.0, Editorial change on ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CN may initiate UE Application Layer Measurement towards the not capable UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.22, 8.22.2, 9.1.33, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR …CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.22	Initial UE Message

### 8.22.1	General

The purpose of the Initial UE Message procedure is to establish an Iu signalling connection between a CN domain and the RNC and to transfer the initial NAS-PDU to the CN node as determined by the NAS Node Selection Function - if this function is active, or otherwise to the default CN node- or by the Rerouting Function (see TS 25.410 [2]) in case of MOCN configuration. The procedure uses connection oriented signalling.

### 8.22.2	Successful Operation

Figure 24: Initial UE Message procedure. Successful operation.

When the RNC has received from radio interface a NAS message (see TS 24.008 [8]) to be forwarded to a CN domain to which no Iu signalling connection for the UE exists, the RNC shall initiate the Initial UE Message procedure and send the INITIAL UE MESSAGE message to the CN. If NNSF is active, the selection of the CN node is made according to TS 23.236 [26].

In addition to the received NAS-PDU, the RNC shall add the following information to the INITIAL UE MESSAGE message:

-	CN domain indicator, indicating the CN domain towards which this message is sent.

-	For CS domain, the LAI which is the last LAI indicated to the UE by the UTRAN via the current RRC connection, or if the UTRAN has not yet indicated any LAI to the UE via the current RRC connection, then the LAI of the cell via which the current RRC connection was established.

-	For PS domain, the LAI+RAC which are the last LAI+RAC indicated to the UE by UTRAN via the current RRC connection, or if the UTRAN has not yet indicated any LAI+RAC to the UE via the current RRC connection, then the LAI+RAC of the cell via which the current RRC connection was established.

-	Service Area corresponding to at least one of the cells from which the UE is consuming radio resources.

-	Iu signalling connection identifier.

-	Global RNC identifier.

-	Selected PLMN Identity, if received from radio interface by a network sharing supporting UE in shared networks.

-	Redirect Attempt Flag, in MOCN configuration for a network sharing non supporting UE in order to indicate that the CN shall respond with a Redirection Indication IE or a Redirection Completed IE.

The Iu Signalling Connection Identifier IE contains an Iu signalling connection identifier which is allocated by the RNC. The value for the Iu Signalling Connection Identifier IE shall be allocated so as to uniquely identify an Iu signalling connection for the RNC. The CN should store and remember this identifier for the duration of the Iu connection.

Whereas several processing entities within the CN (e.g. charging, interception, etc.) may make use of the location information given in the SAI IE and the LAI (and RAC for PS domain) IE, the mobility management within the CN shall rely on the information given within the LAI IE (respectively LAI and RAC IEs for PS domain) only.

If the establishment of the Iu signalling connection towards the CN is performed due to an RRC connection establishment originating from a CSG cell and if the UE is CSG capable, the CSG Id IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the Iu signalling connection towards the CN is performed due to an RRC connection establishment originating from a Hybrid cell and if the UE is CSG capable, the CSG Id IE and Cell Access Mode IE shall be included in the INITIAL UE MESSAGE message.

If the RNC has a co-located L-GW for LIPA operation, it shall include the L-GW Transport Layer Address IE in the INITIAL UE MESSAGE message.

If the SIPTO L-GW Transport Layer Address IE is received in the INITIAL UE MESSAGE message, the CN shall, if supported, use it for SIPTO@LN operation as specified in TS 23.060 [21].

If the LHN ID IE is included in the INITIAL UE MESSAGE message, the CN shall, if supported, use it as specified in TS 23.060 [21].

If the Higher bitrates than 16 Mbps flag IE is included in the INITIAL UE MESSAGE message then the CN shall, if supported, use the IE as described in TS 23.060 [21].

If the Tunnel Information for BBF IE is received in the INITIAL UE MESSAGE message, the CN shall, if supported, use the IE as described in TS 23.139 [65].

If the SGSN Group Identity IE is included in the INITIAL UE MESSAGE message this indicates that the message is rerouted for DCN, and the SGSN shall, if supported, use the IE as described in TS 23.060 [21].

If the UE Usage Type IE is included in the INITIAL UE MESSAGE message, then the selected SGSN node in the DCN shall if supported, use it as defined in TS 23.401 [48].

If the DCN ID IE is included in the INITIAL UE MESSAGE message, the SGSN shall, if supported, use it as defined in TS 23.401 [48].

If the UE indicates the support for UE Application Layer Measurement, the RNC shall if supported include the UE Application Layer Measurement Capability IE in the INITIAL UE MESSAGE message.

Interaction with Direct Transfer procedure

In MOCN configuration, if the RNC receives the Redirection Indication IE in the DIRECT TRANSFER message from a CN node which is not the last attempted, it shall initiate the Initial UE Message procedure towards another CN operator when possible (or possibly to the same CN in case when CS/PS coordination is required), with the following additional information in the INITIAL UE MESSAGE message:

-	NAS Sequence Number IE, if received from previously attempted CN operator;

-	Permanent NAS UE Identity IE, if received from one of previously attempted CN operators.

In MOCN configurations, if the RNC receives the Redirection Indication IE containing the Additional CS/PS coordination information IE in the DIRECT TRANSFER message from a CN node and if the RNC cannot identify one operator from this information as described in TS 23.251 [39], then the RNC shall if supported initiate the UE Registration Query procedure towards the CN nodes of the other domain before initiating the Initial UE Message procedure towards another CN operator.

#### 8.22.2.1	Successful Operation for GERAN Iu-mode

For GERAN Iu-mode, the following shall apply in addition for the successful operation of the Initial UE Message procedure:

-	In case of establishment of a signalling connection towards the CS domain in GERAN Iu-mode, the INITIAL UE MESSAGE message shall contain the GERAN Classmark IE in order to provide the CN with GERAN-specific information (see TS 43.051 [27]).

------------------------------------------

Skip to next change

------------------------------------------

### 9.1.33	INITIAL UE MESSAGE

This message is sent by the RNC to transfer the radio interface initial layer 3 message to the CN.

Direction: RNC  CN.

Signalling bearer mode: Connection oriented.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| CN Domain Indicator | M |  | 9.2.1.5 |  | YES | ignore |
| LAI | M |  | 9.2.3.6 |  | YES | ignore |
| RAC | C - ifPS |  | 9.2.3.7 |  | YES | ignore |
| SAI | M |  | 9.2.3.9 |  | YES | ignore |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | ignore |
| Iu Signalling Connection Identifier | M |  | 9.2.1.38 |  | YES | ignore |
| Global RNC-ID | M |  | 9.2.1.39 | If the Extended RNC-ID IE is included in the message, the RNC-ID IE in the Global RNC-ID IE shall be ignored. | YES | ignore |
| GERAN Classmark | O |  | 9.2.1.57 |  | YES | ignore |
| Selected PLMN Identity | O |  | 9.2.3.33 |  | YES | ignore |
| NAS Sequence Number | O |  | 9.2.3.34 |  | YES | ignore |
| Permanent NAS UE Identity | O |  | 9.2.3.1 |  | YES | ignore |
| Redirect Attempt Flag | O |  | 9.2.3.50 |  | YES | ignore |
| Extended RNC-ID | O |  | 9.2.1.39a | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. | YES | reject |
| CSG Id | O |  | 9.2.1.85 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.93 |  | YES | reject |
| L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to LIPA operation. | YES | ignore |
| Higher bitrates than 16 Mbps flag | O |  | 9.2.3.54 | May only be included towards the PS domain. | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.6 | Indicating HNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to SIPTO@LN operation. | YES | ignore |
| LHN ID | O |  | 9.2.1.123 |  | YES | ignore |
| SGSN Group Identity | O |  | 9.2.3.57 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| DCN ID | O |  | INTEGER (0..65535) |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifPS | This IE shall be present if the CN Domain Indicator IE is set to "PS domain". |

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for RANAP.

--

-- **************************************************************

RANAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AccuracyFulfilmentIndicator,

APN,

BroadcastAssistanceDataDecipheringKeys,

LocationRelatedDataRequestType,

LocationRelatedDataRequestTypeSpecificToGERANIuMode,

DataVolumeReference,

CellLoadInformation,

AreaIdentity,

CN-DomainIndicator,

Cause,

Cell-Access-Mode,

ClientType,

CriticalityDiagnostics,

ChosenEncryptionAlgorithm,

ChosenIntegrityProtectionAlgorithm,

ClassmarkInformation2,

ClassmarkInformation3,

CSG-Id,

CSG-Id-List,

CSG-Membership-Status,

DL-GTP-PDU-SequenceNumber,

DL-N-PDU-SequenceNumber,

DataVolumeReportingIndication,

DeltaRAListofIdleModeUEs,

DRX-CycleLengthCoefficient,

EncryptionInformation,

EncryptionKey,

E-UTRAN-Service-Handover,

ExtendedRNC-ID,

FrequenceLayerConvergenceFlag,

GERAN-BSC-Container,

GERAN-Classmark,

GlobalCN-ID,

GlobalRNC-ID,

GTP-TEI,

IncludeVelocity,

InformationExchangeID,

InformationExchangeType,

InformationRequested,

InformationRequestType,

InformationTransferID,

InformationTransferType,

InterSystemInformationTransferType,

IntegrityProtectionInformation,

IntegrityProtectionKey,

InterSystemInformation-TransparentContainer,

IPMulticastAddress,

IuSignallingConnectionIdentifier,

IuTransportAssociation,

KeyStatus,

L3-Information,

LAI,

LastKnownServiceArea,

Correlation-ID,

MBMS-PTP-RAB-ID,

MBMSBearerServiceType,

MBMSCountingInformation,

MBMSCNDe-Registration,

MBMSHCIndicator,

MBMSRegistrationRequestType,

MBMSServiceArea,

MBMSSessionDuration,

MBMSSessionIdentity,

MBMSSessionRepetitionNumber,

MSISDN,

NAS-PDU,

NAS-SequenceNumber,

NAS-SynchronisationIndicator,

NewBSS-To-OldBSS-Information,

NonSearchingIndication,

NumberOfSteps,

Offload-RAB-Parameters,

Offload-RAB-Parameters-APN,

Offload-RAB-Parameters-ChargingCharacteristics,

OMC-ID,

OldBSS-ToNewBSS-Information,

PagingAreaID,

PagingCause,

PDP-TypeInformation,

PDP-TypeInformation-extension,

PermanentNAS-UE-ID,

PLMNidentity,

PositionData,

PositionDataSpecificToGERANIuMode,

PositioningPriority,

ProvidedData,

PowerSavingIndicator,

RAB-ID,

RAB-Parameters,

RAC,

RAListofIdleModeUEs,

RAT-Type,

RedirectAttemptFlag,

RedirectionCompleted,

RejectCauseValue,

RelocationType,

RequestedGANSSAssistanceData,

RequestType,

Requested-RAB-Parameter-Values,

ResponseTime,

RRC-Container,

RSRVCC-Operation-Possible,

RSRQ-Type,

RSRQ-Extension,

SAI,

SAPI,

Service-Handover,

SessionUpdateID,

SNA-Access-Information,

SourceBSS-ToTargetBSS-TransparentContainer,

SourceID,

Source-ToTarget-TransparentContainer,

SourceRNC-ToTargetRNC-TransparentContainer,

SRVCC-HO-Indication,

SRVCC-Information,

SRVCC-Operation-Possible,

TargetBSS-ToSourceBSS-TransparentContainer,

TargetID,

Target-ToSource-TransparentContainer,

TargetRNC-ToSourceRNC-TransparentContainer,

TemporaryUE-ID,

TimeToMBMSDataTransfer,

TMGI,

TracePropagationParameters,

TraceReference,

TraceType,

UnsuccessfullyTransmittedDataVolume,

TransportLayerAddress,

TriggerID,

UE-AggregateMaximumBitRate,

UE-ID,

UESBI-Iu,

UL-GTP-PDU-SequenceNumber,

UL-N-PDU-SequenceNumber,

UP-ModeVersions,

UserPlaneMode,

VelocityEstimate,

VerticalAccuracyCode,

VoiceSupportMatchIndicator,

Alt-RAB-Parameters,

Ass-RAB-Parameters,

PeriodicLocationInfo,

SubscriberProfileIDforRFP,

RNSAPRelocationParameters,

RABParametersList,

MDT-Configuration,

Priority-Class-Indicator,

Management-Based-MDT-Allowed,

HigherBitratesThan16MbpsFlag,

End-Of-CSFB,

Out-Of-UTRAN,

RSRVCC-HO-Indication,

RSRVCC-Information,

MDT-PLMN-List,

TunnelInformation,

LHN-ID,

Session-Re-establishment-Indicator,

Additional-CSPS-coordination-information,

UERegistrationQueryResult,

SGSN-Group-Identity,

P-TMSI,

BarometricPressure,

CivicAddress,

UE-Usage-Type,

DCN-ID,

UE-Application-Layer-Measurement-Capability

FROM RANAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Container{},

RANAP-PRIVATE-IES,

RANAP-PROTOCOL-EXTENSION,

RANAP-PROTOCOL-IES,

RANAP-PROTOCOL-IES-PAIR

------------------------------------------

Skip to next change

------------------------------------------

id-LHN-ID,

id-LastE-UTRANPLMNIdentity,

id-Session-Re-establishment-Indicator,

id-Additional-CSPS-coordination-information,

id-UERegistrationQueryResult,

id-IuSigConIdRangeEnd,

id-SGSN-Group-Identity,

id-P-TMSI,

id-RANAP-Message,

id-PowerSavingIndicator,

id-BarometricPressure,

id-CivicAddress,

id-UE-Usage-Type,

id-DCN-ID,

id-UE-Application-Layer-Measurement-Capability

------------------------------------------

Skip to next change

------------------------------------------

InitialUE-MessageExtensions RANAP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 5 to enable GERAN support over Iu-cs --

{ ID id-GERAN-Classmark					CRITICALITY ignore	EXTENSION GERAN-Classmark					PRESENCE optional}|

-- Extension for Release 6 to convey the selected PLMN id in shared networks --

{ ID id-SelectedPLMN-ID	 				CRITICALITY ignore	EXTENSION PLMNidentity						PRESENCE optional}|

-- Extension for Release 6 to enable rerouting in MOCN configuration for network sharing non-supporting UEs --

{ ID id-PermanentNAS-UE-ID				CRITICALITY ignore	EXTENSION PermanentNAS-UE-ID				PRESENCE optional}|

-- Extension for Release 6 to enable rerouting in MOCN configuration for network sharing non-supporting UEs --

{ ID id-NAS-SequenceNumber				CRITICALITY ignore	EXTENSION NAS-SequenceNumber				PRESENCE optional}|

-- Extension for Release 6 to indicate rerouting in MOCN configuration for network sharing non-supporting UEs --

{ ID id-RedirectAttemptFlag				CRITICALITY ignore	EXTENSION RedirectAttemptFlag				PRESENCE optional}|

-- Extension for Release 7 to indicate extended RNC-ID --

{ ID id-ExtendedRNC-ID					CRITICALITY reject	EXTENSION ExtendedRNC-ID					PRESENCE optional}|

-- Extension for Release 8 to support CSG --

{ ID id-CSG-Id							CRITICALITY reject	EXTENSION CSG-Id							PRESENCE optional}|

-- Extension for Release 9 to allow communication of the cell access mode --

{ ID id-Cell-Access-Mode				CRITICALITY reject	EXTENSION Cell-Access-Mode					PRESENCE optional}|

-- Extension for Release 10 to support LIPA --

{ ID id-LGW-TransportLayerAddress		CRITICALITY ignore	EXTENSION TransportLayerAddress				PRESENCE optional}|

-- Extension for Release 9 to enable the CN to handle potential UE NAS QoS issues related to higher bitrates --

{ ID id-HigherBitratesThan16MbpsFlag	CRITICALITY ignore	EXTENSION HigherBitratesThan16MbpsFlag	PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF		CRITICALITY ignore	EXTENSION TunnelInformation					PRESENCE optional}|

-- Extension for Release 12 to support SIPTO@LN with collocated L-GW--

{ ID id-SIPTO-LGW-TransportLayerAddress	CRITICALITY ignore	EXTENSION TransportLayerAddress				PRESENCE optional}|

-- Extension for Release 12 to support SIPTO@LN Stand-Alone --

{ ID id-LHN-ID							CRITICALITY ignore	EXTENSION LHN-ID							PRESENCE optional}|

-- Extension for Release 13 to support DCN --

{ ID id-SGSN-Group-Identity				CRITICALITY ignore	EXTENSION SGSN-Group-Identity				PRESENCE optional}|

{ ID id-UE-Usage-Type					CRITICALITY ignore	EXTENSION UE-Usage-Type						PRESENCE optional}|

{ ID id-DCN-ID							CRITICALITY ignore	EXTENSION DCN-ID							PRESENCE optional}|

{ ID id-UE-Application-Layer-Measurement-Capability		CRITICALITY ignore	EXTENSION UE-Application-Layer-Measurement-Capability							PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

------------------------------------------

Skip to next change

------------------------------------------

#### -- U

UE-AggregateMaximumBitRate ::= SEQUENCE {

uE-AggregateMaximumBitRateDownlink		UE-AggregateMaximumBitRateDownlink	OPTIONAL,

uE-AggregateMaximumBitRateUplink		UE-AggregateMaximumBitRateUplink	OPTIONAL,

...

}

UE-AggregateMaximumBitRateDownlink			::= INTEGER (1..1000000000)

-- Unit is bits per sec

UE-AggregateMaximumBitRateUplink			::= INTEGER (1..1000000000)

-- Unit is bits per sec

UE-Application-Layer-Measurement-Capability ::= BIT STRING (SIZE (8))

-- First bit: QoE Measurement

-- Note that undefined bits are considered as a spare bit and spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.

UE-History-Information ::= OCTET STRING

UE-ID ::= CHOICE {

imsi				IMSI,

imei				IMEI,

...,

imeisv				IMEISV

}

UE-IsNotServed::= SEQUENCE {

permanentNAS-UE-ID		PermanentNAS-UE-ID,

iE-Extensions			ProtocolExtensionContainer { { UE-IsNotServed-ExtIEs } }		OPTIONAL,

...

}

UE-IsNotServed-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

UE-IsServed::= SEQUENCE {

permanentNAS-UE-ID		PermanentNAS-UE-ID,

pLMNidentity			PLMNidentity,

iE-Extensions			ProtocolExtensionContainer { { UE-IsServed-ExtIEs } }		OPTIONAL,

...

}

UE-IsServed-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

UE-Usage-Type ::= INTEGER (0..255)

UERegistrationQueryResult ::= CHOICE {

uE-IsServed			UE-IsServed,

uE-IsNotServed		UE-IsNotServed

}

UESBI-Iu ::= SEQUENCE {

uESBI-IuA		UESBI-IuA	OPTIONAL,

uESBI-IuB		UESBI-IuB	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UESBI-Iu-ExtIEs} } OPTIONAL,

...

}

UESBI-Iu-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

UESBI-IuA				::= BIT STRING (SIZE(1..128))

-- Reference: TR25.994 --

UESBI-IuB				::= BIT STRING (SIZE(1..128))

-- Reference: TR25.995 --

UL-GTP-PDU-SequenceNumber		::= INTEGER (0..65535)

UL-N-PDU-SequenceNumber 		::= INTEGER (0..65535)

UPInformation		::=		SEQUENCE {

frameSeqNoUL			FrameSequenceNumber,

frameSeqNoDL			FrameSequenceNumber,

pdu14FrameSeqNoUL		PDUType14FrameSequenceNumber,

pdu14FrameSeqNoDL		PDUType14FrameSequenceNumber,

dataPDUType				DataPDUType,

upinitialisationFrame	UPInitialisationFrame,

iE-Extensions			ProtocolExtensionContainer { { UPInformation-ExtIEs } }		OPTIONAL,

...

}

UPInformation-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

{ID id-TimingDifferenceULDL		CRITICALITY ignore	EXTENSION TimingDifferenceULDL		PRESENCE optional},

...

}

UPInitialisationFrame	::=	OCTET STRING

UP-ModeVersions				::= BIT STRING (SIZE (16))

USCH-ID						::= INTEGER (0..255)

UserPlaneMode ::= ENUMERATED {

transparent-mode,

support-mode-for-predefined-SDU-sizes,

...

}

UTRAN-CellID ::= SEQUENCE {

pLMNidentity			PLMNidentity,

cellID					TargetCellId,

iE-Extensions			ProtocolExtensionContainer { { UTRAN-CellID-ExtIEs} } OPTIONAL

}

UTRAN-CellID-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

RANAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-RAB-Assignment					INTEGER ::= 0

id-Iu-Release						INTEGER ::= 1

id-RelocationPreparation			INTEGER ::= 2

id-RelocationResourceAllocation		INTEGER ::= 3

id-RelocationCancel					INTEGER ::= 4

------------------------------------------

Skip to next change

------------------------------------------

id-Additional-PositioningDataSet 						INTEGER ::= 284

id-CivicAddress											INTEGER ::= 285

id-SGSN-Group-Identity									INTEGER ::= 286

id-P-TMSI												INTEGER ::= 287

id-RANAP-Message										INTEGER ::= 288

id-PowerSavingIndicator									INTEGER ::= 289

id-UE-Usage-Type										INTEGER ::= 290

id-DCN-ID												INTEGER ::= 291

id-UE-Application-Layer-Measurement-Configuration				INTEGER ::= 292

id-UE-Application-Layer-Measurement-Configuration-For-Relocation				INTEGER ::= 293

id-UE-Application-Layer-Measurement-Capability			INTEGER ::= 294

END