3GPP TSG-RAN WG3 Meeting #95bis	R3-171325

Spokane, USA, 3 – 7, April, 2017

Agenda Item:	   16.1

Source: 	   Ericsson

Title: 			Framework for Discussions on Local Caching

Document for:	Discussion and Decision

# Introduction

During discussions at RAN3-95bis it was agreed to provide a TP for a draft status report based on the proposals made in R3-171175. This paper provides such TP.

# TP for Draft Status Report

--------------------------------------Start of the TP--------------------------------------

Discussions on solutions for local caching shall respect the following framework of conditions:

- When accessing locally cached content, user identities, subscriber information, service policies and NAS service request for cached contents shall be managed by and accessed via CN nodes

- Policies (e.g. bit rate enforcements) to apply for locally cached traffic policing shall be managed by and accessed via CN nodes

- The overall function of charging applied to locally cached content needs to be handled by CN nodes, which need to interact with appropriate charging systems

- QoS assignment and modification for locally cached content needs to be performed by means of interaction with CN nodes in charge of QoS management

- Mobility management for locally cached content needs to rely on a UP anchor point that is central and that does not need to necessarily change when the serving eNB changes. Such central UP anchor is today in the S-GW

- Lawful Interception needs to be applied to locally cached content. This implies that locally cached content delivered to a UE needs to be served by nodes such as the S-GW or the P-GW that can perform LI

--------------------------------------End of the TP--------------------------------------