3GPP TSG-RAN WG3 Meeting # 95bis	R3-171351

Spokane, USA, April 3rd – 7th, 2017

Agenda item:		10.3

Source:	LG Electronics Inc.

Title:	TP for exchange of NSSAI in Xn Setup Procedure

Document for:		Discussion

1. Introduction

During the online discussion, we have agreed:

- Slice information exchange over XN

This paper provides the TP for Xn Setup procedure on the agreement above.

Text Proposal for slice information exchange in Xn Setup Procedure (TS 38.423)

X.y.1	Xn Setup

X.y.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn interface.

The procedure uses non UE-associated signalling.

X.y.1.2	Successful Operation

Figure X.y.z.1-1: Xn Setup, successful operation

A NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to a candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

The initiating NG-RAN node1 may include the S-NSSAI List IE in the XN SETUP REQUEST message. The candidate NG-RAN node2 may also include S-NSSAI List IE in the XN SETUP RESPONSE message. The NG-RAN node receiving the IE may use it accordingly.

Note: The name of the node is FFS and the presence of this IE is FFS.