3GPP TSG-RAN WG3 #96	Tdoc R3-171715

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.02

Source:	Ericsson

Title:	TP for NW slicing support at NG-C establishment and update in TS 38.413

Document for:	Approval

# Introduction

This contribution contains a text proposal for [1]  according to the proposals in [2] .

# Text Proposal

*************** START OF MODIFIED SECTION***********

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the gNB and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

Editor’s Note:	Further details are FFS.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The gNB initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the List of Applicable S-NSSAIs IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider this information in the realization and handling of network slicing.

If the List of Applicable S-NSSAIs IE is included in the NG SETUP RESPONSE message, the gNB shall, if supported, consider this information in the realization and handling of network slicing.

Editor’s Note:	Further details are FFS.

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation

If the AMF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.7.1.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

### 8.7.2	gNB Configuration Update

#### 8.7.2.1	General

The purpose of the gNB Configuration Update procedure is to update application level configuration data needed for the gNB and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any.

Editor’s Note:	Further details are FFS.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: gNB configuration update: successful operation

The gNB initiates the procedure by sending a GNB CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a GNB CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data.

If the List of Applicable S-NSSAIs IE is included in the GNB CONFIGURATION UPDATE message, the AMF shall, if supported, consider this information in the realization and handling of network slicing.

If the List of Applicable S-NSSAIs IE is included in the GNB CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB shall, if supported, consider this information in the realization and handling of network slicing.

***************END OF MODIFIED SECTION***************

*************** START OF MODIFIED SECTION***********

#### 9.1.8.1	NG SETUP REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the gNB to transfer application layer information for an NG-C interface instance.

Direction: gNB  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| Global gNB ID | M |  | <ref> | eNote: definition of Global gNB ID is FFS. It may relate to identification of cells hosted by the gNB (FFS). | YES | reject |
| gNB Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the gNB. eNote: TAs applicable for 5G as well | GLOBAL | reject |
| >TAC | M |  | <ref> | Broadcast TAC | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >>PLMN Identity | M |  | <ref> |  |  |  |
| Default Paging DRX | M |  | <ref> | eNote: assumption that this is common for NR and E-UTRA. (to be verified with RAN2 and is FFS) | YES | ignore |
| List of Applicable S-NSSAIs |  | 0..<maxnoofSlicesonNG> |  | Indicates the network slices applicable for the NG association | GLOBAL | ignore |
| >S-NSSAI | M |  | <ref> | NW slice identifier (FFS) | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSlicesonNG | Max number of NW slices per NG association. Value is FFS |

#### 9.1.8.2	NG SETUP RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| AMF Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| AMF pool and AMF code related information | M |  | <ref> | eNote: Will be similar to EPS, but details are pending discussions in SA2 (FFS) | GLOBAL | reject |
| Relative AMF Capacity | M |  | <ref> | eNote: Load balancing concepts are pending discussions in SA2 (FFS) | YES | ignore |
| List of Applicable S-NSSAIs |  | 0..<maxnoofSlicesonNG> |  | Indicates the network slices applicable for the NG association | GLOBAL | ignore |
| >S-NSSAI | M |  | <ref> | NW slice identifier (FFS) | - |  |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSlicesonNG | Max number of NW slices per NG association. Value is FFS |

#### 9.1.8.3	NG SETUP FAILURE

Editor’s Note:	Further details FFS.

#### 9.1.8.4	GNB CONFIGURATION UPDATE

Editor’s Note:	Further details FFS. The List of Applicable S-NSSAIs IE shall be included with the same definition as for the NG Setup procedure, once the message is defined.

#### 9.1.8.5	GNB CONFIGURATION UPDATE ACKNOWLEDGE

Editor’s Note:	Further details FFS. The List of Applicable S-NSSAIs IE shall be included with the same definition as for the NG Setup procedure, once the message is defined.

#### 9.1.8.6	GNB CONFIGURATION UPDATE FAILURE

Editor’s Note:	Further details FFS.

***************END OF MODIFIED SECTION***************

# References

3GPP TS 38.413 ‘NG RAN – NG Application Protocol (NGAP)’, May 2017.

R3-171713, ‘NW slicing support at NG-C establishment and update’, Ericsson, May 2017.