3GPP TSG-RAN3 Meeting #96	R3-171844

Hangzhou, China, 15 – 19 May, 2017

Title: 	TP on Paging message delivery over F1 to 38.401

Source: 	Huawei

Agenda item:	10.10.1

Document for:	Discussion

Introduction

This paper provides text proposal of paging message delivery over F1 to TS 38.401.

Text proposal

----------------------------------------------------1st Change ---------------------------------------------------------------

10.2.2.2.7	Paging function

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters available. The gNB-CU generates RRC paging message and sends it to gNB-DU as a RRC container. The gNB-CU calculates the paging frame and paging occasion per cell and send them to gNB-DU. The paging area is also decided by gNB-CU and gNB-CU provides cells list for paging delivery to gNB-DU. The gNB-DU sends the RRC paging message to UE on paging occasions in the cells.