3GPP TSG-RAN WG3 Meeting #97	R3-172705

Berlin, Germany, EU, 21 - 25 Aug 2017

Agenda item:	9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Introduction of the Xw GTP container

Document for:	Discussion

# 1	Introduction

At the RAN3 #95-bis meeting, RAN3 discussed the problem of the GTP container: the same container was supposed to be used on X2 and Xw, with possibly conflicting message IDs. Several options were considered: splitting the ID pool, assuming cross-plane detection of the interface used and enabling a new container. Eventually, it was decided to ask CT4 to enable a new container for the purpose of Xw [1]. It was requested to be introduced from Rel.13 onwards.

CT4 discussed the request and agreed to enabled the new container. This was introduced in [2] and RAN3 is notified about the decision in [3]. The name of the new GTP header container for Xw purposes is “Xw RAN Container”.

# 2	Discussion and proposal

The GTP header container is mentioned in two Xw specifications: TS 36.464 and TS 36.465. Therefore, both need to be changed. No other changes are expected though: the format of the new container is identical with the old one, so the actual encoding of the information to be transferred does not change.

It is therefore proposed to agree the attached set of CRs: for TS 36.464 in [4] (Rel.13) and [5] (Rel.14) and for TS 36.465 in [6] (Rel.13) and [7] (Rel.14).

# References

- R3-171381, RAN3 #95-bis

- C4-173037, CT4 #78

- C4-173039, CT4 #78

- R3-172706, RAN3 #97

- R3-172707, RAN3 #97

- R3-172708, RAN3 #97

- R3-172709, RAN3 #97