3GPP TSG-RAN WG2 #97                                                                                     R3-172816

Berlin, Germany, 21st – 25th Aug 2017

Source:	CATT

Title:                     TP for 38.473 on UE Context Management function

Agenda Item:	10.10.2.2

Document for:	Discussion and Decision

- Introduction

This contribution provides a stage3 TP for UE Context Management procedures over F1 interface.

- TP for stage3 38.473

///////////////////////////////////////////////////////Text Proposal/////////////////////////////////////////////////////////////////////////////

- 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification REQUIRED | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |

/////////////////////////////////////////////////////Unchanged omited/////////////////////////////////////////////////////////////////////////////////

8.3	UE Context Management procedures

The purpose of the UE Context management procedure is to establish and release the necessary UE Context over F1. C-RNTI is allocated by gNB-DU during this procedure when UE initially accesses to the gNB.

Editor’s note: The detailed procedure for allocating C-RNTI is FFS.

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the necessary overall initial UE Context including bearer context, etc. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.1-1: UE Context Setup procedure. Successful operation.

The gNB-CU initiates the procedure by sending the UE CONTEXT SETUP REQUEST message to the gNB-DU. The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested bearer resource.

Editor’s Note:	Further details are FFS

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.1-2: UE Context Setup procedure. Unsuccessful operation.

If the gNB-DU is not able to establish an F1 UE context, it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message.

#### 8.3.1.4	Abnormal Conditions (FFS)

### 8.3.2	UE Context Release Request (gNB-DU initiated)

#### 8.3.2.1	General

The purpose of the UE Context Release Request procedure is to enable the gNB-DU to request the gNB-CU to release the UE-associated logical F1-connection due to gNB-DU generated reasons. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: UE Context Release Request procedure. Successful operation.

The gNB-DU controlling a UE-associated logical F1-connection initiates the procedure by generating a UE CONTEXT RELEASE REQUEST message towards the affected gNB-CU node.

Editor’s Note:	Further details are FFS

Interactions with UE Context Release procedure:

The UE Context Release procedure should be initiated upon reception of a UE CONTEXT RELEASE REQUEST message.

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection. The procedure uses UE-associated F1 connection.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release procedure. Successful operation.

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

Editor’s Note:	Further details are FFS

#### 8.3.3.3	Abnormal Conditions (FFS)

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is initiated by the gNB-CU to partly modify the established UE Context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation.

The gNB-CU initiates the procedure by sending the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU.

The gNB-DU shall report, in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU the successful update of the UE context.

Editor’s Note:	Further details are FFS

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation.

In case the UE context update cannot be performed successfully the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message to the gNB-CU.

Editor’s Note:	Further details are FFS

#### 8.3.4.4	Abnormal Conditions (FFS)

### 8.3.5	UE Context Modification Required (gNB-DU initiated)

#### 8.3.5.1	General

The purpose of the UE Context Modification Required procedure is initiated by the gNB-DU to partly modify the established UE Context. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation.

The gNB-DU initiates the procedure by sending the UE CONTEXT MODIFICATION REQUIRED message to the gNB-CU.

The gNB-CU shall report, in the UE CONTEXT MODIFICATION ACKNOWLEDGE message to the gNB-DU the successful update of the UE context.

Editor’s Note:	Further details are FFS

#### 8.3.5.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation.

In case the UE context update cannot be performed successfully the gNB-CU shall respond with the UE CONTEXT MODIFICATION REFUSE message to the gNB-CU.

Editor’s Note:	Further details are FFS

#### 8.3.5.4	Abnormal Conditions (FFS)

/////////////////////////////////////////////////////Unchanged omited/////////////////////////////////////////////////////////////////////////////////

9.2	Message Functional Definition and Content

9.2.x	UE Context Management Messages

9.2.x.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | reject |
| Target cell | M |  | <ref> |  |  |  |
| CHOICE QoS Parameters | M |  |  |  | YES | reject |
| >PDU sessions To Be Added List |  | 1 .. <maxnoofPDUsessions> | <ref> |  |  |  |
| >>PDU session ID | M |  | <reference> |  | – | – |
| >>PDU session Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>F1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | – | – |
| >E-RABs To Be Added List |  | 1 .. <maxnoof Bearers> | <ref> |  |  |  |
| >>E-RAB ID | M |  | <reference> |  | – | – |
| >>E-RAB Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >> F1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | gNB-CU endpoint of the F1transport bearer. For delivery of UL PDUs. | – | – |
| UE radio capability | M |  | <ref> |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoof Bearers | Maximum no. of Bearers. Value is 256 |

9.2.x.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| CHOICE QoS Parameters | M |  |  |  | YES | reject |
| > PDU sessions Admitted To Be Added List |  | 1 .. <maxnoofPDUsessions> | <ref> |  |  |  |
| >>PDU session ID | M |  | <reference> |  | – | – |
| >>F1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | – | – |
| > E-RABs Admitted To Be Added List |  | 1 .. <maxnoof Bearers> | <ref> |  |  |  |
| >>E-RAB ID | M |  | <reference> |  | – | – |
| >> F1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | gNB-DU endpoint of the F1transport bearer. For delivery of DL PDUs. | – | – |
| RRC Container | M |  | <ref> | MAC/RLC/Physical layer configuration | YES | reject |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |

9.2.x.3	UE CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the UE context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| Cause | M |  | <ref> |  | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

9.2.x.4	UE CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the release of the UE-associated F1-logical connection over the F1 interface.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | reject |
| Cause | M |  | <ref> |  | YES | ignore |

9.2.x.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the release of the UE-associated F1-logical connection over the F1 interface.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | <ref> |  | ### YES | ### reject |
| ### Cause | ### M |  | <ref> |  | YES | ignore |

9.2.x.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated F1-logical connection over the F1 interface.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

9.2.x.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | reject |
| UE radio capability | O |  | <ref> |  | YES | ignore |

9.2.x.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the performed UE context updates.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

9.2.x.9	UE CONTEXT MODIFICATION FAILURE

This message is sent by gNB-DU in case the performed UE context update is not successful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| Cause | M |  | <ref> |  | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

9.2.x.10	UE CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to provide UE Context information changes to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | reject |
| RRC Container | O |  | <ref> | MAC/RLC/Physical layer configuration | YES | reject |

9.2.x.11	UE CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to confirm the performed UE context updates.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| RRC Container | O |  | <ref> | RRC message and MAC/RLC/Physical layer configuration | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

9.1.x.12	UE CONTEXT MODIFICATION REFUSE

This message is sent by gNB-CU in case the performed UE context update is not successful.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | <ref> |  | YES | ignore |
| Cause | M |  | <ref> |  | YES | ignore |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |