3GPP TSG-RAN3 Meeting #97	R3-173024

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.461 | CR | 0104 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the FDD L-band (Band 74) into TS 25.461 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_FDD_L_Band-Core |  |  |  |  |  | Date: |  |  | 2017-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of requirements for the FDD L-band as Band 74. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are introduced. -4.3.7 Operating bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for the FDD L-band are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS25.466 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The contents are the same as the draft CR (R4-1705257) endorsed in RAN4#83. |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

### 4.3.7	Operating bands

A UTRA/FDD BS, UTRA/TDD BS, E-UTRA BS or antenna modem is designed to operate in one or several of the following frequency bands:

Table 4.3.7.1: Frequency bands

| UTRA Operating Band | E-UTRA Operating Band | UL operating band UE transmit, BS receive | DL operating band UE receive, BS transmit |
| --- | --- | --- | --- |
| I | 1 | 1920 – 1980 MHz | 2110 – 2170 MHz |
| II | 2 | 1850 –1910 MHz | 1930 – 1990 MHz |
| III | 3 | 1710 – 1785 MHz | 1805 – 1880 MHz |
| IV | 4 | 1710 – 1755 MHz | 2110 – 2155 MHz |
| V | 5 | 824 – 849 MHz | 869 – 894 MHz |
| VI | 6 Note 1 | 830 – 840 MHz | 875 – 885 MHz |
| VII | 7 | 2500 – 2570 MHz | 2620 – 2690 MHz |
| VIII | 8 | 880 – 915 MHz | 925 – 960 MHz |
| IX | 9 | 1749.9 – 1784.9 MHz | 1844.9 – 1879.9 MHz |
| X | 10 | 1710 – 1770 MHz | 2110 – 2170 MHz |
| XI | 11 | 1427.9 - 1447.9 MHz | 1475.9 - 1495.9 MHz |
| XII | 12 | 699 – 716 MHz | 729 – 746 MHz |
| XIII | 13 | 777 – 787 MHz | 746 – 756 MHz |
| XIV | 14 | 788 – 798 MHz | 758 – 768 MHz |
| XV | 15 | Reserved | Reserved |
| XVI | 16 | Reserved | Reserved |
| XVII Note 2 | 17 | 704 – 716 MHz | 734 – 746 MHz |
| XVIII Note 2 | 18 | 815 – 830 MHz | 860 – 875 MHz |
| XIX | 19 | 830 – 845 MHz | 875 – 890 MHz |
| XX | 20 | 832 - 862 MHz | 791 - 821 MHz |
| XXI | 21 | 1447.9 - 1462.9 MHz | 1495.9 - 1510.9 MHz |
| XXII | 22 | 3410 – 3490 MHz | 3510 – 3590 MHz |
| – | 23 | 2000 – 2020 MHz | 2180 – 2200 MHz |
| – | 24 | 1626.5 – 1660.5 MHz | 1525 – 1559 MHz |
| XXV | 25 | 1850 – 1915 MHz | 1930 – 1995 MHz |
| XXVI | 26 | 814 – 849 MHz | 859 – 894 MHz |
| – | 27 | 807 – 824 MHz | 852 – 869 MHz |
| – | 28 | 703 – 748 MHz | 758 – 803 MHz |
| – | 29 Note 3 | N/A | 717 – 728 MHz |
| – | 30 | 2305 – 2315 MHz | 2350 – 2360 MHz |
| – | 31 | 452.5 – 457.5 MHz | 462.5 – 467.5 MHz |
| XXXII Note 4 | 32 Note 3 | N/A | 1452-1496 MHz |
| a) | 33 | 1900 – 1920 MHz | 1900 – 1920 MHz |
| a) | 34 | 2010 – 2025 MHz | 2010 – 2025 MHz |
| b) | 35 | 1850 – 1910 MHz | 1850 – 1910 MHz |
| b) | 36 | 1930 – 1990 MHz | 1930 – 1990 MHz |
| c) | 37 | 1910 – 1930 MHz | 1910 – 1930 MHz |
| d) | 38 | 2570 – 2620 MHz | 2570 – 2620 MHz |
| f) | 39 | 1880 – 1920 MHz | 1880 – 1920 MHz |
| e) | 40 | 2300 – 2400 MHz | 2300 – 2400 MHz |
| - | 41 | 2496 – 2690 MHz | 2496 – 2690 MHz |
| - | 42 | 3400 – 3600 MHz | 3400 – 3600 MHz |
| - | 43 | 3600 – 3800 MHz | 3600 – 3800 MHz |
| - | 44 | 703 – 803 MHz | 703 – 803 MHz |
| - | 45 | 1447 – 1467 MHz | 1447 – 1467 MHz |
| - | 46 Note 6 | 5150 – 5925 MHz | 5150 – 5925 MHz |
| - | 48 | 3550 – 3700 MHz | 3550 – 3700 MHz |
| - | 65 | 1920 – 2010 MHz | 2110 – 2200 MHz |
| - | 66 Note 5 | 1710 – 1780 MHz | 2110 – 2200 MHz |
| - | 67 Note 3 | N/A | 738 – 758 MHz |
| – | 68 | 698 – 728 MHz | 753 – 783 MHz |
| – | 69 Note 3 | N/A | 2570 MHz – 2620 MHz |
| - | 70 Note 7 | 1695 – 1710 MHz | 1995 – 2020 MHz |
|  | 74 | 1427 – 1470 MHz | 1475 – 1518 MHz |
| Note 1: Band 6 is not applicable. Note 2: UTRA Operating Band XVII and XVIII are Reserved. Note 3: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. Note 4: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. Note 5: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. Note 6: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. Note 7: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. |  |  |  |

The operating bands of the BS modem or antenna modem shall be declared by the manufacturer.

<Unchanged sections omitted>