3GPP TSG-RAN3 Meeting #97	R3-173115

Berlin, Germany, Aug 21-25, 2017

Agenda Item:	10.10.2.5

Source: 	Huawei

Title:	TP of fast retransmission of lost PDUs to 38.401

Document for:	pCR

Introduction

This paper provides text proposal of fast retransmission of lost PDUs as discussed in [1].

Text Proposal

## 8.3	Mechanism of centralized retransmission of lost PDUs

Editor’s Note: whether this section could be captured in the same paragraph of mobility procedures is FFS.

This mechanism allows to perform the retransmission of the PDUs that were not delivered by a gNB-DU (gNB-DU1), e.g., because the radio link served by gNB-DU1 is subject to temporary outage, via another gNB-DU (gNB-DU2) with a well-functioning radio link toward the UE.

Dual/multi connectivity between gNB-DUs within a gNB-CU and across gNB-CUs enables fast retransmissions. For example, based on flow control information provided by gNB-DU1, the gNB-CU may forward to gNB-DU2 the incoming data traffic for the UE as well as PDCP PDUs with SNs potentially indicated by gNB-DU1, or the gNB-CU may decide to start sending again data traffic through gNB-DU1, or the gNB-CU may decide to start sending the duplicated data to both gNB-DU1 and gNB-DU2.

Editor’s Note: whether new notification mechanism in flow control is needed is FFS.

References

R3-173114,	Fast retransmission of lost PDUs, Huawei.