SMALL CELL FORUM Ltd
PO Box 23
Dursley
GL11 5WA   UK

t  +44 (0)845 6445823 
f  +44 (0)1453 549254 
e  info@smallcellforum.org 
w www.smallcellforum.org

Date: 	7 August 2017

From: 	Small Cell Forum

Contact: 	Richard Kennedy (richard@smallcellforum.org) SCF COO

Mark Grayson (mgrayson@cisco.com) SCF TECH Co-Chair

Prabhakar Chitrapu (pc5542@att.com) SCF TECH Co-Chair

Neeraj Gupta (Neeraj.Gupta@emea.nec.com) SCF TECH Co-Chair

To: 	3GPP TSG RAN-WG3 (RAN3) (3gppliaison@etsi.org)

Liaison Subject: gNB-DU management function

1. Introduction

Small Cell Forum (SCF) has previously liaised with RAN3 concerning its work program to define of the internal LTE RAN interface in R3-161601. In R3-16160 the SCF described its small cell virtualization program, including the publication of its nFAPI interface definition(http://scf.io/documents/082) aimed at facilitating the deployment of multi-vendor virtualized small cell infrastructure.

Specifically, the VNF/PNF split, is supported over 8 separate interfaces, labeled P2 through to P9, as illustrated in Figure 1 below. SCF082 then includes details of Small Cell Forum nFAPI definition for the supporting the P4, P5 and P7 interfaces.

Figure 1: nFAPI Interfaces

In February 2017, the Small Cell Forum completed its initial nFAPI work program with the publication of the P9 PNF Management Model (http://scf.io/documents/167 and http://scf.io/en/nfapi/nfapi-1-0-0.php). This defines an open and extensible TR-069-based management object for the PNF (Distributed Unit).

2. Multi-Vendor Rationale

One of the key rationale’s that led the SCF to drive the development of a multi-vendor CU/DU split is the opportunity to leverage such to enable the deployment of shared DU functionality. Importantly, from a use case scenario, nFAPI has been designed to enable an independent neutral host provider to manage the Distributed Unit and partition/slice resources between multiple operators.

Core Operator B

Core Operator C

Operator B

gNB-CU

Operator C

gNB-CU

Operator B gNB-DU

Operator B gNB-DU

Shared  gNB-DU

Shared  gNB-DU

Operator C gNB-DU

Operator C gNB-DU

Figure 2: New approach to shared RAN enabled by multi-vendor CU/DU split

3. gNB-DU Management Implications

From an nFAPI perspective, the P9 OAM service plays a crucial role in enabling the sharing of the PNF(DU) system across multiple operators. SCF069 (http://scf.io/documents/069) describes how these functions are broadly similar to the OAM system of a traditional DAS. However additional degrees of complexity come from the sharing of PHY baseband processing resources based on nFAPI interface, which is typically not applicable to DAS deployments based on RF interface.

- The opportunity to use virtualization to provide new capabilities to support multi-operator deployments, including shared PNFs that are connected to multiple VNF instances.

Next Meetings:

Small Cell Forum Plenary and Board Meeting: [12] September 2016, Rome, Italy

Small Cell Forum Plenary and Board Meeting: [02] November 2016, Dallas USA

Figure 3: Multi-Operator Shared nFAPI architecture

4. Request to RAN3

In their 5G Era whitepaper, the GSMA survey of operator CEOs identify network sharing as an increasing trend in 5G infrastructure deployments. Hence, the same drivers that have led to the definition by the Small Cell Forum of a split EUTRAN architecture that supports shared nFAPI PNF, will be evident in 5G.

SCF requests that RAN3 considers the enhanced deployment scenarios possible if the 5G Access architecture is able to facilitate the sharing of the gNB-DU between multiple operators.

Importantly, the definition of how the gNB-DU is managed will have a critical impact on the ability of a neutral host provider to manage and partition a shared gNB-DU.

SCF understands that RAN3 is discussing options of how the gNB-DU will be managed and in particular whether the gNB-DU is managed by O&M or the gNB-CU. The earlier SCF analysis indicates that having the gNB-DU managed by O&M can facilitate the deployment of new sharing options.

SCF requests that RAN3 considers the above when deciding on how the gNB-DU will be managed.

Next Meetings:

Small Cell Forum Plenary and Board Meeting: 25-26 September 2017, Beijing, China

Small Cell Forum Plenary and Board Meeting: 4-6 December 2017, San Jose, USA