3GPP TSG-RAN WG3 Meeting #97 	R3-173315

# Berlin, Germany, 21st – 25th August 2017

Agenda item:	10.8

Source:	NTT DOCOMO, INC.

Title:	Flow control: Further consideration on potential issues and possible enhancements

Document for:	Discussion and Decision

1	Introduction

In RAN3 NR AH#2, many proposals for enhancing X2-UP flow control for LTE-NR dual connectivity were contributed [1-6]. And due to time limitation, only some of proposals were discussed well and following principle was agreed [7].

“Same FC mechanism and procedures in F1, X2, Xn; possible enhancements for F1-U are not precluded”

So, further detailed discussions on these flow control enhancements are expected in this RAN3#97 meeting. In this contribution, potential issues of LTE dual connectivity flow control and possible enhancements to resolve those issues are explained reviewing proposals in RAN3 NR AH#2 by updating [1].

NOTE: This contribution is submitted to Agenda Item 10.8 as it relates to all LTE-NR DC, but this contribution is also applicable to F1 and in that sense would also belong to AI 10.10.1based on the above agreement.

NOTE: following terminology is assumed in this contribution considering last RAN3 discussion

- Splitting node: MN on MCG split bearer and SN on SCG split bearer

- Anchor node: SN on MCG split bearer and MN on SCG split bearer

2	Discussion

  - Issues and enhancements in previous contribution [1]

Following issues and enhancements were proposed in [1]

“Proposal 1: RAN3 is requested to confirm the following potential issues with respect to LTE DC flow control, for which it might be beneficial to consider possible enhancements for:

Issue 1-1: DDDS feedback timing is up to SeNB’s implementation.

Issue 1-2: Updating frequency of Acked PDCP PDUs is limited by RLC-ACK reception rate from UE

Issue 2: Delay until first flow control packet

Issue 3: Small packets

Issue 4: Definition of “Desired buffer size” is ambiguous

Proposal 2:  Provided that RAN3 confirms the potential issues (Issues 1-4), RAN3 is requested to discuss possible enhancements for flow control including the followings:

Enhancement 1: Defining triggers for flow control feedback (addresses Issue 1-1)

Enhancement 2: Define DDDS based on Transmitted PDCP PDUs (addresses Issue 1-2)

Enhancement 3: Define information exchange for “achievable throughput” (or parameters to derive it) over C-plane messages (addresses Issues 2&3)”

However, reviewing proposals in RAN3 NR AH#2, we see further potential issues to be addressed. In next section, potential issues are listed.

NOTE: “DDDS” is “DL DATA DELIVERY STATUS” defined in TS 36.425.

  - Potential issues of LTE dual connectivity flow control

LTE dual connectivity defines DDDS, which contains Acked PDCP PDUs and Desired buffer size for the bearer / UE, as the feedback information for X2-UP flow control [8]. However, there are the potential issues shown in table 1 to the LTE dual connectivity flow control mechanism, which may limit performance and therefore might be beneficial to consider enhancements:

Table 1 Potential issues of LTE dual connectivity flow control

| Issue Number | Issue | Detail | Related Tdocs |
| --- | --- | --- | --- |
| 1 | Feedback timing of DDDS is left up to implementation of anchor node | Both may lead to sub-optimal flow control tracking during active data transmissions, especially when throughput variations are large (e.g. due to channel quality and/or cell traffic dynamicity). | [1-6] |
| 2 | Updating frequency of “Highest successfully delivered PDCP Sequence Number” in DDDS is limited by RLC-ACKing rate |  | [1], [6] |
| 3 | Feedback of first DDDS at (re)start of data transmissions might be too slow | With DDDS feedback using Acked PDCP PDUs, it could take time until the first flow control packet is received. This may lead to sub-optimal flow control at the (re)start of data transmissions. | [1], [6] |
| 4 | “Highest successfully delivered PDCP Sequence Number” in DDDS is not enough to estimate “achievable throughput” when packet sizes are small | With DDDS feedback using Acked PDCP PDUs, achievable throughput cannot be obtained when packet sizes of the data transferred in the past are small (throughput estimation will be limited by packet size). | [1] |
| 5 | “Desired buffer size” in DDDS is not clearly defined | The definition of “desired buffer size” is ambiguous. | [1] |
| 6 | DDDS currently (i.e. LTE DC baseline) is not defined to support SCG split bearer | Currently specification only defines that DDDS is transferred from MN to SN. | [2] |

Proposal 1: RAN3 is requested to confirm the following potential issues with respect to the LTE DC flow control baseline, for which it might be beneficial to consider possible enhancements for:

- Issue 1: Feedback timing of DDDS is left up to implementation.

- Issue 2: Updating frequency of “Highest successfully delivered PDCP Sequence Number” in DDDS is limited by RLC-ACKing rate

- Issue 3: Feedback of first DDDS at (re)start of data transmissions might be too slow

- Issue 4: “Highest successfully delivered PDCP Sequence Number” in DDDS is not enough to estimate “achievable throughput” when packet sizes are small

- Issue 5: “Desired buffer size” in DDDS is not clearly defined

- Issue 6: DDDS currently (i.e. LTE DC baseline) is not defined to support SCG split bearer

  - Possible enhancements for flow control

Possible enhancements are described below for the Issues 1-6 mentioned in the previous section.

Table 2 Possible enhancements to LTE dual connectivity flow control

| Issue Number | Issue | Possible enhancements | Detail |
| --- | --- | --- | --- |
| 1 | Feedback timing of DDDS is left up to implementation of anchor node | Enhancement 1-a: Define periodic timer as trigger for DDDS. | A possible solution is to define the trigger of transferring DDDS. Some trigger for regular DDDS feedback would be beneficial. Candidates are polling trigger and periodic timer trigger. However, periodic timer trigger would be preferable considering processing and interface overheads (since it doesn’t require polling for each DDDS), especially for regular DDDS feedback triggering. |
|  |  | Enhancement 1-b: Also define polling as trigger for DDDS. | Periodic timer can be used for regular DDDS feedback triggering. However, it may be beneficial to also support polling trigger to allow the receiver of DDDS to request DDDS feedback based on the receiver’s need (e.g. for fail safe, X2 delay estimation.) |
|  |  | Enhancement 1-c: Define a mechanism to avoid DDDS feedback when there is no change in DDDS contents. | Defining periodic timer as trigger for DDDS is beneficial for regular DDDS feedback triggering. However, there could be periods of no new data transfer. i.e. DDDS contents is not be updated, due to e.g. empty buffer or scheduling delay due to high traffic load. At these times, DDDS feedback with same content may be triggered many times, which unnecessarily increases processing and interface overheads. Therefore, it would be beneficial to reduce unnecessary DDDS feedback. To achieve that, it would be better to introduce a mechanism to stop sending DDDS during the period that DDDS content is not changed. |
| 2 | Updating frequency of “Highest successfully delivered PDCP Sequence Number” in DDDS is limited by RLC-ACKing rate | Enhancement 2: Add “Highest transmitted PDCP Sequence Number” IE to DDDS. | A possible solution is to define DDDS based on Transmitted data amount (e.g. “Highest transmitted PDCP Sequence Number”) instead of Acked data amount (i.e. “Highest successfully delivered PDCP Sequence Number”). This will remove the limitation from RLC-ACKing rate and allow for faster tracking based on achievable throughput. This may be efficient for full buffer type of traffic. |
| 3 | Feedback of first DDDS at (re)start of data transmissions might be too slow | Enhancement 3: Define information exchange for “achievable throughput” over both C-plane and U-plane messages | A possible solution is to report “achievable throughput” from the anchor node to the splitting node. This will resolve Issue 3 and issue 4 as throughput estimation is no longer based on past data transmissions. For the start of data transmission, “achievable throughput” should be included in the relevant C-plane messages (e.g. SN addition request (for SCG split) and acknowledge (for MCG split) over X2/Xn; Beaerer setup request over F1). For the restart of data transmission, “achievable throughput” should be included in DDDS so that it can be transferred immediately at the re-start of data transmission and sudden change of packet sizes. |
| 4 | “Highest successfully delivered PDCP Sequence Number” in DDDS is not enough to estimate “achievable throughput” when packet sizes are small |  |  |
| 5 | “Desired buffer size” in DDDS is not clearly defined | Enhancement 4: At least clarify how “buffer full status” is indicated by “desired buffer size” | It is necessary for splitting node to at least identify “buffer full status” of anchor node to avoid buffer overflow (basic function of flow control). It should at least be defined that “desired buffer size = ‘0’” means “buffer full.” |
| 6 | DDDS currently (i.e. LTE DC baseline) is not defined to support SCG split bearer | Enhancement 5: Define transfer of DDDS also from MN to SN | Current specification [8] assumes DDDS transfer from SN to MN only. Considering SCG split bearer, DDDS should also be transferred from MN to SN. |

Proposal 2:  Provided that RAN3 confirms the potential issues, RAN3 is requested to discuss possible enhancements for flow control including the followings:

- Enhancement 1-a: Define periodic timer as trigger for DDDS. (addresses Issue 1)

- Enhancement 1-b: Also define polling as trigger for DDDS. (addresses Issue 1)

- Enhancement 1-c: Define a mechanism to avoid DDDS feedback when there is no change in DDDS contents. (addresses Issue 1)

- Enhancement 2: Add “Highest transmitted PDCP Sequence Number” IE to DDDS (addresses Issue 2)

- Enhancement 3: Define information exchange for “achievable throughput” over both C-plane and U-plane messages (addresses Issues 3&4)

- Enhancement 4: At least clarify how “buffer full status” is indicated by “desired buffer size”. (addresses Issue 5)

- Enhancement 5: Define transfer of DDDS also from MN to SN (addresses Issue 6)

3	Summary

In this contribution, potential issues of LTE dual connectivity flow control and possible enhancements to resolve those issues were addressed, and the followings are proposed:

Proposal 1: RAN3 is requested to confirm the following potential issues with respect to the LTE DC flow control baseline, for which it might be beneficial to consider possible enhancements for:

- Issue 1: Feedback timing of DDDS is left up to implementation.

- Issue 2: Updating frequency of “Highest successfully delivered PDCP Sequence Number” in DDDS is limited by RLC-ACKing rate

- Issue 3: Feedback first DDDS at (re)start of data transmissions might be too slow

- Issue 4: “Highest successfully delivered PDCP Sequence Number” in DDDS is not enough to estimate “achievable throughput” when packet sizes are small

- Issue 5: “Desired buffer size” in DDDS is not clearly defined

- Issue 6: DDDS currently (i.e. LTE DC baseline) is not defined to support SCG split bearer

Proposal 2:  Provided that RAN3 confirms the potential issues, RAN3 is requested to discuss possible enhancements for flow control including the followings:

- Enhancement 1: Define periodic timer as trigger for DDDS. (addresses Issue 1)

- Enhancement 1-b: Also define polling as trigger for DDDS. (addresses Issue 1)

- Enhancement 1-c: Define a mechanism to avoid DDDS feedback when there is no change in DDDS contents. (addresses Issue 1)

- Enhancement 2: Add “Highest transmitted PDCP Sequence Number” IE to DDDS (addresses Issue 2)

- Enhancement 3: Define information exchange for “achievable throughput” over both C-plane and U-plane messages (addresses Issues 3&4)

- Enhancement 4: At least clarify how “buffer full status” is indicated by “desired buffer size”. (addresses Issue 5)

- Enhancement 5: Define transfer of DDDS also from MN to SN (addresses Issue 6)

References

- R3-172578, “Flow control: Potential issues and possible enhancements”, NTT DOCOMO, INC.

- R3-172456, “Downlink Flow control for EN-DC”, Huawei

- R3-172458, “Consideration of DL Data Delivery Status triggering”, Huawei

- R3-172296, “Flow control enhancements for X2”, Intel

- R3-172299, “Flow control enhancements for F1”, Intel Corporation, Altiostar Networks

- R3-172096, “Enhancements to the X2 UP”, Nokia, Alcatel-Lucent Shanghai Bell, KDDI, Vodafone, Deutsche Telekom

- RAN3#96 chairman’s note

- TS36.425