3GPP TSG-RAN WG3 Meeting #97	R3-173455

# Berlin, Germany, 21-25 August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1041 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Baseline CR to TS 36.413 covering agreements of RAN3 #97 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, LG Electronics Inc., CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2017-08-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This BL CR contains the endorsed CRs for Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR contains the changes from the following endorsed CRs: R3-173168 Stage-3 of Option 3/3a/3x, Huawei, LG Electronics Inc., CATT, ZTE R3-173323 Support 4T bps in X2AP, Ericsson R3-173378 NR access restriction to eNB, Nokia, Nokia Shanghai Bell, Samsung, HTC R3-173428 Addition of a procedure for RRC tunneling, Nokia, Nokia Shanghai Bell R3-173385 QoS information transfer for SCG-split bearers, Nokia, Nokia Shanghai Bell R3-173382 Stage 3 Change: Addition of Target Node ID for SN Change Procedure, LG Electronics Inc. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSA cannot work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 7, 8.X (new), 9.1.XX (new), 9.1.XY (new), 9.1.YY (new), 9.1.ZZ (new), 9.1.YZ (new), 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | ASN.1 related to R3-173168 (incorporated here) is missing. ASN.1 related to R3-173428 (incorporated here) is missing. ASN.1 related to R3-173385 (incorporated here) is missing. |  |  |  |  |  |  |  |  |

Editor’s note: ASN.1  is not complete yet.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[3]	3GPP TS 36.420: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 General Aspects and Principles".

[4]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[5]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[6]	3GPP TS 32.422: "Telecommunication Management; Subscriber and Equipment Trace; Trace Control and Configuration Management".

[7]	3GPP TS 32.421: "Telecommunication Management; Subscriber and Equipment Trace; Trace concepts and requirements".

[8]	3GPP TS 36.424: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 data transport".

[9]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

[10]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[11]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures ".

[12]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[13]	3GPP TS 23.203: "Policy and charging control architecture".

[14]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System; Stage 3".

[15]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; stage 2".

[16] 	3GPP TS 36.104: "Base Station (BS) radio transmission and reception ".

[17]	Void.

[18]	3GPP TS 33.401: "Security architecture".

[19]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[20]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[21]	3GPP TS 36.422: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 signaling transport".

[22]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Layer 2 - Measurements".

[23]	Void.

[24]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling"

[25]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[26]	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[27]	ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[28]	ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[29]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[30]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[xx]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[yy]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Elementary Procedure: X2AP protocol consists of Elementary Procedures (EPs). An X2AP Elementary Procedure is a unit of interaction between two eNBs. An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success or failure),

-	Class 2: Elementary Procedures without response.

E-RAB: Defined in TS 36.401 [2].

CSG Cell: as defined in TS 36.300 [15].

Dual Connectivity: as defined in TS 36.300 [15].

E-UTRA-NR Dual Connectivity: as defined in TS 37.340 [15].

Hybrid cell: as defined in TS 36.300 [15].

Master eNB: as defined in TS 36.300 [15].

Secondary Cell Group: as defined in TS 36.300 [15].

Secondary eNB: as defined in TS 36.300 [15].

------------------------------------------

Skip to next change

------------------------------------------

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

BBF	Broadband Forum

BL	Bandwidth reduced Low complexity

CCO	Cell Change Order

CE	Coverage Enhancement

CoMP	Coordinated Multi Point

DC	Dual Connectivity

DL	Downlink

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

E-CID	Enhanced Cell-ID (positioning method)

eNB	E-UTRAN NodeB

EN-DC	E-UTRA-NR Dual Connectivity

EP	Elementary Procedure

EPC	Evolved Packet Core

E-RAB	E-UTRAN Radio Access Bearer

E-UTRAN	Evolved UTRAN

GNSS	Global Navigation Satellite System

GUMMEI	Globally Unique MME Identifier

HFN	Hyper Frame Number

IE	Information Element

L-GW	Local GateWay

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MeNB	Master eNB

MME	Mobility Management Entity

NAICS	Network-Assisted Interference Cancellation and Suppression

NR	New Radio

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

ProSe	Proximity Service

SCG	Secondary Cell Group

S-GW	Serving Gateway

SeNB	Secondary eNB

SgNB	Secondary gNB

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SN	Sequence Number

TAC	Tracking Area Code

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything

------------------------------------------

Skip to next change

------------------------------------------

7	Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Dual Connectivity. This function allows the eNB to request another eNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the Enb for setup the X2 interface and implicitly perform an X2 Reset.

-	Enb Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the Enb to coordinate adaptation of mobility parameter settings with a peer Enb.

-	Mobility Robustness Optimisation. This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

-	X2 Release. This function allows an Enb to be aware that the signalling connection to a peer Enb is unavailable.

-	Message Transfer. This function allows indirect transport of X2AP messages to a peer Enb.

-	Registration. This function allows registration of Enb in case indirect transport of X2AP messages is supported.

-	Removing the X2. This function allows removing the signaling connection between two eNBs in a controlled manner.

-	Inter-Enb UE Context Resume. This function allows retrieval of a suspended UE context.

The mapping between the above functions and X2 Eps is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP Eps

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change e) MeNB initiated SgNB Release f) SgNB initiated SgNB Release g) SgNB Counter Check h) RRC transfer |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Resume | Retrieve UE Context |

------------------------------------------

Skip to next change

------------------------------------------

8	X2AP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 Eps.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |

------------------------------------------

Skip to next change

------------------------------------------

8.X	Procedures for E-UTRAN-NR Dual Connectivity

8.X.1	SgNB Addition Preparation

8.X.1.1	General

The purpose of the SgNB Addition Preparation procedure is to request the SgNB to allocate resources for EN-DC connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

8.X.1.2	Successful Operation

Figure 8.X.1.2-1: SgNB Addition Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB ADDITION REQUEST message to the SgNB. When the MeNB sends the SGNB ADDITION REQUEST message, it shall start the timer TDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the SGNB ADDITION REQUEST message contains the Serving PLMN IE, the SgNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Expected UE Behaviour IE, the SgNB shall, if supported, store this information and may use it to optimize resource allocation.

The SgNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested E-RABs in the following way:

-	A list of E-RABs which are successfully established shall be included in the E-RABs Admitted To Be Added List IE.

-	A list of E-RABs which failed to be established shall be included in the E-RABs Not Admitted List IE.

NOTE:	The MeNB may trigger the SgNB Addition Preparation procedure in the course of the Inter-MeNB handover without SgNB change procedure as described in 36.300 [15]. The deleted E-RABs are not included in the E-RABs To Be Added List IE in the SGNB ADDITION REQUEST message, from MeNB point of view. If the SgNB reports a certain E-RAB to be successfully established, respective SCG resources, from an SgNB point of view, may be actually successfully established or modified or kept; if a certain E-RAB is reported to be failed to be established, respective SCG resources, from an SgNB point of view, may be actually failed to be established or modified or kept.

For each E-RAB configured with the SCG bearer and SCG split bearer option

-	the SgNB shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

-	the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To be Added Item IE of the SGNB ADDITION REQUEST message. For each E-RAB that it has decided to admit, the SgNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding DL GTP TEID IE in the E-RAB To Be Modified List IE of the E-RAB MODIFICATION INDICATION message (see TS 36.413 [4]) depending on implementation choice.

-	the SgNB may include for each bearer in the E-RABs Admitted To Be Added List IE the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

If the SGNB ADDITION REQUEST message contains for an E-RAB to be added which is configured with SCG split bearer option the MeNB GTP Tunnel Endpoint IE the SgNB shall use it as the DL X2-U address.

For an E-RAB to be added which is configured with SCG split bearer option the SgNB may include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint IE.

Upon reception of the SGNB ADDITION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep.

If the GW Transport Layer Address IE is received in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the MeNB stores this information and use it according to TS 36.300 [15].

If the SgNB UE X2AP ID IE and/or SgNB UE X2AP ID Extension IE are contained in the SGNB ADDITION REQUEST message, the SgNB shall, if supported, store this information and use it as defined in TS 36.300 [15].

If the Tunnel Information for BBF IE is received in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the MeNB shall, if supported, transfer the tunnel information for BBF to the core network.

# Interactions with the SgNB Reconfiguration Completion procedure:

If the SgNB admits at least one E-RAB, the SgNB shall start the timer TDCoverall when sending the SGNB ADDITION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SGNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: SgNB Addition Preparation, unsuccessful operation

If the SgNB is not able to accept any of the bearers or a failure occurs during the SgNB Addition Preparation, the SgNB sends the SGNB ADDITION REQUEST REJECT message with an appropriate cause value to the MeNB.

8.X.1.4	Abnormal Conditions

If the SgNB receives a SGNB ADDITION REQUEST message containing multiple E-RAB ID IEs (in the E-RABs To Be Added List IE) set to the same value, the SgNB shall consider the establishment of the corresponding E-RAB as failed.

If the SgNB receives a SGNB ADDITION REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the SgNB shall consider the establishment of the corresponding E-RAB as failed.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the SgNB (TS 33.401 [18]), the SgNB shall reject the procedure using the SENB ADDITION REQUEST REJECT message.

If the SgNB receives a SGNB ADDITION REQUEST message containing a SgNB UE X2AP ID IE that does not match any existing UE Context that has such ID, the SgNB shall reject the procedure using the SGNB ADDITION REQUEST REJECT message.

# Interactions with the SgNB Reconfiguration Completion and SgNB initiated SgNB Release procedure:

If the timer TDCoverall expires before the SgNB has received the SGNB RECONFIGURATION COMPLETE or the SGNB RELEASE REQUEST message, the SgNB shall regard the requested RRC connection reconfiguration as being not applied by the UE and shall trigger the SgNB initiated SgNB Release procedure.

# Interactions with the MeNB initiated SgNB Release procedure:

If the timer TDCprep expires before the MeNB has received the SGNB ADDITION REQUEST ACKNOWLEDGE message, the MeNB shall regard the SgNB Addition Preparation procedure as being failed and shall trigger the MeNB initiated SgNB Release procedure.

8.X.2	SgNB Reconfiguration Completion

8.X.2.1	General

The purpose of the SgNB Reconfiguration Completion procedure is to provide information to the SgNB whether the requested configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

8.X.2.2	Successful Operation

Figure 8.X.2.2-1: SgNB Reconfiguration Complete procedure, successful operation.

The MeNB initiates the procedure by sending the SGNB RECONFIGURATION COMPLETE message to the SgNB.

The SGNB RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the SgNB. The MeNB may also provide configuration information in the MeNB to SgNB Container IE.

-	or the MeNB has not triggered configuration requested by the SgNB. The MeNB shall provide information with sufficient precision in the included Cause IE to enable the SgNB to know the reason for an unsuccessful reconfiguration. The MeNB may also provide configuration information in the MeNB to SgNB Container IE.

Upon reception of the SGNB RECONFIGURATION COMPLETE message the SgNB shall stop the timer TDCoverall.

8.X.2.3	Abnormal Conditions

Void.

8.X.3	MeNB initiated SgNB Modification Preparation

8.X.3.1	General

This procedure is used to enable an MeNB to request an SgNB to modify the UE context at the SgNB.

The procedure uses UE-associated signalling.

8.X.3.2	Successful Operation

Figure 8.X.3.2-1: MeNB initiated SgNB Modification Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB MODIFICATION REQUEST message to the SgNB. When the MeNB sends the SGNB MODIFICATION REQUEST message, it shall start the timer TDCprep.

The SGNB MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	E-RABs to be added within the E-RABs To Be Added Item IE;

-	E-RABs to be modified within the E-RABs To Be Modified Item IE;

-	E-RABs to be released within the E-RABs To Be Released Item IE;

-	the SgNB UE Aggregate Maximum Bit Rate IE;

-	the MeNB to SgNB Container IE;

-	the SCG Change Indication IE;

If the SGNB MODIFICATION REQUEST message contains the Serving PLMN IE, the SgNB may use it for RRM purposes.

If the SgNB UE Aggregate Maximum Bit Rate IE is included in the SGNB MODIFICATION REQUEST message, the SgNB shall:

-	replace the previously provided SgNB UE Aggregate Maximum Bit Rate by the received SgNB UE Aggregate Maximum Bit Rate in the UE context;

-	use the received SgNB UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE as defined in TS 36.300 [15].

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If at least one of the requested modifications is admitted by the SgNB, the SgNB shall modify the related part of the UE context accordingly and send the SGNB MODIFICATION REQUEST ACKNOWLEDGE message back to the MeNB.

The SgNB shall include the E-RABs for which resources have been either added or modified or released at the SgNB either in the E-RABs Admitted To Be Added List IE or the E-RABs Admitted To Be Modified List IE or the E-RABs Admitted To Be Released List IE. The SgNB shall include the E-RABs that have not been admitted in the E-RABs Not Admitted List IE with an appropriate cause value.

For each E-RAB configured with the SCG bearer and SCG split bearer option

-	the SgNB shall, if included, choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

-	if applicable, the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To Be Added Item IE of the SGNB MODIFICATION REQUEST message. For each E-RAB that it has decided to admit, the SgNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB MODIFICATION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. The MeNB may also provide for an applicable E-RAB to be released the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB MODIFICATION REQUEST message.

-	if applicable, the SgNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

For each E-RAB configured with the split bearer option to be modified, if the SGNB MODIFICATION REQUEST message includes the SCG Change Indication IE and the MeNB GTP Tunnel Endpoint IE in the E-RABs To Be Modified Item IE, the SgNB shall act as specified in TS 37.340 [15].

For each E-RAB configured with the split bearer option to be modified (released)

-	if applicable, the MeNB may provide for an applicable E-RAB to be released the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB MODIFICATION REQUEST message.

If the E-RAB level QoS parameter IE is included in the SGNB MODIFICATION REQUEST message for an E-RAB to be modified the SgNB shall allocate respective resources and provide corresponding radio configuration information within the SgNB to MeNB Container IE as described in TS 36.300 [15].

If the SGNB MODIFICATION REQUEST message contains for an E-RAB to be modified which is configured with the SCG bearer and SCG split bearer option the S1 UL GTP Tunnel Endpoint IE the SgNB shall use it as the new UL S1-U address.

If the SGNB MODIFICATION REQUEST message contains for an E-RAB to be modified which is configured with the split bearer option the MeNB GTP Tunnel Endpoint IE the SgNB shall use it as the new UL X2-U address.

If the SGNB MODIFICATION REQUEST message contains for an E-RAB to be modified which is configured with SCG split bearer option the MeNB GTP Tunnel Endpoint IE the SgNB shall use it as the DL X2-U address.

For an E-RAB to be modified which is configured with the SCG bearer and SCG split bearer option the SgNB may include in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint IE.

For an E-RAB to be modified which is configured with the split bearer option the SgNB may include in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the SgNB GTP Tunnel Endpoint IE.

If the SCG Change Indication IE is included in the SGNB MODIFICATION REQUEST message, the SgNB shall act as specified in TS 37.340 [15].

Upon reception of the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep. If the SGNB MODIFICATION REQUEST ACKNOWLEDGE message has included the SgNB to MeNB Container IE the MeNB is then defined to have a Prepared SgNB Modification for that X2 UE-associated signalling.

# Interactions with the SgNB Reconfiguration Completion procedure:

If the SgNB admits a modification of the UE context requiring the MeNB to report about the success of the RRC connection reconfiguration procedure, the SgNB shall start the timer TDCoverall when sending the SGNB MODIFICATION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SgNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

8.X.3.3	Unsuccessful Operation

Figure 8.X.3.3-1: MeNB initiated SgNB Modification Preparation, unsuccessful operation

If the SgNB does not admit any modification requested by the MeNB, or a failure occurs during the MeNB initiated SgNB Modfication Preparation, the SgNB shall send the SGNB MODIFICATION REQUEST REJECT message to the MeNB. The message shall contain the Cause IE with an appropriate value.

If the SgNB receives a SGNB MODIFICATION REQUEST message containing the MeNB to SgNB Container IE that does not include required information as specified in TS 36.331 [9], the SgNB shall send the SGNB MODIFICATION REQUEST REJECT message to the MeNB.

8.X.3.4	Abnormal Conditions

If the SgNB receives a SGNB MODIFICATION REQUEST message containing multiple E-RAB ID IEs (in the E-RABs To Be Added List IE and/or the E-RABs To Be Modified List IE) set to the same value, the SgNB shall not admit the action requested for the corresponding E-RABs.

If the SgNB receives an SGNB MODIFICATION REQUEST message containing multiple E-RAB ID IEs (in the E-RAB To Be Released List IE) set to the same value, the SgNB shall initiate the release of one corresponding E-RAB and ignore the duplication of the instances of the selected corresponding E-RABs.

If the SgNB receives a SGNB MODIFICATION REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the SgNB shall not admit the corresponding E-RAB.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the SgNB (TS 33.401 [18]), the SgNB shall reject the procedure using the SGNB MODIFICATION REQUEST REJECT message.

If the timer TDCprep expires before the MeNB has received the SGNB MODIFICATION REQUEST ACKNOWLEDGE message, the MeNB shall regard the MeNB initiated SgNB Modification Preparation procedure as being failed and shall release the UE Context at the SgNB.

# Interactions with the SgNB Reconfiguration Completion and SgNB initiated SgNB Release procedure:

If the timer TDCoverall expires before the SgNB has received the SGNB RECONFIGURATION COMPLETE or the SGNB RELEASE REQUEST message, the SgNB shall regard the requested modification RRC connection reconfiguration as being not applied by the UE and shall trigger the SgNB initiated SgNB Release procedure.

Interaction with the SgNB initiated SgNB Modification Preparation procedure:

If the MeNB, after having initiated the MeNB initiated SgNB Modification procedure, receives the SGNB MODIFICATION REQUIRED message, the MeNB shall refuse the SgNB initiated SgNB Modification procedure with an appropriate cause value in the Cause IE.

If the MeNB has a Prepared SgNB Modification and receives the SGNB MODIFICATION REQUIRED message, the MeNB shall respond with the SGNB MODIFICATION REFUSE message to the SgNB with an appropriate cause value in the Cause IE.

8.X.4	SgNB initiated SgNB Modification

8.X.4.1	General

This procedure is used by the SgNB to modify the UE context in the SgNB.

The procedure uses UE-associated signalling.

8.X.4.2	Successful Operation

Figure 8.X.4.2-1: SgNB initiated SgNB Modification, successful operation.

The SgNB initiates the procedure by sending the SGNB MODIFICATION REQUIRED message to the MeNB. When the SgNB sends the SGNB MODIFICATION REQUIRED message, it shall start the timer TDCoverall.

The SGNB MODIFICATION REQUIRED message may contain

-	the SgNB to MeNB Container IE.

-	E-RABs to be released within the E-RABs To Be Released Item IE;

-	E-RABs to be modified within the E-RABs To Be Modified Item IE;

-	the SCG Change Indication IE.

If the MeNB is able to perform the change requested by the SgNB, the MeNB shall send the SGNB MODIFICATION CONFIRM message to the SgNB. The SGNB MODIFICATION CONFIRM message may contain the MeNB to SgNB Container IE.

Upon reception of the SGNB MODIFICATION CONFIRM message the SgNB shall stop the timer TDCoverall.

Interaction with the MeNB initiated SgNB Modification Preparation procedure:

If applicable, as specified in TS 37.340 [15], the SgNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released List IE.

If applicable, as specified in TS 37.340 [15], the SgNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the SgNB Security Key IE within the UE Context Information IE.

If the SgNB has initiated the SgNB initiated SgNB Modification procedure with the SGNB MODIFICATION REQUIRED message including the E-RABs To Be Released Item IE, it may receive the SGNB MODIFICATION REQUEST message including the SCG Change Indication IE, upon which the SgNB shall provide respective information in the SgNB to MeNB Container IE within the SGNB MODIFICATION REQUEST ACKNOWLEDGMENT message, as specified in TS 37.340 [15].

8.X.4.3	Unsuccessful Operation

Figure 8.X.4.3-1: SgNB initiated SgNB Modification, unsuccessful operation.

In case the request modification cannot be performed successfully the MeNB shall respond with the SGNB MODIFICATION REFUSE message to the SgNB with an appropriate cause value in the Cause IE.

The MeNB may also provide configuration information in the MeNB to SgNB Container IE.

8.X.4.4	Abnormal Conditions

If the timer TDCoverall expires before the SgNB has received the SGNB MODIFICATION CONFIRM or the SGNB MODIFICATION REFUSE message, the SgNB shall regard the requested modification as failed and may take further actions like triggering the SgNB initiated SgNB Release procedure to release all SgNB resources allocated for the UE.

If the MeNB is aware that the SgNB didn’t receive the latest configuration information concerning the MCG, the MeNB may respond with the SGNB MODIFICATION REFUSE message to the SgNB with an appropriate cause value in the Cause IE.

If the value received in the E-RAB ID IE of any of the E-RABs To Be Released Items IE is not known at the MeNB, the MeNB shall regard the procedure as failed and may take appropriate actions like triggering the MeNB initiated SgNB Release procedure.

Interaction with the MeNB initiated SgNB Modification Preparation procedure:

If the SgNB, after having initiated the SgNB initiated SgNB Modification procedure, receives the SGNB MODIFICATION REQUEST message including other IEs than an applicable SgNB Security Key IE and/or applicable forwarding addresses and/or the SCG Change Indication IE the SgNB shall

-	regard the SgNB initiated SgNB Modification Procedure as being failed,

-	stop the TDCoverall, which was started to supervise the SgNB initiated SgNB Modification procedure,

-	be prepared to receive the SGNB MODIFICATION REFUSE message from the MeNB and

-	continue with the MeNB initiated SgNB Modification Preparation procedure as specified in section 8.X.3.

8.X.5	SgNB Change

8.X.5.1	General

This procedure is used by the SgNB to change to another SgNB.

The procedure uses UE-associated signalling.

8.X.5.2	Successful Operation

Figure 8.X.5.2-1: SgNB Change, successful operation.

The SgNB initiates the procedure by sending the SGNB CHANGE REQUIRED message to the MeNB including the Target SgNB ID Information IE. When the SgNB sends the SGNB CHANGE REQUIRED message, it shall start the timer TDCoverall.

The SGNB CHANGE REQUIRED message may contain

-	the SgNB to MeNB Container IE (the container is FFS).

If the MeNB is able to perform the change requested by the SgNB, the MeNB shall send the SGNB CHANGE CONFIRM message to the SgNB. For each E-RAB configured with the SCG bearer option or SCG split bearer, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer. For each E-RAB configured with the split bearer option, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of downlink packets to be performed for that bearer.

The SgNB may start data forwarding and stop providing user data to the UE and shall stop the timer TDCoverall upon reception of the SGNB CHANGE CONFIRM message.

8.X.5.3	Unsuccessful Operation

Figure 8.X.5.3-1: SgNB Change, unsuccessful operation.

In case the request change cannot be performed successfully the MeNB shall respond with the SGNB CHANGE REFUSE message to the SgNB with an appropriate cause value in the Cause IE.

8.X.5.4	Abnormal Conditions

If the timer TDCoverall expires before the SgNB has received the SGNB CHANGE CONFIRM or the SGNB CHANGE REFUSE message, the SgNB shall regard the requested change as failed and may take further actions like triggering the SgNB initiated SgNB Release procedure to release all SgNB resources allocated for the UE.

If the MeNB is aware that the SgNB didn’t receive the latest configuration information concerning the MCG, the MeNB may respond with the SGNB CHANGE REFUSE message to the SgNB with an appropriate cause value in the Cause IE.

8.X.6	MeNB initiated SgNB Release

8.X.6.1	General

The MeNB initiated SgNB Release procedure is triggered by the MeNB to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

8.X.6.2	Successful Operation

Figure 8.X.6.2-1: MeNB initiated SgNB Release, successful operation

The MeNB initiates the procedure by sending the SGNB RELEASE REQUEST message. Upon reception of the SGNB RELEASE REQUEST message the SgNB shall stop providing user data to the UE. The SgNB UE X2AP ID IE and, if available, the SgNB UE X2AP ID Extension IE shall be included if it has been obtained from the SgNB. The MeNB may provide appropriate information within the Cause IE.

If the bearer context in the SgNB was configured with the SCG bearer and SCG split bearer option, for each SCG bearer and SCG split bearer for which the MeNB requests forwarding of uplink/downlink data, the MeNB includes the UL Forwarding GTP Tunnel Endpoint/ DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB RELEASE REQUEST message to indicate that the SgNB should perform data forwarding of uplink/downlink packets for that SCG bearer and SCG split bearer.

If the bearer context in the SgNB was configured with the split bearer option, for each Split bearer for which the MeNB requests forwarding of downlink data, the MeNB includes the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB RELEASE REQUEST message to indicate that the SgNB should perform data forwarding of downlink packets for that split bearer.

Upon reception of the SGNB RELEASE REQUEST message containing UE Context Kept Indicator IE set to “True”, the SgNB shall, if supported, only initiate the release of the resources related to the UE-associated signalling connection between the MeNB and the SgNB.

8.X.6.3	Unsuccessful Operation

Not applicable.

8.X.6.4	Abnormal Conditions

Should the SGNB RELEASE REQUEST message refer to a context that does not exist, the SgNB shall ignore the message.

When the MeNB has initiated the procedure and did not include the SgNB UE X2AP ID IE the MeNB shall regard the resources for the UE at the SgNB as being fully released.

8.X.7	SgNB initiated SgNB Release

8.X.7.1	General

This procedure is triggered by the SgNB to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

8.X.7.2	Successful Operation

Figure 8.X.7.2-1: SgNB initiated SgNB Release, successful operation.

The SgNB initiates the procedure by sending the SGNB RELEASE REQUIRED message to the MeNB.

Upon reception of the SGNB RELEASE REQUIRED message, the MeNB replies with the SGNB RELEASE CONFIRM message. For each E-RAB configured with the SCG bearer and SCG split bearer option, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer. For each E-RAB configured with the split bearer option, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of downlink packets to be performed for that bearer.

The SgNB may start data forwarding and stop providing user data to the UE upon reception of the SGNB RELEASE CONFIRM message,

8.X.7.3	Unsuccessful Operation

Not applicable.

8.X.7.4	Abnormal Conditions

Void.

8.X.8	SgNB Counter Check

8.X.8.1	General

This procedure is initiated by the SgNB to request the MeNB to execute a counter check procedure to verify the value of the PDCP COUNTs associated with SCG bearers and SCG split bearers established in the SgNB. (FFS, pending on RAN2’s discussion)

The procedure uses UE-associated signalling.

8.X.8.2	Successful Operation

Figure 8.X.8.2-1: SgNB Counter Check procedure, successful operation.

The SgNB initiates the procedure by sending the SGNB COUNTER CHECK REQUEST message to the MeNB.

Upon reception of the SGNB COUNTER CHECK REQUEST message, the MeNB may perform the RRC counter check procedure as defined in TS 33.401 [18].

8.X.8.3	Unsuccessful Operation

Not applicable.

8.X.8.4	Abnormal Conditions

Not applicable.

------------------------------------------

Skip to next change

------------------------------------------

8.X.Y	RRC Transfer

8.X.Y.1	General

The purpose of the RRC Transfer procedure is to deliver an RRC message encapsulated in a PDCP-C PDU to the SgNB so that it may then be forwarded to the UE.

The procedure uses UE-associated signalling.

8.X.Y.2	Successful Operation

Figure 8.X.Y.2-1: RRC Transfer procedure, successful operation.

The MeNB initiates the procedure by sending the RRC TRANSFER message to the SgNB.

8.X.Y.3	Abnormal Conditions

The receiving node may ignore the message, if the sending node has not notified about possibility of RRC transfer at the bearer setup.

------------------------------------------

Skip to next change

------------------------------------------

9.1.X	Messages for E-UTRAN-NR Dual Connectivity Procedures

Editor’s Note: The IEs SCG Split Bearer and QoS are pending on further discussion. FFS.

9.1.X.1	SGNB ADDITION REQUEST

This message is sent by the MeNB to the SgNB to request the preparation of resources for EN-DC operation for a specific UE

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| UE Security Capabilities | C- ifSCGorSCGsplitBearerOption |  | 9.2.29 |  | YES | reject |
| SgNB Security Key | C- ifSCGorSCGsplitBearerOption |  | 9.2.YY | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. | YES | reject |
| SgNB UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.12 | The UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SgNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and SgNB respectively. | YES | reject |
| Serving PLMN | O |  | PLMN Identity 9.2.4 | The serving PLMN of the SCG in the SgNB. | YES | ignore |
| E-RABs To Be Added List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | reject |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters for the SCG part. | – | – |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>> E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB Level QoS parameters as received on S1-MME for the E-RAB | – | – |
| >>>> Maximum admittable E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB Level QoS parameters admittable by the MeNB | – | – |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| MeNB to SgNB Container | M |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 38.331 [xx]. Editor’s Note: to be checked with RAN2 | YES | reject |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| SgNB UE X2AP ID | O |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| Requested MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for MCG Split SRB are requested. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofSCG | Maximum no. of NR SCG. Value is FFS |

| Condition | Explanation |
| --- | --- |
| ifSCGorSCGsplitBearerOption | This IE shall be present if the Bearer Option IE is set to the value "SCG bearer" or "SCG split bearer". |

9.1.X.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the SgNB to confirm the MeNB about the SgNB addition preparation.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>> E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the RRCConnectionReconfiguration message (NR RRC message name is FFS) as defined in TS 38.331[xx]. Editor’s Note: to be checked with RAN2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating GW Transport Layer Address. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.89 | Indicating eNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| Admitted MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.3	SGNB ADDITION REQUEST REJECT

This message is sent by the SgNB to inform the MeNB that the SgNB Addition Preparation has failed.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

9.1.X.4	SGNB RECONFIGURATION COMPLETE

This message is sent by the MeNB to the SgNB to indicate whether the configuration requested by the SgNB was applied by the UE.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  |  |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>MeNB to SgNB Container | M |  | OCTET STRING | Includes the NR RRCConnectionReconfigurationComplete message (NR RRC message name is FFS) as defined in TS 38.331 [xx]. Editor’s Note: to be checked with RAN2 | - | - |
| >>Configuration rejected by the MeNB |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.6 |  | - | - |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

9.1.X.5	SGNB MODIFICATION REQUEST

This message is sent by the MeNB to the SgNB to request the preparation to modify SgNB resources for a specific UE.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SCG Change Indication | O |  | 9.2.73 |  | YES | ignore |
| Serving PLMN | O |  | PLMN Identity 9.2.4 | The serving PLMN of the SCG in the SgNB. | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.29 |  | – | – |
| >SgNB Security Key | O |  | 9.2.YY |  | – | – |
| >SgNB UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.12 |  | – | – |
| >E-RABs To Be Added List |  | 0..1 |  |  | – | – |
| >>E-RABs To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB Level QoS parameters as received on S1-MME for the E-RAB | – | – |
| >>>>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>> Maximum admittable E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes the E-RAB Level QoS parameters admittable by the MeNB | – | – |
| >E-RABs To Be Modified List |  | 0..1 |  |  | – | – |
| >>E-RABs To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes QoS parameters to be modified | – | – |
| >>>>>S1 UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes QoS parameters to be modified | – | – |
| >>>>>MeNB GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB Level QoS parameters as received on S1-MME for the E-RAB | – | – |
| >>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >E-RABs To Be Released List |  | 0..1 |  |  | – | – |
| >>E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| MeNB to SgNB Container | M |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 38.331 [xx] Editor’s Note: to be checked with RAN2 | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.6	SGNB MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the SgNB to confirm the MeNB’s request to modify the SgNB resources for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| E-RABs Admitted List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added List |  | 1 |  |  | – | – |
| >>E-RABs Admitted To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |
| >E-RABs Admitted To Be Modified List |  | 0..1 |  |  | – | – |
| >>E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >E-RABs Admitted To Be Released List |  | 0..1 |  |  | – | – |
| >>E-RABs Admitted To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the NR RRCConnectionReconfiguration message (NR RRC message name is FFS) as defined in TS 38.331 [xx]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.7	SGNB MODIFICATION REQUEST REJECT

This message is sent by the SgNB to inform the MeNB that the MeNB initiated SgNB Modification Preparation has failed.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

9.1.X.8	SGNB MODIFICATION REQUIRED

This message is sent by the SgNB to the MeNB to request the modification of SgNB resources for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SCG Change Indication | O |  | 9.2.73 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>Cause | M |  | 9.2.6 |  | – | – |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the NR RRCConnectionReconfiguration message (NR RRC message name is FFS) as defined in TS 38.331 [xx]. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |
| E-RABs To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.9	SGNB MODIFICATION CONFIRM

This message is sent by the MeNB to inform the SgNB about the successful modification.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the NR RRCConnectionReconfigurationComplete message (NR RRC message name is FFS) as defined in TS 38.331 [xx]. Editor’s Note: to be checked with RAN2 | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.10	SGNB MODIFICATION REFUSE

This message is sent by the MeNB to inform the SgNB that the SgNB initiated SgNB Modification has failed.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

9.1.X.11	SGNB RELEASE REQUEST

This message is sent by the MeNB to the SgNB to request the release of resources.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | O |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | O |  | 9.2.6 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| > E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.12	SGNB RELEASE REQUIRED

This message is sent by the SgNB to request the release of all resources for a specific UE at the SgNB.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

9.1.X.13	SGNB RELEASE CONFIRM

This message is sent by the MeNB to confirm the release of all resources for a specific UE at the SgNB.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| E-RABs to be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | – | – |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.14	SGNB COUNTER CHECK REQUEST

This message is sent by the SgNB to request the verification of the value of the PDCP COUNTs associated with SCG bearers and SCG split bearers established in the SgNB.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| E-RABs Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >E-RABs Subject to Counter Check Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | - | - |
| >>UL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of uplink COUNT associated to this E-RAB. | - | - |
| >>DL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of downlink COUNT associated to this E-RAB. | - | - |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.15	SGNB CHANGE REQUIRED

This message is sent by the SgNB to the MeNB to request the change of SgNB for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB. | YES | reject |
| Target SgNB ID Information | M |  | 9.2.ZZ |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Editor’s note: the container is FFS | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. cells that can be served by an gNB. Value is 256. |

9.1.X.16	SGNB CHANGE CONFRIM

This message is sent by the MeNB to inform the SgNB about the successful change.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| E-RABs to be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | – | – |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.X.17	SGNB CHANGE REFUSE

This message is sent by the MeNB to inform the SgNB that the SgNB initiated SgNB Change has failed.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

9.1.Z.1	RRC TRANSFER

This message is sent by the MeNB to the SgNB to transfer an RRC message.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| RRC Container | M |  | OCTET STRING | RRC message encapsulated in a PDCP-C PDU and ciphered with the key of the MeNB | YES | reject |
| SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type | YES | reject |

------------------------------------------

Skip to next change

------------------------------------------

9.2.3	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.2.4 |  | – | – |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of “equivalent PLMNs list” as defined in TS 24.301 [14]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the serving PLMN and Equivalent PLMNs. | – | – |
| >PLMN Identity | M |  | 9.2.4 |  | – | – |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | intra E-UTRAN roaming restrictions | – | – |
| >PLMN Identity | M |  | 9.2.4 | The PLMN of forbidden TACs | – | – |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | – | – |
| >>TAC | M |  | OCTET STRING(2) | The forbidden TAC | – | – |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | inter-3GPP RAT roaming restrictions | – | – |
| >PLMN Identity | M |  | 9.2.4 |  | – | – |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  | – | – |
| >>LAC | M |  | OCTET STRING(2) |  | – | – |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …,GERAN and UTRAN, CDMA2000 and UTRAN) | inter-3GPP and 3GPP2 RAT access restrictions. “ALL” means that all RATs mentioned in the enumeration of this IE are restricted. | – | – |
| NR restriction | O |  | ENUMERATED(NRrestricted, …) | Restriction to use NR. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMN Ids. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of equivalent PLMN Ids plus one. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbLACs | Maximum no. of forbidden Location Area Codes. Value is 4096. |

------------------------------------------

Skip to next change

------------------------------------------

9.2.10	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR E-RAB for downlink and uplink.

NOTE:	The SeNB regards the GBR QoS Information IE as an E-RAB level parameter also for E-RABs configured with the split bearer option, although for the split bearer option the bitrates signalled by the MeNB are typically not equal to the bitrates signalled by the MME for that E-RAB (see TS 36.300 [15]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Downlink IE is included, the E-RAB Maximum Bit Rate Downlink IE shall be ignored. | – | – |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Uplink IE is included, the E-RAB Maximum Bit Rate Uplink IE shall be ignored. | – | – |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Downlink IE is included, the E-RAB Guaranteed Bit Rate Downlink IE shall be ignored. | – | – |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Uplink IE is included, the E-RAB Guaranteed Bit Rate Uplink IE shall be ignored. | – | – |
| Extended E-RAB Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Guaranteed Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Guaranteed Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – | – |

9.2.11	Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL or by UE in sidelink within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR E-RAB, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0..10,000,000,000) | The unit is: bit/s |

9.2.12	UE Aggregate Maximum Bit Rate

On Handover Aggregate Maximum Bitrate is transferred to the target eNB. In Dual Connectivity, UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SeNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and SeNB respectively as specified in TS 36.300 [15]. The UE Aggregate Maximum Bitrate is applicable for all Non-GBR bearers per UE which is defined for the Downlink and the Uplink direction and provided by the MME to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Aggregate Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Downlink IE is included, the UE Aggregate Maximum Bit Rate Downlink IE shall be ignored. | – | – |
| UE Aggregate Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Uplink IE is included, the UE Aggregate Maximum Bit Rate Uplink IE shall be ignored. | – | – |
| Extended UE Aggregate Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | UE Aggregate Maximum Bit Rate in DL. Details in TS 23.401 [12]. | – | – |
| Extended UE Aggregate Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | UE Aggregate Maximum Bit Rate in UL. Details in TS 23.401 [12]. | – | – |

------------------------------------------

Skip to next change

------------------------------------------

9.2.xx	Extended Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR bearer, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Bit Rate |  |  | INTEGER (10,000,000,001..4,000,000,000,000,...) | The unit is: bit/s |

9.2.xx+1	gNB UE X2AP ID

This information element, combined with the gNB UE X2AP ID Extension when present, uniquely identifies an UE over the X2 interface within an gNB.

The usage of this IE is defined in TS 36.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB UE X2AP ID | M |  | INTEGER (0..4095) |  |

9.2.XY	gNB UE X2AP ID Extension

This information element combined with the gNB UE X2AP ID uniquely identifies an UE over the X2 interface within an gNB. If the setup of an UE associcated signalling connection was initiated including the gNB UE X2AP ID Extension, the gNB UE X2AP ID Extension shall be used by both peers for the life-time of the respective UE-associated signalling connection.

The usage of this IE is defined in TS 36.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB UE X2AP ID Extension | M |  | INTEGER (0..4095,…) |  |

9.2.YY	SgNB Security Key

The SgNB Security Key IE is used to apply security in the SgNB as defined in TS 33.401 [18].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SgNB Security Key | M |  | BIT STRING (SIZE(256)) | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. |

9.2.ZZ	Target SgNB ID Information

This IE contains the target SgNB ID used by MeNB to find the target SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Target SgNB ID | M |  | reference |  |  |  |

------------------------------------------

Skip to next change

------------------------------------------

9.3.5	Information Element definitions

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-NRrestriction,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

------------------------------------------

Skip to next change

------------------------------------------

#### -- E

EARFCN ::= INTEGER (0..maxEARFCN)

EARFCNExtension ::= INTEGER(maxEARFCNPlusOne..newmaxEARFCN, ...)

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTP ::= SEQUENCE {

enhancedRNTPBitmap			BIT STRING (SIZE(12..8800, ...)),

rNTP-High-Power-Threshold	RNTP-Threshold,

enhancedRNTPStartTime		EnhancedRNTPStartTime  OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { EnhancedRNTP-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTP-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTPStartTime ::= SEQUENCE {

startSFN				INTEGER (0..1023, ...),

startSubframeNumber		INTEGER (0..9, ...),

iE-Extensions			ProtocolExtensionContainer { { EnhancedRNTPStartTime-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTPStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID ::= CHOICE {

macro-eNB-ID	BIT STRING (SIZE (20)),

home-eNB-ID		BIT STRING (SIZE (28)),

... ,

short-Macro-eNB-ID		BIT STRING (SIZE(18)),

long-Macro-eNB-ID		BIT STRING (SIZE(21))

}

EncryptionAlgorithms ::= BIT STRING (SIZE (16, ...))

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity

E-RAB-ID ::= INTEGER (0..15, ...)

E-RAB-Level-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-RAB-Level-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

E-RAB-Level-QoS-Parameters-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-List ::= SEQUENCE (SIZE(1.. maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RAB-ItemIEs} }

E-RAB-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RAB-Item	 CRITICALITY ignore 	TYPE E-RAB-Item 	PRESENCE mandatory },

...

}

E-RAB-Item ::= SEQUENCE {

e-RAB-ID				E-RAB-ID,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

E-RAB-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

...

}

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

EUTRANTraceID		::= OCTET STRING (SIZE (8))

EventType ::= ENUMERATED{

change-of-serving-cell,

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedActivity		ExpectedUEActivityBehaviour OPTIONAL,

expectedHOInterval		ExpectedHOInterval			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { ExpectedUEBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod					ExpectedActivityPeriod					OPTIONAL,

expectedIdlePeriod						ExpectedIdlePeriod						OPTIONAL,

sourceofUEActivityBehaviourInformation	SourceOfUEActivityBehaviourInformation	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExtendedULInterferenceOverloadInfo ::= SEQUENCE {

associatedSubframes							BIT STRING (SIZE (5)),

extended-ul-InterferenceOverloadIndication	UL-InterferenceOverloadIndication,

iE-Extensions								ProtocolExtensionContainer { { ExtendedULInterferenceOverloadInfo-ExtIEs} } OPTIONAL,

...

}

ExtendedULInterferenceOverloadInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedBitRate	::= INTEGER (10000000001..4000000000000,...)

------------------------------------------

Skip to next change

------------------------------------------

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for maximum bitrate > 10Gbps --

{ ID id-extended-e-RAB-MaximumBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-MaximumBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

GlobalENB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalENB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTPtunnelEndpoint ::= SEQUENCE {

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEI,

iE-Extensions					ProtocolExtensionContainer { {GTPtunnelEndpoint-ExtIEs} } OPTIONAL,

...

}

GTPtunnelEndpoint-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEI					::= OCTET STRING (SIZE (4))

GUGroupIDList		::= SEQUENCE (SIZE (1..maxPools)) OF GU-Group-ID

GU-Group-ID			::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

mME-Group-ID		MME-Group-ID,

iE-Extensions		ProtocolExtensionContainer { {GU-Group-ID-ExtIEs} } OPTIONAL,

...

}

GU-Group-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

------------------------------------------

Skip to next change

------------------------------------------

#### -- H

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

...,

interRATpingpong

}

HandoverRestrictionList ::= SEQUENCE {

servingPLMN					PLMN-Identity,

equivalentPLMNs				EPLMNs					OPTIONAL,

forbiddenTAs				ForbiddenTAs			OPTIONAL,

forbiddenLAs				ForbiddenLAs			OPTIONAL,

forbiddenInterRATs			ForbiddenInterRATs		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {HandoverRestrictionList-ExtIEs} }	OPTIONAL,

...

}

HandoverRestrictionList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRrestriction		CRITICALITY ignore	EXTENSION NRrestriction	PRESENCE optional},

...

}

HFN ::= INTEGER (0..1048575)

HFNModified ::= INTEGER (0..131071)

HFNforPDCP-SNlength18 ::= INTEGER (0..16383)

HWLoadIndicator ::= SEQUENCE {

dLHWLoadIndicator			LoadIndicator,

uLHWLoadIndicator			LoadIndicator,

iE-Extensions				ProtocolExtensionContainer { { HWLoadIndicator-ExtIEs} } OPTIONAL,

...

}

HWLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- I

Asn1 not modified

#### -- N

Neighbour-Information ::= SEQUENCE (SIZE (0..maxnoofNeighbours)) OF SEQUENCE {

eCGI						ECGI,

pCI					 		PCI,

eARFCN						EARFCN,

iE-Extensions		 ProtocolExtensionContainer { {Neighbour-Information-ExtIEs} } OPTIONAL,

...

}

Neighbour-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NeighbourTAC		CRITICALITY ignore	EXTENSION TAC				PRESENCE optional}|

{ ID id-eARFCNExtension		CRITICALITY reject	EXTENSION EARFCNExtension	PRESENCE optional},

...

}

NextHopChainingCount ::= INTEGER (0..7)

Number-of-Antennaports ::= ENUMERATED {

an1,

an2,

an4,

...

}

NRrestriction ::= ENUMERATED {

NRrestricted,

...

}

#### -- O

------------------------------------------

Skip to next change

------------------------------------------

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEaggregateMaximumBitRateDownlink	BitRate,

uEaggregateMaximumBitRateUplink		BitRate,

iE-Extensions						ProtocolExtensionContainer { {UEAggregate-MaximumBitrate-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-extended-uEaggregateMaximumBitRateDownlink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-uEaggregateMaximumBitRateUplink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

UE-ContextKeptIndicator ::= ENUMERATED {

true,

...

}

------------------------------------------

Skip to next change

------------------------------------------

9.3.7	Constant definitions

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

------------------------------------------

Skip to next change

------------------------------------------

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 186

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 187

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 188

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 189

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 190

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 191

id-NRrestriction															ProtocolIE-ID ::= 192

END