3GPP TSG-RAN WG3 #97-bis	R3-173498

Prague, Czechia, EU, 9 - 13 August 2017

Agenda item:	10.8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	S-RLF handling for MR-DC

Document for:	Decision

# 1	Introduction

During the RAN2 NR AdHoc meeting in Qingdao and the following RAN2 meeting #99, it was decided the SCG RLF report received from the UE shall be forwarded to the SgNB. This is further analysed in [1].

In this document, we are providing the stage-3 TP for X2AP needed for the solution.

# References

- R3-173497, RAN3 #97-bis

# Text proposal for baseline CR for TS 36.423

First change, ommited text not changed

### 8.X.Y	S-RLF Indication

#### 8.X.Y.1	General

The purpose of the S-RLF Indication procedure is to report an SCG RLF to the SgNB and to provide the NR measurements included in the UE report.

The procedure uses UE-associated signalling.

#### 8.X.Y.2	Successful Operation

Figure 8.X.Y.2-1: S-RLF Indication procedure, successful operation.

The MeNB initiates the procedure by sending the S-RLF INDICATION message to the SgNB.

#### 8.X.Y.3	Abnormal Conditions

None.

Next change, ommited text not changed

#### 9.1.Z.1	S-RLF INDICATION

This message is sent by the MeNB to the SgNB to transfer an SCG RLF report.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| RRC Container | M |  | OCTET STRING | NR measurements received from the UE | YES | reject |

Next change, ommited text not changed

# FROM X2AP-CommonDataTypes

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ENBConfigurationUpdate,

ENBConfigurationUpdateAcknowledge,

ENBConfigurationUpdateFailure,

ErrorIndication,

HandoverCancel,

HandoverReport,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

LoadInformation,

PrivateMessage,

ResetRequest,

ResetResponse,

ResourceStatusFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusUpdate,

RLFIndication,

SNStatusTransfer,

UEContextRelease,

X2SetupFailure,

X2SetupRequest,

X2SetupResponse,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

X2Release,

X2APMessageTransfer,

SeNBAdditionRequest,

SeNBAdditionRequestAcknowledge,

SeNBAdditionRequestReject,

SeNBReconfigurationComplete,

SeNBModificationRequest,

SeNBModificationRequestAcknowledge,

SeNBModificationRequestReject,

SeNBModificationRequired,

SeNBModificationConfirm,

SeNBModificationRefuse,

SeNBReleaseRequest,

SeNBReleaseRequired,

SeNBReleaseConfirm,

SeNBCounterCheckRequest,

X2RemovalFailure,

X2RemovalRequest,

X2RemovalResponse,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

SgNBAdditionRequest,

SgNBAdditionRequestAcknowledge,

SgNBAdditionRequestReject,

SgNBReconfigurationComplete,

SgNBModificationRequest,

SgNBModificationRequestAcknowledge,

SgNBModificationRequestReject,

SgNBModificationRequired,

SgNBModificationConfirm,

SgNBModificationRefuse,

SgNBReleaseRequest,

SgNBReleaseRequired,

SgNBReleaseConfirm,

SgNBCounterCheckRequest,

SgNBChangeRequired,

SgNBChangeConfirm,

SgNBChangeRefuse,

RrcTransfer,

SRLFIndication

# FROM X2AP-PDU-Contents

id-cellActivation,

id-eNBConfigurationUpdate,

id-errorIndication,

id-handoverCancel,

id-handoverReport,

id-handoverPreparation,

id-loadIndication,

id-privateMessage,

id-reset,

id-resourceStatusReporting,

id-resourceStatusReportingInitiation,

id-rLFIndication,

id-snStatusTransfer,

id-uEContextRelease,

id-x2Setup,

id-mobilitySettingsChange,

id-x2Release,

id-x2APMessageTransfer,

id-seNBAdditionPreparation,

id-seNBReconfigurationCompletion,

id-meNBinitiatedSeNBModificationPreparation,

id-seNBinitiatedSeNBModification,

id-meNBinitiatedSeNBRelease,

id-seNBinitiatedSeNBRelease,

id-seNBCounterCheck,

id-x2Removal,

id-retrieveUEContext,

id-sgNBAdditionPreparation,

id-sgNBReconfigurationCompletion,

id-meNBinitiatedSgNBModificationPreparation,

id-sgNBinitiatedSgNBModification,

id-meNBinitiatedSgNBRelease,

id-sgNBinitiatedSgNBRelease,

id-sgNBChange,

id-sgNBCounterCheck,

id-rRCTransfer,

id-sRLFIndication

Next change, ommited text not changed

# X2AP-ELEMENTARY-PROCEDURES-CLASS-2 X2AP-ELEMENTARY-PROCEDURE ::= {

snStatusTransfer						|

uEContextRelease						|

handoverCancel							|

errorIndication							|

resourceStatusReporting					|

loadIndication							|

privateMessage							|

rLFIndication							|

handoverReport							|

x2Release								|

x2APMessageTransfer						|

seNBReconfigurationCompletion			|

meNBinitiatedSeNBRelease				|

seNBCounterCheck						|

sgNBReconfigurationCompletion			|

meNBinitiatedSgNBRelease				|

sgNBCounterCheck						|

rRCTransfer								|

sRLFIndication							,

...

Next change, ommited text not changed

rRCTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RrcTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

sRLFIndication	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SRLFIndication

PROCEDURE CODE			id-sRLFIndication

CRITICALITY				reject

}

Next change, ommited text not changed

-- **************************************************************

--

#### -- RRC Transfer

--

-- **************************************************************

# RrcTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{RrcTransfer-IEs}},

...

}

# RrcTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-RRCContainer						CRITICALITY ignore	TYPE RRCContainer				PRESENCE mandatory}|

{ ID id-SRBType								CRITICALITY ignore	TYPE SRBType					PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- S-RLF Indication

--

-- **************************************************************

# SRLFIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SRLFIndiciation-IEs}},

...

}

# SRLFIndiciation-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-RRCContainer						CRITICALITY ignore	TYPE RRCContainer				PRESENCE mandatory},

...

}