3GPP TSG-RAN WG3 Meeting #97-bis	R3-173629

Prague, Czechia, EU, 9 - 13 Oct 2017

Agenda item:	10.8.3.1

Source:	Nokia, Nokia Shanghai Bell, KDDI, Vodafone

Title:	Further enhancements to the flow control

Document for:	Decision

# 1	Introduction

At the RAN3 #97 meeting, it was agreed to enable Downlink Data Delivery Report polling (requesting). The needed changes are provided separately in [1]. However, other proposals made in [2] were not discussed. They are recalled and further considered in [3]. In this paper, we adapt the proposals made there for Xn.

# References

- R3-173507, RAN3 #97-bis

- R3-172857, RAN3 #97

- R3-173628, RAN3 #97-bis

# Text proposal for the draft TS 38.425

The changes below are proposed to be added to the draft TS 38.425.

First change, omitted text not changed

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

Editor’s Note: All the text below is FFS.

The purpose of the Transfer of Downlink User Data procedure is to provide Xn-U specific sequence number information at the transfer of user data carrying a DL PDCP PDU from the gNB hosting the PDCP entity to the corresponding gNB via the Xn-U interface.

An Xn user plane instance making use of the Transfer of Downlink User Data procedure is associated to a single data bearer only. The Transfer of Downlink User Data procedure is invoked whenever user data for that particular data bearer needs to be sent across the Xn-U interface.

The gNB hosting the PDCP entity shall assign consecutive Xn-U sequence numbers to each transferred Xn-U packet.

The corresponding gNB shall detect whether an Xn-U packet was lost and memorise the respective sequence number after it has declared the respective Xn-U packet as being "lost".

The corresponding gNB shall transfer the remaining PDCP PDUs towards the UE and memorise the highest PDCP PDU sequence number of the PDCP PDU that was successfully delivered in sequence towards the UE and the highest PDCP PDU sequence number of the PDCP PDU that contained data and was transferred towards the UE. It shall also memorise the time it took to process the last successfully delivered PDU (the time since the PDU arrived to the assisting eNB until it was sent towards the UE).

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost Xn-U packets assist the gNB hosting the PDCP entity in avoiding PDCP HFN de-synchronisation. If an E-UTRAN deployment decides to not use the Transfer of Downlink User Data procedure, PDCP HFN synchronization should be ensured by other means.

Figure z.4.1.1-1: Successful Transfer of Downlink User Data

#### 5.4.1.2	Unsuccessful operation

Void.

### 5.4.2	Downlink Data Delivery Status

#### 5.4.2.1	Successful operation

Editor’s Note: All the text below is FFS.

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding gNB to the gNB hosting the PDCP entity to allow the gNB hosting the PDCP entity to control the downlink user data flow via the corresponding gNB for the respective data bearer. The corresponding gNB may also transfer uplink user data for the concerned data bearer to the gNB hosting the PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding gNB to the gNB hosting the PDCP entity to allow the gNB hosting the PDCP entity to control the successful delivery of DL control data to the corresponding gNB. In this case the corresponding gNB is always the SgNB and the gNB hosting the PDCP entity is always the MgNB [FFS whether this sentence needs rewording].

When the corresponding gNB decides to trigger the Feedback for Downlink Data Delivery procedure it shall report:

a)	the highest PDCP PDU sequence number successfully delivered in sequence to the UE among those PDCP PDUs received from the gNB hosting the PDCP entity;

b)	the desired buffer size in bytes for the concerned data bearer;

c)	the minimum desired buffer size in bytes for the UE;

d)	the Xn-U packets that were declared as being "lost" by the corresponding gNB and have not yet been reported to the gNB hosting the PDCP entity within the DL DATA DELIVERY STATUS frame.

e)	the highest PDCP PDU sequence number successfully transferred towards the UE;

f)	the time between the receiving from the eNB hosting the PDCP and sending to the UE of the PDCP PDU containing data and reported as the highest successfully delivered in sequence (RLC-AM) or transmitted (RLC-UM) to the UE;

NOTE:	If an E-UTRAN deployment has decided not to use the Transfer of Downlink User Data procedure, d) above is not applicable.

The DL DATA DELIVERY STATUS frame shall also include an indication whether the frame is the last DL status report received in the course of releasing a bearer from the corresponding gNB. When receiving such indication, if applicable, the gNB hosting the PDCP entity considers that no more UL data is to be expected from the corresponding gNB.

The gNB hosting the PDCP entity, when receiving the DL DATA DELIVERY STATUS frame:

-	regards the desired buffer size under b) and c) above as the amount of data desired from the corresponding gNB being declared

-	from the PDCP sequence number reported under a) above within the same frame, as well as from the most recently reported PDCP sequence number(s) of all other data bearers established for the UE;

-	as the momentary desired buffer sizes, independent of buffer sizes indicated in the past.

-	is allowed to remove the buffered PDCP PDUs according to the feedback of successfully delivered PDCP PDUs;

-	decides upon the actions necessary to take for PDCP PDUs reported other than successfully delivered.

After being reported to the gNB hosting the PDCP entity, the corresponding gNB removes the respective PDCP sequence numbers.

Figure z.4.2.1-1: Successful Downlink Data Delivery Status

#### 5.4.2.2	Unsuccessful operation

Void .

Next change, omitted text not changed

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

Editor’s Note: All the text below is FFS.

This frame format is defined to transfer feedback to allow the receiving gNB (i.e. gNB that hosts the PDCP entity) to control the downlink user data flow via the sending gNB (i.e. gNB that does not host the PDCP entity).

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | Spare |  | Final Frame Ind. | Lost Packet Report | 1 |
| Highest successfully delivered PDCP Sequence Number |  |  |  |  |  |  |  | 3 |
| Desired buffer size for the data bearer |  |  |  |  |  |  |  | 4 |
| Minimum desired buffer size for the UE |  |  |  |  |  |  |  | 4 |
| Number of lost Xn-U Sequence Number ranges reported |  |  |  |  |  |  |  | 1 |
| Start of lost Xn-U Sequence Number range |  |  |  |  |  |  |  | 6* (Number of reported lost Xn-U SN ranges) |
| End of lost Xn-U Sequence Number range |  |  |  |  |  |  |  |  |
| Queuing time |  |  |  |  |  |  |  | 1 |
| Highest successfully transferred PDCP Sequence Number |  |  |  |  |  |  |  | 2 |
| Spare extension |  |  |  |  |  |  |  | 1-7 |

Figure z.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

Next change, omitted text not changed

#### 5.5.3.B	Queuing time

Description: This parameter indicates the queuing time (in ms) of the PDU containing data and reported as last successfully delivered in sequence (RLC-AM) or transmitted (RLC-UM) to the UE. The queuing time is the time since the PDU was received from the eNB hosting the PDCP until it was sent towards the UE. The maximum value (255) indicates the time is higher than the 254 ms

Value range: {0..28-1}.

Field length: 1 octet.

#### 5.5.3.C	Highest successfully transferred PDCP Sequence Number

Description: This parameter indicates the X2-U sequence number as assigned by the respective eNB when the length of PDCP SN is less than 16 bits.

Value range: {0..224-1}.

Field length: 2 octets.

#### 5.5.3.D	Highest successfully transferred PDCP Sequence Number Extended

Description: This parameter indicates the X2-U sequence number as assigned by the respective eNB when the length of PDCP SN is 18 bits.

Value range: {0..224-1}.

Field length: 3 octets.

Remaining text not changed