3GPP TSG-RAN3 Meeting # 97bis	R3-173733

# Prague, Czech, Oct 9-13, 2017

Agenda Item:	10.10.1.9

Source: 	Huawei

Title:	TP of fast retransmission of lost PDUs to 38.401

Document for:	pCR

Introduction

This paper provides text proposal of fast retransmission of lost PDUs as discussed in [1]. The main intention of this TP is to explicitly show that centralized retransmission should be based on dual/multi-connection, for which general mechanism was also described.

Text Proposal

8.3	Mechanism of centralized retransmission of lost PDUs

Editor’s Note: whether this section could be captured in the same paragraph of mobility procedures is FFS.

This mechanism allows to perform the retransmission of the PDUs that were not delivered by a gNB-DU (gNB-DU1), e.g., because the radio link served by gNB-DU1 is subject to temporary outage, via another gNB-DU (gNB-DU2) with a well-functioning radio link toward the UE.

The gNB-DU1 shall detect the outage (e.g., based on measurement reports, CQI feedback, UL measurements, HARQ ACK/NACK ratios, RLC status reports). The gNB-DU1 shall send a notification to the gNB-CU about the outage.

Editor’s Note: how to define radio link outage is FFS.

Editor’s Note: whether the notification is over F1-C or F1-U is FFS.

Editor’s Note: whether new notification mechanism is needed is FFS.

Editor’s Note: whether the notification includes the PDCP PDU SNs corresponding to the RLC SDUs that have been either partially or totally undelivered in DU1 is FFS.

Dual/multi connectivity between gNB-DUs within a gNB-CU and across gNB-CUs enables fast retransmissions, the gNB-CU shall decide on which gNB-DU (e.g. gNB-DU2) with well-functioning radio link to forward traffic to the UE when Dual/multi connectivity is activated. Based on flow control information provided by gNB-DU1, the gNB-CU may decide to forward to gNB-DU2 the incoming data traffic for the UE as well as PDCP PDUs with SNs potentially indicated by gNB-DU1, or to start sending the duplicated data to both gNB-DU1 and gNB-DU2. Through monitoring the radio link served by gNB-DU1, the gNB-CU may decide to start sending again data traffic through gNB-DU1 if the radio link served by gNB-DU1 starts functioning.

References

R3-173114,	Fast retransmission of lost PDUs, Huawei.