3GPP TSG-RAN WG3 #97-BIS	R3-173901

Prague, Czech, 09 – 13 October 2017

Title:  	5GC involved inter registration area mobility

Agenda Item:	10.2.1

Source: 	Huawei

Document for:  pCR

# 1. Introduction

RAN3 has agreed that “some slices may be available only in part of the network,” and the CN involved handover has been captured in TS38.300 for mobility cross registration areas. In this paper, we further discuss the inter registration area mobility.

# 2. Discussion

Figure 1	Procedure of 5GC involved handover

For the active mode mobility across different Registration Areas, the procedure in figure 1 has been captured in TS38.300, and it was agreed in SA2 that:

“All PDU sessions handled by S-RAN (i.e. all existing PDU sessions with active UP connections) shall be included in the Handover Required message, indicating which of those PDU session(s) are requested by S-RAN to handover.”

Hence, in the Handover Required message, all existing PDU sessions with active UP connections shall be included, to indicate AMF which PDU sessions should handover to the target. Otherwise, the AMF cannot identify whether the transferred PDU session(s) is active or deactivate, thus it may contact each SMF for both active and deactivate PDU sessions.

Proposal 1: All existing PDU sessions with active UP connections shall be included in Handover Required message and captured in TS38.413.

For the case of active mode mobility across different Registration Areas, the target AMF is responsible for identifying and removing slices that are not supported by the target NG-RAN, and thus the target AMF shall be informed the associated slice of each PDU session. As the AMF may not store the SM information, i.e., the AMF may not store the slice information relevant to PDU session, the S-NSSAI of each PDU session shall be included in the Handover Required message.

Proposal 2: The S-NSSAI of each PDU session shall be included in the Handover Required message.

If any PDU session which has not been admitted by the target NG-RAN, the source NG-RAN should be informed the reason for the unsuccessful preparation, e.g., via cause or flag indication. For case that the target NG-RAN cannot support all the slices currently having RAN resources setup in the source cell, the AMF shall identify and remove the PDU sessions that cannot be admitted by the target NG-RAN due to slice availability. In this case, the AMF only sends the PDU sessions with supported slices to the target NG-RAN in the Handover Request message. For the PDU sessions that shall be released due to slice availability, the AMF shall inform to the source NG-RAN by PDU Sessions to Release List IE with appropriate cause value or flag indication (i.e., slice not supported) in the Handover Command message.

Proposal 3: A flag indication or cause value indicating “slice not supported” shall be introduced in NR.

# 3. Conclusion

In this paper, we further discussed the inter registration area mobility, and we have the following proposals

Proposal 1: PDU sessions requested by source NG-RAN to handover shall be included in Handover Required message and captured in TS38.413.

Proposal 2: The S-NSSAI of each PDU session shall be included in the Handover Required message.

Proposal 3: A flag or cause value indicating “slice not supported” shall be introduced in NR.

# References

- S2-174019 Response LS on slice re-mapping during connected mode mobility.

Beginning of Text Proposal to TS 38.413

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

Editor's note:	Further details are FFS.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source NG-RAN node initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source NG-RAN node sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source NG-RAN node shall indicate the appropriate cause value for the handover in the Cause IE.

The source NG-RAN node shall inform the AMF all existing PDU sessions with active UP connections in the HANDOVER REQUIRED message.

The source NG-RAN shall, if supported, include the S-NSSAI IE for each PDU session in the HANDOVER REQUIRED message. When the AMF reports the release of a PDU session due to slice availability, the cause value should be precise enough to enable the source NG-RAN to know the reason for the release, e.g., slice not supported.

In case of inter-system handover to LTE, the information in the Source to Target Transparent Container IE shall be encoded according to the Source eNB to Target eNB Transparent Container IE definition as specified in TS 36.413 [16].

Editor’s Note:	It is FFS whether the E-RABs Information List IE should be included in the Source eNB to Target eNB Transparent Container IE.

Editor’s Note:	It is pending to RAN2 whether the RRC Container IE should be encoded according to the target system.

When the preparation, including the reservation of resources at the target side is ready, the AMF responds with the HANDOVER COMMAND message to the source NG-RAN node.

If the Target to Source Transparent Container IE has been received by the AMF from the handover target then the transparent container shall be included in the HANDOVER COMMAND message.

In case of inter-system handover to LTE, the information in the Target to Source Transparent Container IE shall be encoded according to the definition of the Target eNB to Source eNB Transparent Container IE as specified in TS 36.413 [16].

Editor’s Note:	Further details are FFS.

### 9.2.3	UE Mobility Management Messages

#### 9.2.3.1	HANDOVER REQUIRED

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the source NG-RAN node to the AMF to request the preparation of resources at the target.

Direction: NG-RAN node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Handover Type | M |  | 9.3.1.33 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Target ID | M |  | <ref> |  | YES | reject |
| Direct Forwarding Path Availability | O |  | <ref> |  | YES | ignore |
| PDU Sessions Request List | M | 1..<maxnoofPDUSessions> |  |  |  |  |
| >PDU Sessions Request Item IEs |  |  |  |  |  |  |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | <ref> |  | - |  |
| Source to Target Transparent Container | M |  | 9.3.1.31 |  | YES | reject |

End of Text Proposal to TS 38.413