3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-173963

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	10.10.1.4

Source:	Ericsson

Title:	Other SI delivery command

Document for:	pCR TS 38.473

# Introduction

In R3-173961 we discuss the ownership and the delivery of the system information (SI) in the disaggregated (CU-DU) architecture. We propose to rely on the F1 setup request / response and configuration update messages to exchange and update the SI parameters over the F1 interface. We also propose to introduce an “Other SI Delivery Command” from gNB-CU to gNB-DU. The scope of the “Other SI Delivery Command” is to trigger the broadcast of the Other SI for a defined time interval. In this contribution, we present a stage 3 text proposal for TS 38.473 to introduce this procedure.

# TP for 38.473

Start of Text Proposal 1 for TS 38.473

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Other SI Delivery Command | OTHER SI DELIVERY COMMAND |

End of Text Proposal 1 for TS 38.473

Start of Text Proposal 2 for TS 38.473

## 8.X.Z 	Other SI delivery command

## 8.X.Z.1 	General

The purpose of the Other SI Delivery Command is to initiate the broadcast over the air of the Other SI for a defined time interval.

The procedure uses non-UE associated signalling.

## 8.X.Z.3 	Successful Operation

Figure 8.X.Z-1: Other SI delivery, successful operation.

The gNB-CU initiates the procedure by sending OTHER SI DELIVERY COMMAND message to the gNB-DU.

Upon reception of the OTHER SI DELIVERY COMMAND message, the gNB-DU shall broadcast the Other SI for the time interval indicated in the Broadcast Time Interval IE.

Editor’s note: Further details of this procedure are FFS.

## 8.X.Z.3 	Unsuccessful Operation

Not applicable.

## 8.X.Z.4 	Successful Operation

Not applicable.

End of Text Proposal 2 for TS 38.473

Start of Text Proposal 3 for TS 38.473

#### 9.2.x.1	OTHER SI DELIVERY COMMAND

This message is sent by the gNB-CU and is used to initiate the broadcast over the air of the Other SI for a defined time interval.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Broadcast Time Interval | M |  | <ref> |  |  |  |

End of Text Proposal 3 for TS 38.473