3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-173996

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	22.2

Source:	Ericsson, Vodafone, AT&T

Title:	SgNB addition with E1

Document for:	pCR for TR 38.806

# Introduction

In this contribution, we present the stage 2 call-flow for the secondary gNB (SgNB) addition in a disaggregated gNB (CU-DU) architecture in which the CU is separated in a control plane part (CU-CP) and user plane part (CU-UP) connected via an E1 interface.

# Discussion

In this contribution, we analyse the SgNB addition procedure in option 3 with a disaggregated gNB and an open E1 interface between CU-CP and CU-UP. The SgNB addition call-flow is shown in Figure 1.

Before proceeding with the description of the procedure, an observation that is relevant to some of the steps is that we assume that the CU-CP assigns the UL GTP TEID of the CU-UP. This is possible because we foresee that the CU-CP controls the operation of the CU-UP, which also implies that the CU-CP acquires detailed knowledge of the CU-UPs (including the UL GTP TEID) via configuration (during the E1 setup phase).

Observation 1	CU-CP allocates the UL GTP TEID of the CU-UP.

The following terminology is utilized to describe the entities involved in the procedure:

- MeNB: master eNB

- SgNB: secondary gNB

- DU: secondary gNB-DU

- CU-UP: secondary CU-UP

- CU-CP: secondary CU-CP

The UE is initially connected to LTE. The call-flow for adding the secondary gNB node encompasses the fowling steps.

- UE sends a RRC MEASUREMENT REPORT to MeNB. The RRC MEASUREMENT REPORT indicates the presence of an NR cell with good radio quality. The event that triggers the measurement report depends on the UE measurement configuration.

- MeNB sends the X2AP SgNB ADDITION REQUEST message to SgNB (CU-CP). MeNB requests radio resources for specific E-RABs, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type). The MeNB can provide the latest measurement results for the SCG cell(s) requested to be added.

- CU-CP performs admission control, creates a UE context, identifies DU, and selects CU-UP. Then, CU-CP sends the F1-AP CONTEXT SETUP REQUEST message to DU, including UE context information and CU-UP-UL-TEID for data radio bearers.

- DU performs admission control, configures the lower-layers, and creates a local UE context. Then, DU sends the F1-AP CONTEXT SETUP RESPONSE message to CU-CP, including lower-layers configuration, and DU-DL-TEID for data radio bearers.

- CU-CP sends the E1-AP BEARER SETUP message to CU-UP, including information for security configuration, and DU-DL-TEID.

- CU-UP applies the configuration received from CU-CP. Then, CU-UP sends the E1AP BEARER SETUP RESPONSE message to CU-CP.

- SgNB (CU-CP) sends the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message to MeNB. For SCG and SCG split bearers, the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message may provide S1 DL TNL address information for the respective E-RAB, for split bearers together with X2 DL TNL address information.

- MeNB sends the RRC CONNECTION RECONFIGURATION message to UE.

- UE applies the new configuration and replies with the RRC CONNECTION RECONFIGURATION COMPLETE message to MeNB.

- MeNB sends the SgNB ADDITION RECONFIGURATION COMPLETE message to SgNB (CU-CP) to indicate the success of the procedure.

- UE sends a RANDOM ACCESS preamble to DU.

- DU replies with a RANDOM ACCESS RESPONSE to UE.

- MeNB sends the X2AP SN STATUS TRANSFER message to SgNB (CU-CP) for data forwarding (if applicable).

- CU-CP sends the E1AP SN STATUS TRANSFER message to CU-UP for data forwarding (if applicable).

- For SCG and SCG split bearers, MeNB sends the E-RAB MODIFICATION INDICATION message to the MME to update the UP path.

- For SCG and SCG split bearers, MME confirms the new UP path by sending the E-RAB MODIFICATION CONFIRMATION message to MeNB.

- and 18: UL and DL data transmission.

NOTE: It is possible that steps 3-6 are performed in parallel if the CU-CP knows the DU-DL-TEID.

- Fig. 1: SgNB addition in option 3 with E1 interface.

# Conclusion

We conclude with the following proposals:

Proposal 1	RAN3 is kindly asked to agree with the proposed call-flow for SgNB addition.

Proposal 2	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for 38.806

Start of Text Proposal for TR 38.806

7.5.2 SgNB addition

This section presents the SgNB addition procedure. The SgNB addition call-flow is shown in Figure 7.5.2-1.

The following terminology is utilized to describe the entities involved in the procedure:

- MeNB: master eNB

- SgNB: secondary gNB

- DU: secondary DU

- CU-UP: secondary CU-UP

- CU-CP: secondary CU-CP

The UE is initially connected to LTE. The call-flow for adding the secondary gNB node encompasses the fowling steps.

- UE sends a RRC MEASUREMENT REPORT to MeNB. The RRC MEASUREMENT REPORT indicates the presence of an NR cell with good radio quality. The event that triggers the measurement report depends on the UE measurement configuration.

- MeNB sends the X2AP SgNB ADDITION REQUEST message to SgNB (CU-CP). MeNB requests radio resources for specific E-RABs, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type). The MeNB can provide the latest measurement results for the SCG cell(s) requested to be added.

- CU-CP performs admission control, creates a UE context, identifies DU, and selects CU-UP. Then, CU-CP sends the F1-AP CONTEXT SETUP REQUEST message to DU, including UE context information and CU-UP-UL-TEID for data radio bearers.

- DU performs admission control, configures the lower-layers, and creates a local UE context. Then, DU sends the F1-AP CONTEXT SETUP RESPONSE message to CU-CP, including lower-layers configuration, and DU-DL-TEID for data radio bearers.

- CU-CP sends the E1-AP BEARER SETUP message to CU-UP, including information for security configuration, and DU-DL-TEID.

- CU-UP applies the configuration received from CU-CP. Then, CU-UP sends the E1AP BEARER SETUP RESPONSE message to CU-CP.

- SgNB (CU-CP) sends the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message to MeNB. For SCG and SCG split bearers, the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message may provide S1 DL TNL address information for the respective E-RAB, for split bearers together with X2 DL TNL address information.

- MeNB sends the RRC CONNECTION RECONFIGURATION message to UE.

- UE applies the new configuration and replies with the RRC CONNECTION RECONFIGURATION COMPLETE message to MeNB.

- MeNB sends the SgNB ADDITION RECONFIGURATION COMPLETE message to SgNB (CU-CP) to indicate the success of the procedure.

- UE sends a RANDOM ACCESS preamble to DU.

- DU replies with a RANDOM ACCESS RESPONSE to UE.

- MeNB sends the X2AP SN STATUS TRANSFER message to SgNB (CU-CP) for data forwarding (if applicable).

- CU-CP sends the E1AP SN STATUS TRANSFER message to CU-UP for data forwarding (if applicable).

- For SCG and SCG split bearers, MeNB sends the E-RAB MODIFICATION INDICATION message to the MME to update the UP path.

- For SCG and SCG split bearers, MME confirms the new UP path by sending the E-RAB MODIFICATION CONFIRMATION message to MeNB.

- and 18: UL and DL data transmission.

NOTE: It is possible that steps 3-6 are performed in parallel if the CU-CP knows the DU-DL-TEID.

- Fig. 7.5.2-1: SgNB addition procedure.

End of Text Proposal for TR 38.806