3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-174073

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	10.10.1.4

Source:	Ericsson

Title:	On system information

Document for:	pCR TS 38.470

# Introduction

In R3-173961 we discuss the ownership and the delivery of the system information (SI) in the disaggregated (CU-DU) architecture. We propose to rely on the F1 setup request / response and configuration update messages to exchange and update the SI parameters over the F1 interface. We also propose to introduce an “Other SI Delivery Command” from gNB-CU to gNB-DU. The scope of the “Other SI Delivery Command” is to trigger the broadcast of the Other SI for a defined time interval. In this contribution, we present text proposal for TS 38.470 to introduce this procedure as part of the system information management function.

Proposal 1	RAN3 is kindly asked to agree with the text proposal in Annex I.

# Annex I: pCR for TS 38.470

Start of Text Proposal 1 for TS 38.470

Procedures of the F1 interface

## 5.1	Control plane procedures

### 5.1.1	Interface Management procedures

The F1 Interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	F1 Setup procedure

-	gNB-DU Configuration Update procedure

-	gNB-CU Configuration Update procedure

Editor’s Note: the direction of the update procedures, i.e. from gNB-CU to gNB-DU, or the other way around, or both, are FFS. Therefore, one of the procedures above may later be removed.

### 5.1.2	Context Management procedures

The F1 Context management procedures are listed below:

-	UE Context setup procedure

-	UE Context Release (gNB-DU initiated) procedure

-	UE Context Release (gNB-CU initiated) procedure

-	UE Context Modification (gNB-CU triggered) procedure

-	UE Context Modification Required (gNB-DU initiated) procedure

Editor’s Note: the naming of each procedure is FFS

### 5.1.4 	RRC Message Transfer procedures

The F1 RRC message transfer procedures are listed below:

-	Initial UL RRC Message Transfer procedure

-	UL RRC Message Transfer procedure

-	DL RRC Message Transfer procedure

### 5.1.5	System Information Exchange procedures (FFS)

The F1 System information exchange procedures are listed below:

-	Other SI Delivery Command

Editor’s Note: The details on these procedures is FFS.

End of Text Proposal 1 for TS 38.470