3GPP TSG-RAN WG3 #97bis								R3-174125

Prague, Czech Republic, 09 – 13 October 2017

Agenda Item:	10.8.1.2

Source:	InterDigital Communications

Title:	Documentation of option 3 in in 38.401

# Introduction

At the last meeting R3-172714 started to address the areas that needed to be modified to correctly address option 3 in general but in particular. in the F1 specifications. That discussion got bogged down in a terminology discussion because it took the route of introducing some new terminology. This proposal doesn’t introduce any new terminology it just hopefully correctly specifies en-gNB alongside of gNB, and is in line with R3-173543.

# 38.401 Changes

In 38.401 the one thing that needs to clarified is that the gNB-CU and the gNB-DU apply to splits for both a gNB and a en-gNB. And of course, adding in the definitions a en-gNB

Proposal 1: Make the following changes in a pCR to 38.401:

# Definitions and abbreviations

## Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB, and controls the operation of one or more gNB-DUs. The gNB-CU also terminates F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates F1 interface connected with the gNB-CU.

Editor’s note: Whether it is possible that one cell spans over multiple DUs, potentially in an implementation specific way, is FFS. The reasons, conditions, and scenarios for this are also FFS.

gNB: as defined in TS 38.300 [2].

en-gNB: as defined in TS 37.340 [14]

## Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

CM	Connection Management

CMAS	Commercial Mobile Alert Service

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

NAS	Non-Access Stratum

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

TDD	Time Division Duplex

TNL	Transport Network Layer