3GPP TSG-RAN WG3 #98	R3-174315

Reno, US, 27 Nov – 1 Dec 2017

Agenda item:	10.8.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Reconfiguration failure handling

Document for:	Discussion and Decision

# 1	Introduction and discussion

As discussed in [1], the current information on the rejection is misleading. The TP below corrects the issue for XnAP.

# References

- R3-174314, RAN3 #98

# Text proposal

The changes below are proposed to be incorporated in the draft TS 38.423.

First change, ommited text not changed

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  |  |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in xxx. Editor’s Note: FFS if same RRC message is applicable for SCG bearer. | - | - |
| >>Configuration rejected |  |  |  |  |  |  |
| >>>Cause | M |  | <reference> |  | - | - |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in xxx . Editor’s Note: FFS if same RRC message is applicable for SCG bearer. | - | - |

Remaing text not changed