3GPP TSG-RAN WG3 #98	R3-174322

Reno, US, 27 Nov – 1 Dec 2017

Agenda item:	10.8.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Tunnel ID switching in case of reconfiguration

Document for:	Discussion and Decision

# 1	Introduction

In case of the SCG change, the PDCP ciphering may be re-established and the keys assigned anew. To enable differentiation of PDUs ciphered with the new key, the TEID is changed. This solution, known from LTE DC, seems incomplete in the EN-DC. In this paper we discuss what needs to be added.

# 2	Discussion

In LTE DC, the solution is used at both ends: at the SeNB for DL data and in the MeNB, for UL data. The SeNB may need to change the TEID in case of MN-initiated SN modification with the split bearer. The MeNB needs to provide the new TEID in case of an intra-MN HO.

In EN-DC, both descriptions were copied from the LTE, so that the intra-MN HO and the MN-initiated SN modification with the split bearer are covered. The modification description was extended to enable providing the UL TEID when SCG split bearer is used. However, the case of MN-initiated SN modification is not described.

In case of SCG split bearer, the hosting node is the SN, so the DL is actually from the SN to the MN. Therefore, when initiating SN modification in case of SCG split bearer, the MN shall provide the new TEID already in the modification request.

# 3	Conclusion

In this paper, we show that in case of the split bearers, the MN shall provide the new TEID already when initiating SN modification. The TP below adds the needed description to the draft stage-2 TS 37.340. Stage-3 seems complete.

# Text proposal

The changes below are proposed to be incorporated in the draft TS 37.340.

First change, ommited text not changed

### 10.3.1	EN-DC

The Secondary Node Modification procedure may be initiated either by the MN or by the SN and be used to modify, establish or release bearer contexts, to transfer bearer contexts to and from the SN or to modify other properties of the UE context within the same SN. It may also be used to transfer an NR RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (when SRB3 is not used).

The Secondary Node modification procedure does not necessarily need to involve signalling towards the UE.

MN initiated SN Modification

Figure 10.3.1-1: SN Modification procedure - MN initiated

Editor’s note: The figure above needs further revision, mainly to refer to the embedded RRC messages/Ies that will be agreed to be exchanged over X2.

The MN uses the procedure to initiate configuration changes of the SCG within the same SN, e.g the addition, modification or release of SCG bearer(s) and the SCG part of MCG split and SCG split bearer(s). The MN uses this procedure to perform handover within the same MN while keeping the SN. The MN also uses the procedure to query the current SCG configuration, e.g. when delta configuration is applied in a MN initiated SN change. MN may not use the procedure to initiate the addition, modification or release of SCG Scells. The SN may reject the request, except if it concerns the release of SCG bearer(s) or the SCG part of MCG split bearer(s) and SCG split bearer(s). Figure 10.3.1-1 shows an example signalling flow for a MN initiated SN Modification procedure.

1.	The MN sends the SgNB Modification Request message, which may contain bearer context related or other UE context related information, data forwarding address information (if applicable) and the requested SCG configuration information, including the UE capability coordination result to be used as basis for the reconfiguration by the SN. In case of SCG Change, SCG Change Indication is included. In case of SCG RLC re-establishment for E-RABs configured with the MCG split bearer option for which no bearer type change is performed, the MN provides a new UL GTP TEID to the SN. The SN shall continue sending UL PDCP PDUs to the MN with the previous UL GTP TEID until it re-establishes the RLC and use the new UL GTP TEID after re-establishment. In case of PDCP re-establishment or PDCP data recovery for E-RABs configured with the SCG split bearer option for which no bearer type change is performed, the MN provides a new DL GTP TEID to the SN. The SN shall continue sending DL PDCP PDUs to the MN with the previous DL GTP TEID until it performs PDCP re-establishment or PDCP data recovery, and use the new DL GTP TEID starting with the PDCP re-establishment or data recovery.

NOTE:	MN may request the establishment or release of SCG or MCG Split or SCG split bearer while not reconfiguration to MCG bearer, which can be performed without SCG change.

2.	The SN responds with the SgNB Modification Request Acknowledge message, which may contain SCG radio resource configuration information within a NR RRC configuration message and data forwarding address information (if applicable).  In case of SCG Change, for E-RABs configured with the MCG split bearer option for which no bearer type change is performed, the SN provides a new DL GTP TEID to the MN. The MN shall continue sending DL PDCP PDUs to the SN with the previous DL GTP TEID until it performs PDCP re-establishment or PDCP data recovery, and use the new DL GTP TEID starting with the PDCP re-establishment or data recovery. In case of SCG Change, for E-RABs configured with the SCG split bearer option for which no bearer type change is performed, the SN provides a new UL GTP TEID to the MN. The MN shall continue sending UL PDCP PDUs to the SN with the previous UL GTP TEID until it re-establishes the RLC and use the new UL GTP TEID after re-establishment.

3-5.	The MN initiates the RRC connection reconfiguration procedure, including the NR RRC configuration message. The UE applies the new configuration, synchronizes to the MN (if instructed, in case of intra-MN handover) and replies with RRCConnectionReconfigurationComplete, including a NR RRC response message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SgNB Reconfiguration Complete message.

7.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SgNB addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

8/9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.1-1 depicts the case where a bearer context is transferred from the MN to the SN).

10.	If applicable, a path update is performed.

Remaing text not changed