3GPP TSG-RAN WG3 Meeting #98	R3-17XXXX

# Reno, USA, November 27th – December 1st 2017

Agenda item:	10.8.1.2. General

Source:	HTC

Title:	TP for EN-DC containers

Document for:	Discussion & Decision

- Introduction

About the SN inititated SN change procedure, the current stage 2 TS [1] captures “The source SN initiates the SN change procedure by sending SgNB Change Required message which contains target SN ID information and may include the SCG configuration (to support delta configuration) and measurement results related to the target SN.” Also the RRC message names have been determined in the RAN2 email discussion on NR RRC [2] and Inter-node RRC messages [3].  The SCG-Config message may include CellGroupConfig (Containing RRCReconfiguration) and RadioBearerConfig	(Containing RadioBearerConfiguration). Therefore, in the stage 3 specification, we have the following proposed changes.

- The SgNB to MeNB Container in the SgNB Change Required message should carry the SCG-Config message.

- The SgNB to MeNB Container in the SgNB Addition Request Acknowledge message should carry the SCG-Config message.

- The MeNB to SgNB Container in the SgNB Reconfiguration Complete message should carry the RRCReconfigurationComplete message.

- The SgNB to MeNB Container in the SgNB Modification Request Acknowledge message should carry the SCG-Config message.

- The SgNB to MeNB Container in the SgNB Modification Required mesaage should carry the SCG-Config message.

- The MeNB to SgNB Container in the SgNB Modification Confirm message should carry RRCReconfigurationComplete message.

Proposal: RAN3 to adopt the text proposal to the TS 36.423 provided in this paper.

References

[1] TS 37.340 v1.2.0

[2] 99bis#16[NR] TS 38.331 (Ericsson)

[3] R2-1713454, 99bis#25[NR] on Inter-node RRC messages

# 3 Text Proposal to TS 36.423 BL

Start of 1st Change

9.1.X.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the SgNB to confirm the MeNB about the SgNB addition preparation.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>> E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331[xx]. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating GW Transport Layer Address. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.89 | Indicating eNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| Admitted MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

End of 1st Change

Start of 2nd Change

9.1.X.4	SGNB RECONFIGURATION COMPLETE

This message is sent by the MeNB to the SgNB to indicate whether the configuration requested by the SgNB was applied by the UE.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  |  |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>MeNB to SgNB Container | M |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in TS 38.331 [xx]. | - | - |
| >>Configuration rejected by the MeNB |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.6 |  | - | - |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |

End of 2nd Change

Start of 3rd Change

9.1.X.6	SGNB MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the SgNB to confirm the MeNB’s request to modify the SgNB resources for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| E-RABs Admitted List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added List |  | 1 |  |  | – | – |
| >>E-RABs Admitted To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |
| >E-RABs Admitted To Be Modified List |  | 0..1 |  |  | – | – |
| >>E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SgNB. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>SgNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes necessary QoS parameters for the MCG part. | – | – |
| >E-RABs Admitted To Be Released List |  | 0..1 |  |  | – | – |
| >>E-RABs Admitted To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331 [xx]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

End of 3rd Change

Start of 4th Change

#### 9.1.X.8	SGNB MODIFICATION REQUIRED

This message is sent by the SgNB to the MeNB to request the modification of SgNB resources for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SCG Change Indication | O |  | 9.2.73 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>Cause | M |  | 9.2.6 |  | – | – |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331 [xx]. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | reject |
| E-RABs To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes necessary QoS parameters for the MCG part | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

End of 4th Change

Start of 5th Change

#### 9.1.X.9	SGNB MODIFICATION CONFIRM

This message is sent by the MeNB to inform the SgNB about the successful modification.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | ignore |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in TS 38.331 [xx]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID Extension | O |  | Extended gNB UE X2AP ID 9.2.XY | Allocated at the SgNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

End of 5th  Change

Start of 6th Change

#### 9.1.X.15	SGNB CHANGE REQUIRED

This message is sent by the SgNB to the MeNB to request the change of SgNB for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB. | YES | reject |
| Target SgNB ID Information | M |  | 9.2.ZZ |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331 [xx]. | YES | reject |

| Range bound | Explanation |
| --- | --- |

End of Changes