3GPP TSG-RAN WG3 Meeting #98	R3-174712

# Reno, USA, 27th November - 1st  December 2017

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Notification Control

Document for:	pCR

# Information

This pCR provides a new Notify procedure from one NG-RAN node to another NG-RAN node in XnAP.

# Text Proposal for NGAP TS 38.423 v0.4.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 8.3.X	Notify

#### 8.3.X.1	General

The purpose of the Notify procedure is to allow one NG-RAN node to send notification to another NG-RAN node for the already established QoS flow(s) or PDU session(s) for a given UE. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: Notify

The NG-RAN node initiates the procedure by sending a NOTIFY message.

The NOTIFY message shall contain notification for the already established PDU sessions or QoS flows.

For each PDU session of which some QoS flows that a notification is provided, the PDU Session Resource Notify Transfer IE shall be included to report for each PDU session of which some QoS flow(s).

-	The list of QoS flows for which a notification is issued to indicate if the Qos requirement can no longer be fulfilled, or fulfilled again, if any, shall be included in the QoS Flows Notification Control IE, and the Notification Cause IE per QoS flow.

Upon reception of the NOTIFY message, the receiving NG-RAN node shall, for each PDU session indicated in the PDU Session ID IE, decide if to inform the MN node or 5GC that the PDU sessions or QoS flows have fulfilled the condition for sending the notification.

Editor’s Note:	Further details are FFS.

#### 8.3.X.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.X	NOTIFY

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by a NG-RAN node to send notification to another NG-RAN node for the already established QoS flow(s) or PDU session(s) for a given UE

Direction: NG-RAN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resource Notify Item List |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >PDU Session ID | M |  | <ref> |  | - |  |
| >PDU Session Resource Notify Transfer | M |  | 9.2.X |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is FFS. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.X	PDU Session Resource Notify Transfer

This IE is used to indicate the Notification cause for the QoS flows or PDU sessions.

Editor’s Note: 	Further details FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE PDU Session Resource Notify Transfer | M |  |  |  |
| >QoS Flows Notification Control |  |  |  |  |
| >>QoS Flows Notify List |  | 1..<maxnoofQoSFlows> |  |  |
| >>>QoS Flow Indicator | M |  | <ref> |  |
| >>>Notification Cause | M |  | ENUMERATED(Fullfilled, Not Fulfilled, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>