3GPP TSG-RAN WG3 Meeting #98	R3-174722

# Reno, USA, 27th November - 1st  December 2017

Agenda Item:	10.5.2

Source:	Ericsson

Title:	Correction on Path Switch Request

Document for:	pCR

# 1	Discussion

Due to the AMF/SMF split, the AMF does not always have the full knowledge of PDU sessions being setup, especially after handover.

In Path Switch procedure, it is thus desirable that source RAN could provide the PDU sessions been requested to switch of a downlink GTP tunnel towards a new GTP tunnel endpoint, and a list of PDU session failed to be setup, during, e.g. Xn/S1 handover.

Proposal 1: RAN3 to include a PDU session failed to setup list in Path Switch Request.

# 2	Proposals

Proposal 1: RAN3 to include a PDU session failed to setup list in Path Switch Request.

# 3	Reference

- [1]	S2-178202	TS 23.502: Correction to Xn based HO procedure with UPF relocation, Samsung

# Text Proposal for NGAP TS 38.413 v0.4.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

Editor’s Note:	Further details are FFS.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF.

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU Session Resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

The list of PDU sessions which failed to be setup, if any, shall be included in the PATH SWITCH REQUEST message.

If the RRC Inactive Assistance Information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

Editor’s Note:	Further details are FFS.

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the 5GC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all PDU Session Resources, the AMF shall send the PATH SWITCH REQUEST FAILURE message to the NG-RAN node with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.4.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.8	PATH SWITCH REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node to inform the AMF of the new serving NG-RAN node and to transfer some NG-U DL tunnel termination point(s) to the SMF via the AMF for one or multiple PDU sessions.

Direction: NG-RAN node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Source AMF UE NGAP ID | M |  | AMF UE NGAP ID 9.3.3.1 |  | YES | reject |
| User Location Information | M |  | 9.3.1.27 |  | YES | ignore |
| UE Security Capabilities | M |  | <ref> |  | YES | ignore |
| PDU Session To Be Switched in Downlink List |  | 1 |  |  | YES | reject |
| >PDU Session To Be Switched in Downlink Item IEs |  | 1..<maxnoofPDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI (FFS) | O |  | <ref> |  | - |  |
| >>PDU Session Path Switch Request Transfer | M |  | 9.3.1.21 |  | YES | ignore |
| PDU Sessions Failed to Setup List |  | 0..<maxnoofPDUSessionResources> |  |  | YES | ignore |
| >PDU Session ID | M |  | <ref> |  | - |  |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>